3GPP TSG-RAN WG4 Meeting #105	R4-2219561

Toulouse, France, 14 – 18 November, 2022

Title: 	Reply LS on support of positioning in FR2-2

Source: 	Huawei, HiSilicon

Agenda item:	10.2.1.3

Document for:	Approval

Introduction

RAN4 received LS [1] from RAN2#119-e related to support of positioning in FR2-2. RAN2 asks RAN4 whether SRS/PRS with 480/960kHz SCS can be supported in Rel-17.

On the other hand, in POS WI, there has been no discussion on the support of positioning (especially related to RAT-dependent positioning) in FR2-2 and thus it is unclear whether SRS for positioning/DL-PRS can be used with the new 480/960 kHz SCS in FR2-2 or not. Based on the above observation, RAN2 would like to ask RAN1 and RAN4 the following question: Question: Can SRS for positioning/DL-PRS with 480/960 kHz SCS be supported in FR2-2 in R17?

In this paper we will provide our views on the question in the LS and a draft reply.

Discussion

In our view, whether SRS/PRS with 480/960kHz SCS can be supported functional wise with Rel-17 spec should be up to RAN1 and RAN2, while RAN4 should focus on the performance requirements aspect.

RRM requirements for positioning have been defined without considering FR2-2. For example, in the measurement period requirements, scaling factor for Rx beam sweeping has not accounted for the increased Rx beam number in FR2-2. For the accuracy requirements, larger SCS will lead to different performance especially for timing related measurements because the resolution is depending on the BW in Hz. The RF calibration margin may also be different from FR2-1 and needs to be discussed separately.

Based on above, we suggest RAN4 to clarify that RRM requirements for positioning are not defined for FR2-2.

Proposal: RAN4 to clarify that RRM requirements for positioning are not defined for FR2-2.

Conclusions

In this paper we provided our views on the question in the LS [1].

Proposal: RAN4 to clarify that RRM requirements for positioning are not defined for FR2-2.

A draft reply LS is provided in the Annex.

Reference

- R2-2208810, LS on support of positioning in FR2-2, RAN2

Annex: draft Reply LS on support of positioning in FR2-2

3GPP TSG-RAN WG4 Meeting #105-e R4-22xxxxx Toulouse, France, 14 – 18 November, 2022 Title: Reply LS on support of positioning in FR2-2 Response to: R2-2208810 Release: Release 17 Work Item: NR_pos_enh-Core Source: RAN4 To: RAN2, RAN1 Cc: Contact Person: #### Name: Li Zhang - E-mail Address: zhangli164@huawei.com Attachments: - 1. Overall Description: RAN4 would like to thanks RAN2 for the question in LS R2-2208810. Based on discussion in RAN4#105, RAN4 understands that functional wise whether SRS for positioning/DL-PRS with 480/960 kHz SCS can be supported in FR2-2 in R17 is up to RAN1 and RAN2, while RRM requirements for positioning are not defined for FR2-2. RAN4 respectfully asks RAN2 to take the above information into account. 2. Actions: To RAN2: RAN4 respectfully asks RAN2 to take the above information into account. 3. Date of Next TSG-RAN4 Meetings: RAN WG4 Meeting #106 Feb 27 – Mar 3, 2023 Athens, Greece RAN WG4 Meeting #106-bis-e Apr 17 – Apr 26, 2023 Online