3GPP TSG-RAN WG4 Meeting #105	R4-2219604

Toulouse, France, November 14 – November 18, 2022

Title:	Regulatory information about ESIM

Source:	OPPO

Agenda item:	8.23.1.2

Document for:	Discussion

Introduction

In 2019, MIIT released the notice to specify the relevance regarding GSO in fixed satellite service deploying ESIM earth station in Ka bands. The requirements of the operational and technical characteristics of the transmitting station of ESIM in 29.5-30GHz has been attached [1]. The enacted notice defined that the ESIM earth station in Ka band is the earth station with the ability to conduct the radiocommunication with satellite, which are installed on the movable platform such as motor vehicles, rail way locomotives (including EMU trains), ships and aircraft. The ESIM earth station is transmitting in 29.5-30GHz and receiving in 17.7-20GHz.

Additionally, in the Radio Regulations, the regulatory requirements of space services including FSS and MSS are intricacy, especially for FR 2. As for ESIM, the several Articles and Resolutions needs to be noted.

This paper presents current status and information of the regulatory aspects about ESIM in China and provide some ITU regulatory information.

Discussion

Current status of regulatory requirements in China

The following requirements are interpreted from the attachment of the MIIT’s notice released in 2019 (the requirements of the operational and technical characteristics of the transmitting station of ESIM in 29.5-30GHz). The technical characteristics requirements are similar to the Resolution 156 (WRC-15) [2] with slight modifications and additional operational requirements.

In bullets 1, 2, 3 and 4, the maximum EIRP and antenna relevant characteristics have been specified, and in bullets 5 and 6, operational requirements have been specified. The purpose of setting technical requirements is to avoid harmful interference between the ESIM earth stations and other legitimate radio services, and to maintain the order of using radio waves. The setting of specific index values fully considers the electromagnetic compatibility analysis results between the ESIM and the same frequency service, the content of relevant ITU recommendations, and the research and development capabilities of domestic manufacturers of ESIM.

- On the condition of clear sky, the maximum EIRP density of the transmitting station pointing to the target GSO satellite within 3° shall not exceed the limitation in any φ nadir in the following table: Off-axis angle φ; maximum EIRP density per 40kHz (dB(W/40kHz)). Co-Polarization; Cross Polarization. 2°≤φ≤7°; 19-25log φ; 9-25log φ. 7°< φ ≤9.2°; -2; -12. 9.2°< φ ≤48°; 22-25log φ. 48°< φ ≤180°; -10. - Regarding to the ESIM earth station operating in elevation angle ε, When ε is no greater than 5°, the maximum EIRP could be increased by 2.5dB; When ε is between 5° and 30°, the maximum EIRP could be increased by (3-0.1ε) dB. - Other types of ESIM earth station except aeronautical ESIM station, the angle between main beam axis and horizon shall not less than 10°; the antenna isolation shall greater than 20dB. - The aperture of parabolic antenna or equivalent aperture of non-parabolic antenna of an ESIM earth station shall be no less than 0.45m. - The antenna pointing error shall be less than 0.2°. During operation, once the error of the main lobe axis of the antenna pointing to the target satellite is greater than 0.5°, all signal transmission should be automatically suspended within 100ms until the pointing error returns to less than 0.2° before proceeding to transmit signals. - The ESIM earth station shall have the function of automatically shutting down the transmitted signal. This function could be managed by the monitoring and control center of the satellite communication network or the ESIM earth station itself. Once the maximum EIRP density exceeds the limit or the communication with the corresponding satellite is interrupted and other satellites are captured, the ESIM earth station should be able to automatically shut down the transmission, or remotely stop the transmission by the monitoring and control center.

In recent, the MIIT released the exposure draft to collect the public comments on the regulatory aspects to manage the GSO ESIM earth station using FSS spectrum. The exposure draft has updated some new requirements on ESIM earth station.

For the frequency bands in 27.5-30.0 GHz, in bullet 1, the requirements of cross polarization requirements of maximum EIRP has been removed from the Table; in bullet 4, the requirement of the aperture has been amended from “0.45m” to “0.3m”, and the requirement of cross polarization isolation has been added. Besides those general updates, there are 2 new requirements added including the requirement of maximum EIRP power of maritime ESIM earth station, PFD limitation of aeronautical ESIM stations in different altitude.

Further information will be supplemented once the formal release posts.

Observation 1: 	The incoming regulatory requirement will be based on the frequency range 27.5-30.0 GHz wider than 29.5-30GHz.

Proposal 1:	The regulatory information of China could be noticed and considered.

The regulatory requirement in the Radio Regulations

The frequency bands 17.7-18.6 GHz, 18.8-19.3 GHz and 19.7-20.2 GHz (space-to-Earth) and 27.5-29.1 GHz and 29.5-30 GHz (Earth-to-space) are globally allocated on a co-primary basis to the FSS,

As mentioned in [3], it is provisioned for ESIM by ITU in the Radio Regulations for FSS and MSS and the frequency range options are captured in the WF [4].

-Option 1: the downlink frequency range 17.7-20.2 GHz, the uplink frequency range 27.5-30.0 GHz for NTN Ka-band definition (i.e. spectrum allocated by ITU to FSS). -Option 2: the downlink frequency range 19.7-20.2 GHz, the uplink frequency range 29.5-30.0 GHz for NTN Ka-band definition (i.e. spectrum allocated by ITU to MSS and FSS). -Option 3: the downlink frequency range 17.3-17.7 GHz, the uplink frequency range 29.5-30.0 GHz for NTN Ka-band definition (the DL part is out of frequency range of current harmonized Ka band) -Option 4: define multi bands in this frequency range i.e. one band as option 1 and another band as option 2; and/or another band as option 3

For further information, the following footnotes related to ESIM in the Radio Regulations are worth of remark. The different frequency bands are allocated to different regions as highlighted in yellow:

5.517A The operation of earth stations in motion communicating with geostationary fixed-satellite service space stations within the frequency bands 17.7-19.7 GHz (space-to-Earth) and 27.5-29.5 GHz (Earth-to-space) shall be subject to the application of Resolution 169 (WRC-19). 5.526 In the bands 19.7-20.2 GHz and 29.5-30 GHz in Region 2, and in the bands 20.1-20.2 GHz and 29.9-30 GHz in Regions 1 and 3, networks which are both in the fixed-satellite service and in the mobile-satellite service may include links between earth stations at specified or unspecified points or while in motion, through one or more satellites for point-to-point and point-to-multipoint communications. 5.527A The operation of earth stations in motion communicating with the FSS is subject to Resolution 156 (WRC-15).

Observation 2: 	According to No. 5.526, there are some regional information related to the frequency range. The frequency range for Region 1, 2 and 3 are different.

As indicated in above cited footnotes, there would be also some Resolutions to be noted such as Resolutions 156(WRC-15) and 169(WRC-19) [5], where some detailed provisions for ESIM might be considered for this WI. Within the two Resolutions, there are some technical and operational requirements stipulated, and some ITU-R recommendations and reports are also referred, e.g. Report ITU-R S.2357-0 [6] (Technical and operational guidelines for earth stations on mobile platforms communicating with geostationary space stations in the fixed-satellite service in the frequency bands 19.7 20.2 GHz and 29.5-30.0 GHz) and Report ITU-R S.2464-0 [7] (Operation of earth stations in motion communicating with geostationary space stations in the fixed-satellite service allocations at 17.7-19.7 GHz and 27.5-29.5 GHz).

Resolution 156(WRC-15) describes the use of the frequency bands 19.7-20.2 GHz and 29.5-30.0 GHz by earth stations in motion communicating with geostationary space stations in the fixed-satellite service.

Resolution 169(WRC-19) describes the use of the frequency bands 17.7-19.7 GHz and 27.5-29.5 GHz by earth stations in motion communicating with geostationary space stations in the fixed-satellite service. The major parts of the Resolutions were embedded in the attachment 1 and 2.

In addition, in the Radio Regulations, there are 2 articles not specified for ESIM but for the FSS system, which are also something needed to be noticed. Article 21 [8] determines power flux-density (pfd) limits applicable to non-GSO FSS systems to protect fixed and mobile land stations. Article 22 [9] contains epfd limits for non-GSO FSS systems in the frequency bands 17.8-18.6 GHz, 19.7-20.2 GHz (space-to-Earth), 27.5-28.6 GHz (Earth-to-space), 29.5-30 GHz (Earth-to-space) and 17.8-18.4 GHz (inter-satellite).

Observation 3: 	The resolutions related to ESIM provided different requirements according to the category of the ESIM. Some provisions are irrelevant to the technical part of RAN 4 correspondence but others are ought to be considered such as power limits, maximum power in the OOB domain, etc.

Proposal 2: 	Discuss if there is the necessity to consider regional information with respect to the frequency range options.

Conclusions

This paper provided some information on regulatory aspects. And got the following conclusions:

Observation 1: 	The incoming regulatory requirement will be based on the frequency range 27.5-30.0 GHz wider 29.5-30GHz.

Observation 2: 	According to No. 5.526, there are some regional information related to the frequency range. The frequency range for Region 1, 2 and 3 are different.

Observation 3: 	The resolutions related to ESIM provided different requirements according to the category of the ESIM. Some provisions are irrelevant to the technical part of RAN 4 correspondence but others are ought to be considered such as power limits, maximum power in the OOB domain, etc.

Proposal 1:	The regulatory information of China could be noticed and considered.

Proposal 2: 	Discuss if there is the necessity to consider regional information with respect to the frequency range options.

References

[1] MIIT [2019]120, The requirements of the operational and technical characteristics of the transmitting station of ESIM in 29.5-30 GHz

[2] ITU-R Radio Regulation, volume III, RESOLUTION 156 (WRC-15), Use of the frequency bands 19.7-20.2 GHz and 29.5-30.0 GHz by earth stations in motion communicating with geostationary space stations in the fixed-satellite service

[3] R4-2216515, NTN enhancements – Regulatory aspects, Ericsson

[4] R4-2217467, Way Forward on NR NTN Enhancements Part1, Moderator (THALES)

[5] ITU-R Radio Regulation, volume III, RESOLUTION 169 (WRC-19), Use of the frequency bands 17.7-19.7 GHz and 27.5-29.5 GHz by earth stations in motion communicating with geostationary space stations in the fixed-satellite service

[6] ITU-R Report S.2261-0 (2012), https://www.itu.int/pub/R-REP-S.2261-2012

[7] ITU-R Report S.2464-0 (2019), https://www.itu.int/pub/R-REP-S.2464-2019

[8] ITU-R Radio Regulation, volume I, Article 21

[9] ITU-R Radio Regulation, volume I, Article 22

Attachment:

- Attachment 1: ANNEX to Resolution 156 (WRC-15)

- Attachment 2: ANNEXs to Resolution 169 (WRC-19)