3GPP TSG-RAN WG4 Meeting #105                                R4-2219620

Toulouse, France, 14 November– 18 November, 2022

Source: 	Huawei, HiSilicon

Title: 	TP for TR 38.872 to capture the RF architecture assumption and IMD evaluation for CA_n8-n20-n28

Agenda Item:	8.2.4.2

Document for:	Approval

Introduction

In RAN4#104bis-e meeting, a WF [1] was approved with the following outcomes.

Agreement:

- Use 3 antenna UE architecture as the baseline for evaluation.

  - Do not preclude 2 and 4 antennas in the study item

Way forward:

- Assuming it is the worst case, analysis is prioritized for 3 antenna architecture based on:

  - n28UL/n20+n28DL/n20UL triplexer on antenna 1

  - n8UL/n8DL duplexer on antenna 2

  - n20+n28DL/n8DL duplexer on antenna 3

  - For comparison purpose, analysis of alternate two or three antenna solutions are not precluded, and if a worse case is found it can be discussed whether it should be used for the MSD feasibility.

- 10db antenna isolation is used for MSD calculations

- The normal Tx/Rx 50dB isolation is used between UL and DL of the same FDD bands for duplexer/triplexer/quadplexer. If higher insertion loss is needed to meet the 50dB it can be accounted in DeltaT/R

- For different FDD bands within triplexer/quadplexer companies are encouraged to provide feasible isolation between ULs and DLs

- For diversity path, the DL filter provide at least 40dB of Rejection of the UL of the same band and companies are encouraged to provide feasible rejection for UL of other bands.

- Companies are encouraged to provide input on the feasibility of such values

Observations:

- All single band MSDs should already be covered

- IMD3 MSD of DC_20_n8A in n20 and n8 is already specified and can be reused for CA_n8-n20

- All 2 band cases are already specified

- Only MSD to third band needs to be analysed

  - IMD3 of CA_n20-n8 UL in n28 DL

  - IMD3 of CA_n20-n28 UL in n8 DL

Way forward on IMD3 of CA_n20-n8 UL in n28 DL:

- 5MHz n8 UL at 887.5MHz with 25RB0

- 5MHz n20 UL at 834.5MHz with 25RB0

- 5MHz n28 DL at 781.5MHz

- IMD3 MSD is evaluated by calculation, simulation, or measurement

Way forward on	IMD3 of CA_n20-n28 UL in n8 DL:

- 5MHz n20 UL at 834.5MHz with 25RB0

- 5MHz n20 UL at 715.5MHz with 25RB0

- 5MHz n8 DL at 952.5MHz

- IMD3 MSD is evaluated by calculation, simulation, or measurement

In this paper, we provide the following text proposals to capture the corresponding agreements and technical analysis into TR.

- References

[1] R4-2217710, WF on CA_n8-n20-n28, Skyworks Solutions Inc.

- Text proposal

- <<Start of Change for TR 38.872>>

- 5.3	CA_n8-n20-n28

- 5.3.1	UE RF architecture assumption

The UE RF architectures with 2 antenna, 3 antenna, 4 antenna can be assumed to analyze and study for CA_n8-n20-n28.

The antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands. UE RF architecture with three antennas for CA_n8-n20-n28 is used as baseline for evaluation. The UE RF architectures with two or four antennas are not precluded in the study item.

The specific assumptions for UE RF architecture with three antennas for CA_n8-n20-n28 are shown as below.

1) n28UL/n20+n28DL/n20UL triplexer on antenna 1

2) n8UL/n8DL duplexer on antenna 2

3) n20+n28DL/n8DL duplexer on antenna 3

			4) 10dB antenna isolation is used for MSD calculations

- <<Next of Change>>

- 5.3.2.8	REFSENS requirements for UL inter-band CA

IMD3 MSD of DC_20_n8A in band 20 and n8 specified in TS 38.101-3 can be reused for CA_n8-n20 as below.

Table 5.3.2.8-1: REFSENS exceptions due to IMD interference for CA_n8-n20

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n8-n20 | n8 | 892.5 | 5 | 25 | 937.5 | 25 | FDD | IMD3 |
|  | n20 | 849.5 | 5 | 25 | 808.5 | 25 | FDD | IMD3 |

- <<Next of Change>>

- 5.3.5.5	REFSENS requirements

The MSD test configurations can be specified as below for band n8 MSD due to IMD3 of Tx band n20 + band n28 and band n28 MSD due to IMD3 of Tx band n8 + band n20.

Table 5.3.5.5-1: REFSENS exceptions due to IMD interference for CA_n8-n20-n28

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n8-n20-n28 | n20 | 834.5 | 5 | 25 | 793.5 | N/A | FDD | N/A |
|  | n28 | 715.5 | 5 | 25 | 770.5 | N/A | FDD | N/A |
|  | n8 | 908.5 | 5 | 25 | 953.5 | TBD | FDD | IMD3 |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | FDD | N/A |
|  | n20 | 834.5 | 5 | 25 | 793.5 | N/A | FDD | N/A |
|  | n28 | 726.5 | 5 | 25 | 781.5 | TBD | FDD | IMD3 |

- <<End of Change>>