3GPP TSG-RAN WG4 Meeting #105-e	R4-2219659

# Toulouse, 14th – 18th Nov, 2022

Title: 	Simulation results on UE FeMIMO demod mTRP

Source: 	Huawei, HiSilicon

Agenda item:	6.5.3.1.2

Document for:	Information

Background

During RAN4#104bis-e meeting, WF [1] on UE demodulation and CSI requirements for FeMIMO was approved. In this contribution, we provide our updated simulation results about the FeMIMO Multi-TRP demodulation requirements.

Simulation

PDCCH with FDM repetition scheme

Here we provide the updated simulation results for PDCCH with FDM repetition scheme for alignment, as shown in Table 2.1-2 and Figure 2.1-1, based on simulation assumption as shown in Table 2.1-1.

Table 2.1-1 Simulation assumption for PDCCH with FDM repetition scheme

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15 kHz SCS | TDD 30 kHz SCS |
| Repetition transmission schemes | FDM |  |
| CBW | 10 MHz | 40 MHz |
| CORESET RB | 24 | 48 |
| CORESET duration | 2 |  |
| Aggregation level | 2 |  |
| Propagation Condition | TDLA30-10 |  |
| Antenna configuration | 2x2, 2x4 |  |
| CCE to REG mapping type | nonInterleaved |  |
| REG bundle size | 6 |  |
| Payload bits(without CRC) | 39 | 41 |
| Test metric | 1% of Pm-dsg (%) |  |

Table 2.1-2 Simulation results for PDCCH with FDM repetition scheme

| Case Number | Duplex | Antenna configuration | Aggregation level | SNR (dB) |
| --- | --- | --- | --- | --- |
| 1 | FDD | 2x2 | 2 | -1.71 |
| 2 | FDD | 2x4 | 2 | -5.78 |
| 3 | TDD | 2x2 | 2 | -1.37 |
| 4 | TDD | 2x4 | 2 | -5.73 |

Figure 2.1-1 Simulation results for PDCCH with FDM repetition scheme

Conclusion

In this contribution, we provide our updated simulation results on FeMIMO Multi-TRP demodulation requirements.

Reference

R4-2217440, WF on UE demodulation and CSI requirements for FeMIMO, RAN4#104bis-e, Samsung