3GPP TSG-RAN WG4 Meeting # 105                	R4-2219720

Toulouse, November 14 – November 18, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	ATP Simulation Results

Agenda item: 	8.17.1

Document for: 	Discussion

Introduction

This paper presents Nokia’s simulation results based on the agreed configurations from the Rel.17 feasibility study on Application Throughput Layer requirement definition. The agreed detailed simulation assumptions can be seen below [1], [2]:

Table 1 Simulation Assumptions used in Rel.17 SI

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 | FR2 |
| Bandwidth |  | MHz | 10 | 40 | 100 |
| Subcarrier spacing |  | kHz | 15 | 30 | 120 |
| Duplex Mode |  |  | FDD | TDD | TDD |
| TDD Slot Configuration |  |  | N/A | 7D1S2U S:6D+4G+4U | DDSU S:11D+3G+0U |
| SNR |  | dB | 0:2:20 | 0:2:20 | 0:2:16 |
| Propagation channel |  |  | TDLA30-5 | TDLA30-5 | TDLA30-35 |
| Antenna configuration |  |  | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2 |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A | Type A | Type A |
|  | Starting symbol (S) |  | 2 | 2 | 2 |
|  | Length (L) |  | 12 | 12 | 12 |
|  | PRB bundling size |  | 2 | 2 | 2 |
|  | PRB bundling type |  | Static | Static | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved | Non-interleaved | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 | Type 1 | Type 1 |
|  | Number of additional DMRS |  | 1 | 1 | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 | 1 | 1 |
| CSI measurement channels (Note 2) |  |  | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-1 Rank 2: TBS.2-2 | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-3 Rank 2: TBS.2-4 | As specified in Table A.4-1 of TS 38.101-4: Rank 1: TBS.1-1 Rank 2: TBS.1-2 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5, (4) | Row 5, (4) | Row 5, (8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 | 9 | 13 |
|  | CSI-RS periodicity and offset | slot | 5/1 | 10/1 | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic | Periodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 2 | 2 | 2 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3 (6) | Row 3 (6) | Row 3 (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic | Periodic | Periodic |
|  | CSI-IM RE pattern |  | Pattern 0 | Pattern 0 | Pattern 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) | (4,9) | (8,13) |
|  | CSI-IM timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| ReportConfigType |  |  | Aperiodic | Aperiodic | Aperiodic |
| CQI-table |  |  | Table 2 | Table 2 | Table 2 |
| reportQuantity |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| Sub-band Size |  | RB | 8 | 16 | 8 |
| csi-ReportingBand |  |  | 1111111 | 1111111 | 111111111 |
| CSI-Report periodicity and offset |  | slot | Not configured | Not configured | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 | 9 | 7 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 10) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 | 1 | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  | Codebook Mode |  | 1 | 1 | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A | N/A | N/A |
|  | CodebookSubsetRestriction |  | Not configured | Not configured | Not configured |
|  | RI Restriction |  | N/A | N/A | N/A |
| Physical channel for CSI report |  |  | PUSCH | PUSCH | PUSCH |
| CQI/RI/PMI delay |  | ms | 6 | 5.5 | 1.375 |
| Maximum number of HARQ transmission |  |  | 1 | 1 | 1 |
| Test metric |  |  | T% of max throughput at target SNR. |  |  |
| Note 1: Other common test parameters are defined in Section 6.1.2 of 38.101-4 for Tests 1 and 2 and Section 8.1.2 of 38.101-4 for Test 3. Note 2: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition, CSI-RS for tracking and CSI-RS for beam refinement (for Test 3 only). |  |  |  |  |  |

In addition, the following were agreed [3]:

- Number of slots containing PDSCH grant

  - Skip scheduling PDSCH grant on special slots and slots containing SSB, TRS, CSI-RS for CSI reporting.

    - This results in number of allocated full DL slots within 20ms as: 14 for FR1 FDD, 22 for FR1 TDD and 58 for FR2 TDD.

- Processing Delay (Based on aperiodic CSI reporting)

  - FR1 FDD: 6ms – 6slots

  - FR1 TDD: 5.5ms – 11slots

  - FR2 TDD: 1.375ms – 11slots

Simulation Results

The following sections present Nokia’s simulation results under the agreed simulation assumptions detailed above.  Each simulation test is characterized by its absolute physical layer throughput, block error rate (BLER), median rank indicator (RI) and median channel quality indicator (CQI). Simulation results show that Nokia is well aligned with other companies, as illustrated for some representative cases below.

Test 1 - FR1 FDD

Table 1: ATP Link Adaptation results for Test 1 – FR1 FDD

| SNR (dB) | 2x2 |  |  |  | 2x4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER | Median Rank | Median CQI | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 3.32 | 0.13 | 1 | 3 | 6.33 | 0.12 | 2 | 3 |
| 2 | 4.47 | 0.14 | 1 | 4 | 8.01 | 0.12 | 2 | 3 |
| 4 | 5.76 | 0.16 | 1 | 5 | 10.79 | 0.17 | 2 | 4 |
| 6 | 7.68 | 0.14 | 1 | 6 | 13.90 | 0.17 | 2 | 5 |
| 8 | 9.42 | 0.13 | 1 | 7 | 17.90 | 0.13 | 2 | 6 |
| 10 | 11.29 | 0.12 | 1 | 8 | 21.69 | 0.09 | 2 | 7 |
| 12 | 13.19 | 0.13 | 1 | 9 | 25.15 | 0.11 | 2 | 8 |
| 14 | 16.26 | 0.19 | 2 | 6 | 28.42 | 0.14 | 2 | 9 |
| 16 | 19.64 | 0.18 | 2 | 7 | 31.61 | 0.03 | 2 | 9 |
| 18 | 21.89 | 0.19 | 2 | 8 | 35.83 | 0.04 | 2 | 10 |
| 20 | 23.27 | 0.24 | 2 | 9 | 39.95 | 0.05 | 2 | 11 |

Figure 1: Throughput (Mbps) – FR1 FDD 2x4

Figure 2: BLER – FR1 FDD 2x4

Test 2 - FR1 TDD

Table 2: ATP Link Adaptation results for Test 2 – FR1 TDD

| SNR (dB) | 2x2 |  |  |  | 2x4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER | Median Rank | Median CQI | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 9.45 | 0.09 | 1 | 2 | 20.3338 | 0.11 | 2 | 3 |
| 2 | 13.65 | 0.18 | 1 | 3 | 26.7903 | 0.13 | 2 | 3 |
| 4 | 19.24 | 0.17 | 1 | 5 | 35.351 | 0.13 | 2 | 4 |
| 6 | 24.12 | 0.17 | 1 | 6 | 44.6172 | 0.19 | 2 | 5 |
| 8 | 29.48 | 0.13 | 1 | 7 | 54.647 | 0.18 | 2 | 6 |
| 10 | 35.88 | 0.17 | 1 | 4 | 65.1881 | 0.11 | 2 | 7 |
| 12 | 42.66 | 0.15 | 2 | 9 | 76.8401 | 0.14 | 2 | 8 |
| 14 | 50.43 | 0.21 | 2 | 6 | 89.4876 | 0.19 | 2 | 8 |
| 16 | 58.84 | 0.19 | 2 | 7 | 100.6422 | 0.19 | 2 | 9 |
| 18 | 67.98 | 0.19 | 2 | 8 | 113.4398 | 0.03 | 2 | 10 |
| 20 | 75.92 | 0.21 | 2 | 9 | 122.4378 | 0.11 | 2 | 11 |

Test 3 – FR2-1 TDD

Table 3: ATP Link Adaptation results for Test 3 – FR2-1 TDD

| SNR (dB) | 2x2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 15.40 | 0.17 | 1 | 3 |
| 2 | 21.17 | 0.18 | 1 | 3 |
| 4 | 27.99 | 0.20 | 1 | 4 |
| 6 | 36.01 | 0.23 | 1 | 5 |
| 8 | 45.15 | 0.23 | 1 | 7 |
| 10 | 55.47 | 0.18 | 2 | 4 |
| 12 | 67.54 | 0.18 | 2 | 5 |
| 14 | 79.84 | 0.21 | 2 | 6 |
| 16 | 94.11 | 0.20 | 2 | 7 |

Figure 3: Throughput (Mbps) – FR2 TDD 2x2

Figure 4: BLER – FR2 TDD 2x2

Conclusion

This paper has presented Nokia’s simulation results based on the agreed configuration from the discussion of feasibility to define requirements for Application Layer Throughput with link adaptation.

For our presented results, we see a good alignment with companies results provided in the Rel.17 SI.

References

- 3GPP TR 37.901-5 “Study on 5G NR User Equipment (UE) application layer data throughput performance (Release 16)”, V16.8.0.

- R4-2106122 “WF on 5G NR UE Application Layer Data Throughput Performance”, Qualcomm, RAN4 #98bis-e, April 2021.

- R4-2108102 “WF on 5G NR UE Application Layer Data Throughput Performance”, Qualcomm, RAN4 #99-e, May 2021.