3GPP TSG-RAN WG4 Meeting # 105	R4-2219755

# Toulouse, France, Nov 14 - 18, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | <CR#> | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS 38.101-1 to update delta TIB and RIB template for SUL combinations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SUL_combos_R18-Core |  |  |  |  |  | Date: |  |  | 2022-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To unify the template as the delta TIB and RIB in inter-band CA/DC configurations, it is suggested to apply the same template as CA/DC configurations for SUL configurations in Rel-18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To update the delta TIB and RIB template for SUL configurations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The delta TIB and RIB template for SUL configurations will be different from which for CA/DC configurations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C.2, 7.3C.3.2.1, 7.3C.3.2.2, 7.3C.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... 38.521-1 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

6.2C.2	ΔTIB,c

For the UE which supports SUL band combination, ΔTIB,c in Tables below applies. Unless otherwise stated, ΔTIB,c is set to zero.

Table 6.2C.2-1: ΔTIB,c due to SUL

| Band combination for SUL | ΔTIB,c for NR band / SUL band (dB)3 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration4 |  |
| SUL_n41-n80 | 0.31 / 0.82 | 0.5 |
| SUL_n41-n81 | 0.3 | 0.3 |
| SUL_n41-n83 | 0.3 | 0.3 |
| SUL_n41-n97 | 0.5 | 0.5 |
| SUL_n41-n98 | 0.5 | 0.5 |
| SUL_n41-n99 | 0.41 / 0.92 | 0.3 |
| SUL_n48-n99 | 0.6 | 0.8 |
| SUL_n77-n80 | 0.8 | 0.6 |
| SUL_n77-n84 | 0.8 | 0.6 |
| SUL_n77-n99 | 0.6 | 0.8 |
| SUL_n78-n80 | 0.8 | 0.6 |
| SUL_n78-n81 | 0.8 | 0.6 |
| SUL_n78-n82 | 0.8 | 0.6 |
| SUL_n78-n83 | 0.8 | 0.5 |
| SUL_n78-n84 | 0.8 | 0.3 |
| SUL_n78-n86 | 0.8 | 0.6 |
| SUL_n79-n83 | 0.8 | 0.5 |
| SUL_n79-n97 | 0.8 | 0.3 |
| SUL_n79-n98 | 0.8 | 0.3 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2515 MHz. NOTE 3: “-” denotes ΔTIB,c = 0. NOTE 4: The component band order in the configuration should be listed by the order of NR band and SUL band, such as for SUL_n41-n81 the band order from left to right is n41 and n81. |  |  |

Table 6.2C.2-2: ΔTIB,c for SUL band combination (Three bands)

| Band combination for SUL | ΔTIB,c for NR bands / SUL band (dB)3 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration4 |  |  |
| CA_n1_SUL_n78-n80 | 0.6 | 0.8 | 0.6 |
| CA_n1_SUL_n78-n84 | 0.6 | 0.8 | 0.6 |
| CA_n3_SUL_n41-n80 | 0.5 | 0.31 / 0.82 | 0.5 |
| CA_n3_SUL_n78-n80 | 0.6 | 0.8 | 0.6 |
| CA_n3_SUL_n79-n80 | 0.3 | 0.8 | 0.3 |
| CA_n28_SUL_n41-n83 | 0.3 | 0.3 | 0.3 |
| CA_n28_SUL_n79-n83 | 0.5 | 0.8 | 0.5 |
| CA_n41_SUL_n79-n80 | 0.31 / 0.82 | 0.8 | 0.3 |
| CA_n41_SUL_n79-n83 | 0.3 | 0.8 | 0.5 |
| CA_n41_SUL_n79-n97 | 0.5 | 0.5 | 0.5 |
| CA_n79_SUL_n41-n80 | 0.31 / 0.82 | 0.8 | 0.3 |
| CA_n79_SUL_n41-n83 | 0.3 | 0.8 | 0.5 |
| CA_n79_SUL_n41-n97 | 0.5 | 0.5 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2515-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2515 MHz. NOTE 3: “-” denotes ΔTIB,c = 0. NOTE 4: The component band order in the configuration should be listed by the order of NR bands and SUL band, such as for CA_n1_SUL_n78-n80 the band order from left to right is n1, n78 and n80. |  |  |  |

Table 6.2C.2-3: ΔTIB,c for SUL band combination (Four bands)

| Band combination for SUL | ΔTIB,c for NR bands / SUL band (dB)1 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |  |  |
| CA_n28-n79_SUL_n41-n83 | 0.3 | 0.3 | 0.5 | 0.3 |
| CA_n28-n41_SUL_n79-n83 | 0.3 | 0.3 | 0.5 | 0.3 |
| NOTE 1: “-” denotes ΔTIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of NR bands and SUL band, such as for CA_n28-n79_SUL_n41-n83 the band order from left to right is n28, n41, n79 and n83. |  |  |  |  |

<< Unchanged sections omitted >>

7.3C.3	ΔRIB,c for SUL

7.3C.3.1	General

For a UE supporting a SUL configuration, the ΔRIB,c applies for both SC and SUL operation.

7.3C.3.2	SUL band combination

For the UE which supports SUL band combiantion, the minimum requirement for reference sensitivity in clause 7.3C.2 shall be increased by the amount given in ΔRIB,c defined in clause 7.3C.3.2 for the applicable operating bands. Unless otherwise stated, ΔRIB,c is set to zero.

In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

-	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔRIB,c shall be the average value for all band combinations defined in clause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.101-3 [3], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔRIB,c among the different supported band combinations involving such band shall be applied

-	When the operating band frequency range is > 1 GHz, the applicable additional ΔRIB,c shall be the maximum value for all band combinations defined in clause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.101-3 [3] for the applicable operating bands.

7.3C.3.2.1	ΔRIB,c  for two bands

Table 7.3C.3.2.1-1: ΔRIB,c due to SUL (two bands)

| Band combination for SUL | ΔRIB,c for NR band / SUL band (dB)2 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration3 |  |
| SUL_n41-n80 | 0.51 | - |
| SUL_n41-n95 | 0.2 | - |
| SUL_n41-n98 | 0.2 | - |
| SUL_n48-n99 | 0.5 | - |
| SUL_n77-n80 | 0.5 | - |
| SUL_n77-n84 | 0.5 | - |
| SUL_n77-n99 | 0.5 | - |
| SUL_n78-n80 | 0.5 | - |
| SUL_n78-n81 | 0.5 | - |
| SUL_n78-n82 | 0.5 | - |
| SUL_n78-n83 | 0.5 | - |
| SUL_n78-n84 | 0.5 | - |
| SUL_n78-n86 | 0.5 | - |
| SUL_n79-n83 | 0.5 | - |
| SUL_n79-n97 | 0.5 | - |
| SUL_n79-n98 | 0.5 | - |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 2: “-” denotes ΔRIB,c = 0. NOTE 3: The component band order in the configuration should be listed by the order of NR band and SUL band, such as for SUL_n41-n80 the band order from left to right is n41 and n80. |  |  |

7.3C.3.2.2	ΔRIB,c  for three bands

Table 7.3C.3.2.2-1: ΔRIB,c due to SUL (three bands)

| Band combination for SUL | ΔRIB,c for NR bands / SUL band (dB)2 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration3 |  |  |
| CA_n1_SUL_n78-n80 | 0.2 | 0.5 | - |
| CA_n1_SUL_n78-n84 | 0.2 | 0.5 | - |
| CA_n3_SUL_n41-n80 | - | 0.51 | - |
| CA_n3_SUL_n78-n80 | 0.2 | 0.5 | - |
| CA_n3_SUL_n79-n80 | - | 0.5 | - |
| CA_n28_SUL_n41-n83 | 0.2 | - | - |
| CA_n28_SUL_n79-n83 | 0.2 | 0.5 | - |
| CA_n41_SUL_n79-n80 | 0.5 | 0.5 | - |
| CA_n41_SUL_n79-n83 | 0.5 | 0.5 | - |
| CA_n41_SUL_n79-n97 | - | 0.8 | - |
| CA_n79_SUL_n41-n80 | 0.5 | 0.5 | - |
| CA_n79_SUL_n41-n83 | 0.5 | 0.5 | - |
| CA_n79_SUL_n41-n97 | - | 0.8 | - |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 2: “-” denotes ΔRIB,c = 0. NOTE 3: The component band order in the configuration should be listed by the order of NR bands and SUL band, such as for CA_n1_SUL_n78-n80 the band order from left to right is n1, n78 and n80. |  |  |  |

7.3C.3.2.3	ΔRIB,c  for four bands

Table 7.3C.3.2.3-1: ΔRIB,c due to SUL (four bands)

| Band combination for SUL | ΔRIB,c for NR bands / SUL band (dB)1 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |  |  |
| CA_n28-n79_SUL_n41-n83 | 0.2 | - | 0.8 | - |
| CA_n28-n41_SUL_n79-n83 | 0.2 | - | 0.8 | - |
| NOTE 1: “-” denotes ΔRIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of NR bands and SUL band, such as for CA_n28-n79_SUL_n41-n83 the band order from left to right is n28, n41, n79 and n83. |  |  |  |  |

==============================================================

<< End of Changes >>