3GPP TSG-RAN4 Meeting #105	R4-2219877

Nov 2022, Toulouse, France

Title:	UE requirements during overlaps due to NZ-MTTD

Source:	Qualcomm Incorporated

Agenda item:	8.20.2.2

Release:	Rel-18

WI: 	NR_MC_enh

Lead WG: 	RAN1

Document for:	Approval

# 1.	Introduction

In previous RAN4#104Bis-e agreements were made on the switching period. Namely in [13] it was agreed that switching period is per band pair and then in [16,18] it was agreed that the switching period is configured for one of the bands in each switching band pair for single TAG. The WF and LS [17] had also other agreements. This paper notes and observes some cases as a result of the agreements.

# 2. 	Discussion

## 2.1	Discussion on NZ-MTTD

The 2-TAG case as opposed to single TAG case only differs from any other case such way that the transmit timing maybe different by set MTTD time as specified for the UE in 38.133. For 3&4 band tx switching configuration, UE maybe configured for multiple switching periods in a band configuration. Uplink transmission overlapping on any band during any configured switching period should not occur and the switching periods can be of different lengths and they can be positioned on bands such way that they overlap perfectly, partially or do not overlap at all. With these assumptions, the requirements setting for single Tag and 2 Tag are very similar since there has to be language for UE behaviour when UE does not need to transmit because switching period is not time aligned with the switching period on an other band. This is also discussed in our companion paper for single TAG case submitted for this meeting. The difference with requirements for NZ-MTTD is that also the difference band timings has to be accounted for but in ran4 domain, it just means adding language such as we propose in proposal 2 in this paper.

Observation : Ran4 requirements for single Tag and NZ-MTTD are very similar since even with single Tag switching periods maybe mis-aligned in time and for UE RF, the resulting behaviour is the same

The 2-Tag case did not agree to configure the switching period but as we state here, it does not change the issue much. We do make the assumption that the sw period needs to be somewhere and that other band do not have to transmit during the switching period.

## 2.2	Discussion on specific language for UE requirements

The discussion on how to specify the UE behaviour in RAN4 requirements did not conclude in previous meeting. The question on how to spell out what happens during the overlapping time with switching period is open. It seems to be clear that no-one is expecting UE to use neither TX chain for transmissions during the overlapping time, so UE behaviour is not open but there is an underlying discussion between the specification language.

“UE not expected to transmit” – can be considered to mean that UE is also not allowed to transmit. This come relevant in case there is a conflicting grant, i.e. for one reason or an other, UE is scheduled to transmit in violation of the switching time. If specification says “UE is not expected to…” then it is assumed that UE detects the conflicts and behaves as expected, i.e. does not transmit. This then means that UE is essentially creating the frame instead of the network.

“UE is not expected to be scheduled..” – if overlapping conflicting scheduling happens anyway, with this language the UE behaviour is unspecified and UE can choose to transmit as scheduled if it sees better than to drop the transmissions. Or raise an error flag in the configuration. Regardless, this way simplifies the UE implementation greatly since it does not need to check the scheduling for all bands especially in mTAG configuration before transmitting on any.

“UE may omit transmission” – the language “may” leaves the UE options what to do since it is not mandating UE behaviour.

Our view is that the UE requirements should be written as the first option since this deals with an commonly agreed case that should not happen and UE should not be mandated to fix possible conflicts in scheduling.

Proposal 1: UE requirements are written assuming known network behaviour i.e.not using  “UE is not expected to ..” language.

Or if UE expectations are very important, then ran4 can use language “UE is not expected to be scheduled..”.

Our view is such that the discussion on the UE behaviour during unspecified overlap situation is maybe not even needed since ran4 should focus on specifying when UE should transmit, instead when it should not.  It would be easier to concentrate on wording “UE is expected to transmit on target carrier after switching period…”. For example, for the case discussed in section 2.1 of this document, the specification language could say:

Proposal 2: Language to be used for specifying UE behaviour for concurrent switching periods of different lengths:

“UE shall be capable of starting transmission after the end of switching periods on all bands that are configured for uplink in tx switching. Transient periods are not included in the switching periods.”

# Conclusion

Observation : Ran4 requirements for single Tag and NZ-MTTD are very similar since even with single Tag switching periods maybe mis-aligned in time and for UE RF, the resulting behaviour is the same

Proposal 1: UE requirements are written assuming known network behaviour i.e.not using  “UE is not expected to ..” language.

Proposal 2: Language to be used for specifying UE behaviour for concurrent switching periods of different lengths:

“UE shall be capable of starting transmission after the end of switching periods on all bands that are configured for uplink in tx switching. Transient periods are not included in the switching periods.”

# References

[1]	RP-221435, “Revised WID on Multi-carrier enhancements”, NTT DOCOMO, INC., 3GPP TSG RAN Meeting #96, Budapest, Hungary, June 6 - 9, 2022

[2] R4-2211508 (R1-2205502), “LS on UL Tx switching across 3 or 4 bands in Rel-18”, RAN1, Ran4#104, Aug 2022

[3]	R4-2204605, “Introduction of TX switching for non-collocated UL CA”, Ericsson, 3GPP TSG-RAN WG4 Meeting#102-e, Electronic meeting, 21 February – 3 March 2022

[5]	R1-2204724, “On multi-carrier UL Tx switching scheme”, MediaTek Inc.

[6]	RP-221880, “Discussion on target scenarios for Rel-18 UL Tx switching in NR Multi-carrier enhancements WI”, 3GPP TSG RAN Meeting #96, Budapest, Hungary, June 6 - 9, 2022

[7]	R1-2205052, “Discussion on Rel-18 UL Tx switching”, Qualcomm, 3GPP TSG RAN WG1 #109-emeeting, e-Meeting, May 9th – May 20th, 2022

[8]	R4-2214044, “UL TX switching requirements for two TAGs”, RAN4#104e, Online, Aug 2022

[9]	R4-2214043, “UL TX switching requirements for multiple TAGs”, RAN4#104e, Online, Aug 2022

[10]	RP-222251, “Revised WID on Multi-carrier enhancements”, NTT DOCOMO, INC., 3GPP TSG RAN Meeting #97-e, Electronic meeting, September 12 - 16, 2022

[11]	RP-222672, “Moderator's summary for discussion [97e-19-R18-Multicarrier]”, Moderator (NTT DOCOMO, INC.), 3GPP TSG RAN Meeting #97-e, Electronic meeting, September 12 - 16, 2022

[12]	R4-2214463, “WF on UL Tx switching across 3/4 bands with single TAG”, China Telecom, RAN4#104e, Online, Aug 2022

[13]	R4-2214464, “Reply LS on UL Tx switching across 3 or 4 bands”, China Telecom, RAN4#104e, Online, Aug 2022

[14]	R4-2215163, “WF on UL Tx switching with multiple TAGs”, Ericsson, RAN4#104e, Online, Aug 2022

[15]	R4-2215306, “LS on switching option capability for UL 2Tx-2Tx switching”, RAN2, RAN4#104Bis-e, Online, Oct 2022

[16]	R4-2217740, “WF on UL Tx switching across 3/4 bands with single TAG, China Telecom, RAN4#104Bis-e, Online, Oct 2022

[17]	R4-2217741, “LS on Rel-18 UL Tx switching”, China Telecom, RAN4#104Bis-e, Online, Oct 2022

[18]	R4-2217742, “WF on UL Tx switching with multiple TAGs”, Ericsson, RAN4#104Bis-e, Online, Oct 2022