3GPP TSG-RAN WG4 Meeting #105	R4-2219887

Electronic Meeting, 14 Nov. – 18 Nov. 2022

Agenda Item:	8.22.2.2

Source:	Charter Communications, Inc.

Title:	Discussion on priority of MUSIM

Document for:	Decision

# Introduction

In RAN4 #104-bis-e meeting, a WF on RRM requirements for Rel-17 MUSIM gaps was agreed [1].

Issue 1-1-3: Priority of MUSIM against other legacy gaps - Proposals: - P1: Up to network configuration - Up to NW A configuration if priority field is introduced to MUSIM, otherwise use default priority - P2: If an explicit priority level is not provided for MUSIM gaps via signalling, MUSIM gaps are assumed to have higher priority than all measurement gaps configured by the network. - P3: Aperiodic MUSIM gap is always prioritized over legacy MG in NW A. - P4: When MUSIM gaps collide with legacy MG - MUSIM paging and AGC occasions should have higher priority than NW-A MG - The priority between other MUSIM gaps and legacy MG can be indicated by NW

# Discussion

To start off, we would like to go back to the basic. What are the MUSIM gaps and what are they intended for. A MUSIM gap is requested by the UE and it is the user of the UE device that wishes to have multiple SIM in one device. It is not any of the networks. Hence, MUSIM gaps requested should be prioritized. Network A should be lean to such request.

Hence, we believe MUSIM gaps always should have higher priority than measurement gaps configured by the Network (NW A).

Proposal 1: Aperiodic MUSIM gap is always prioritized over legacy MG in NW A.

Proposal 2: If an explicit priority level is not provided for MUSIM gaps, MUSIM gaps are assumed to have higher priority than all measurement gaps configured by the network.

If it is up to NW A to configure priority, the UE shall send its preferred priority level in its request. Then NW A gets a notion of importance of the certain MUSIM gap to make a more consent decision for the configuration. After all, it is important for NW A, to the best of their ability, to serve their UE the best way possible.

Proposal 3: A UE shall indicate/notify the NW A of a preferred priority of its MUSIM gaps. NW A shall take this into consideration when setting up the configurations.

# Reference

[1] R4-2217261, WF on RRM requirements for Rel-17 MUSIM gaps, vivo, RAN4#104-bis-e