3GPP TSG-RAN WG4 Meeting # 105	R4-2219920

Meeting, November 14 – November 18, 2022

Agenda Item: 8.9.2.2

Source: MediaTek Inc.

Title: Discussion on L1 part enhancement for FR2 SCell activation

Document for: Discussion

# Introduction

In R15, SCell activation delay requirement is introduced, and in R16/R17, some activation requirements are enhanced to accelerate the activation procedures. The delay requirement may be different according to the various conditions, e.g., frequency range, known/unknown cell and activating serving cell on the same band, etc. However, some of them, especially for unknown case, are still too long. Therefore, following the discussion in the last meeting, in this paper we further discuss L1 part enhancement for unknown SCell activation in FR2. The WF from the last meeting is captured in [1].

# Discussion

In SCell activation delay, L1-RSRP measurement is performed for the unknown SCell in FR2 after cell search is completed. Then L1-RSRP report is sent to the network for TCI indication as shown in Figure 1.

TCI activation and CSI-RS meas. Max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay)

L1-RSRP meas. & report

Cell search

MAC-CE processing & RF retuning

THARQ

MAC-CE For SCell activation

CSI reporting

AGC

Figure 1: Unknown SCell activation delay in FR2 (when P CSI-RS is used).

## 2.1 Open issues related to L1 part

Several issues have been discussed in the last meeting and captured in the WF [1]. Here we discuss the open issues related to L1 part.

Issue 3-1-3: Whether and how to skip L1-RSRP measurement of FR2 unknown SCell activation? Agreements: - RAN4 to discuss the conditions for skipping L1-RSRP measurement when activating an FR2 unknown SCell.

For Issue 3-1-3, in our view, there are two potential solutions to skip L1-RSRP measurements when activating unknown SCell in FR2:

- Contention based random access (CBRA) approach: CBRA can be used to activate first unknown SCell in one band, which can help skipping L1 part and TCI state indication, and as a result, enhance the overall activation delay for unknown SCell (detailed discussion is provided in our paper in 8.9.2.3 AI).

- RSRP report before L1 part: the NW can trigger the UE to report the latest RSRP measurements before L1 part (L1-RSRP measurement and report), which can help skipping L1 part for unknown SCell activation in FR2.

Note: both approaches will have an impact on RAN1/RAN2 work.

Proposal 1: Two potential solutions to skip L1-RSRP measurements when activating unknown SCell in FR2:

- Contention based random access (CBRA) approach: CBRA can be used to activate first unknown SCell in one band, which can help skipping L1 part and TCI state indication, and as a result, enhance the overall activation delay for unknown SCell.

- RSRP report before L1 part: the NW can trigger the UE to report the latest RSRP measurements before L1 part (L1-RSRP measurement and report), which can help skipping L1 part for unknown SCell activation in FR2.

Note: both approaches will have an impact on RAN1/RAN2 work.

Issue 3-1-4: Prioritization enhancement for L1-RSRP measurement of FR2 unknown SCell activation Agreement: - L1-RSRP measurement is performed in non-DRX mode even DRX is configured. - FFS on whether prioritization needs to be changed between L1-RSRP measurements and L3 measurements during FR2 unknown SCell activation

For Issue 3-1-4, if L1-RSRP measurement is prioritised over L3 measurements to enhance L1 part of SCell activation, it could have negative impact on the mobility performance (e.g., handover). Therefore, it is preferred not to change the priority between L1-RSRP measurements and L3 measurements during FR2 unknown SCell activation.

Proposal 2: The priority between L1-RSRP measurements and L3 measurements during FR2 unknown SCell activation should not be changed as it could impact the mobility requirements.

Issue 3-2-1: Fine timing tracking for SSB corresponding to the TCI state during FR2 unknown SCell activation FFS: - Option 1 (Apple, QC, Intel, HW, ZTE, MTK): Fine timing tracking is still needed no matter whether TCI activation is skipped or not. - Option 2 (Apple, QC, Intel, Xiaomi, HW, OPPO, vivo, ZTE, MTK): Skipping fine timing tracking for SSB corresponding to TCI state (by reusing SSB timing from L3/ L1 measurements) is not recommended, because during L1/L3 measurements UE tracking performance is not accurate enough. In addition, this skipping does not reduce the delay much (only 1*Trs) compared to its impact on acquiring the accurate timing. - Option 3 (Ericsson): RAN4 to confirm, UE timing assumption when performing L1-RSRP measurements. - Option 4 (Apple): for unknown R18 FR2 SCell activation enhancement, skip fine timing tracking for SSB corresponding to the TCI state, if SSB timing from L3 measurement stage or SSB timing from L1-RSRP measurement stage can be reused.

Reusing SSB timing from L3 or L1 measurements to skip the fine timing tracking for SSB corresponding to TCI state is not recommended, because during L1/L3 measurements UE tracking performance for SSB timing will not be accurate enough, and therefore fine timing tracking is still required to be accomplished. In addition, skipping fine timing tracking does not reduce the delay much (only 1*Trs) compared to its impact on acquiring the accurate timing. Therefore, we still support Option 2 and we think Option 1 is same.

Proposal 3: Skipping fine timing tracking for SSB corresponding to TCI state (by reusing SSB timing from L3/ L1 measurements) is not recommended, because during L1/L3 measurements UE tracking performance is not accurate enough. In addition, this skipping does not reduce the delay much (only 1*Trs) compared to its impact on acquiring the accurate timing.

Issue 3-2-2: TCI activation enhancement during FR2 unknown SCell activation FFS: - Option 1 (Apple, Intel, CMCC, Xiaomi, HW, OPPO, CTC, MTK): the uncertainty of TCI configuration/activation can be saved when TCI of PDCCH/PDSC/CSI-RS is associated with the best L1-RSRP report. - Option 1a(Apple, Intel, HW, CTC): for FR2 unknown SCell activation enhancement, the TCI of PDCCH/PDSC/CSI-RS is associated with the best L1-RSRP report if no MAC CE or RRC indication for TCI is sent to UE. - Option 2 (vivo): During the Scell activation, only the TCI from CSI-RS used for CQI needs to be configured. The PDCCH/PDSCH can follow the same TCI state information as CSI-RS. In this way, the PDCCH/PDSCH TCI configuration can be saved and the Scell activation delay can be reduced accordingly. - Option 3 (QC, Ericsson, Nokia, vivo, ZTE): Gain in delay reduction is quite small for skipping TCI state indication and RAN4 not consider TCI state skipping.

For Issue 3-2-2, we support Option 1. Saving the time spent on TCI activation based on MAC CE (which include Tuncertinity_MAC, THARQ and MAC CE processing) by assuming that TCI of PDCCH/PDSC/CSI-RS is associated with the best L1-RSRP report can enhance SCell activation delay in FR2.

Proposal 4: The uncertainty of TCI configuration/activation can be saved when TCI of PDCCH/PDSC/CSI-RS is associated with the best L1-RSRP report.

Issue 3-3-2: Aperiodic RS for L1-RSRP measurement during FR2 unknown SCell activation FFS: - Option 1 (Apple): Use AP CSI-RS for L1-RSRP measurement if UE can indicate the completion of L3 stage or can indicate the readiness of L1 measurement. - Option 2 (Ericsson, LGE, Nokia): - RAN4 to study usage of AP-RS and A-TRS for L1-RSRP measurement. - RAN4 to send LS to RAN1 if A-TRS can be used for L1-RSRP measurement. If it cannot be used as it is, RAN4 to ask if it can be enhanced to support L1-RSRP measurement on the A-TRS.

For Issue 3-3-2, the problem with AP RS is that it is only one-shot. Network has no idea when to trigger it because the exact UE processing during the SCell activation is unknown to the network. Even if we have a reference UE behaviour during the discussion, but UE is not mandated to follow the reference behaviour (e.g., UE can do faster, if possible). On Option 1, the key point is whether the if condition can be true. It seems some additional new reporting mechanism is needed for UE to tell network about the readiness of L1 measurement. On Option 2, it is pending on the feedback from RAN1. It should be fine to send an LS to RAN1 to check.

Proposal 5: AP-CSI-RS and/or A-TRS based fast Scell activation is not applied to L1 part for unknown FR2 Scell activation enhancement.

# Summary

In this paper, the discussion of SCell activation delay reduction for L1 part is provided. The following proposals were introduced:

Proposal 1: Two potential solutions to skip L1-RSRP measurements when activating unknown SCell in FR2:

- Contention based random access (CBRA) approach: CBRA can be used to activate first unknown SCell in one band, which can help skipping L1 part and TCI state indication, and as a result, enhance the overall activation delay for unknown SCell.

- RSRP report before L1 part: the NW can trigger the UE to report the latest RSRP measurements before L1 part (L1-RSRP measurement and report), which can help skipping L1 part for unknown SCell activation in FR2.

Note: both approaches will have an impact on RAN1/RAN2 work.

Proposal 2: The priority between L1-RSRP measurements and L3 measurements during FR2 unknown SCell activation should not be changed as it could impact the mobility requirements.

Proposal 3: Skipping fine timing tracking for SSB corresponding to TCI state (by reusing SSB timing from L3/ L1 measurements) is not recommended, because during L1/L3 measurements UE tracking performance is not accurate enough. In addition, this skipping does not reduce the delay much (only 1*Trs) compared to its impact on acquiring the accurate timing.

Proposal 4: The uncertainty of TCI configuration/activation can be saved when TCI of PDCCH/PDSC/CSI-RS is associated with the best L1-RSRP report.

Proposal 5: AP-CSI-RS and/or A-TRS based fast Scell activation is not applied to L1 part for unknown FR2 Scell activation enhancement.

# Reference

- R4-2217249, WF on R18 eFeRRM - FR2 SCell activation enhancement, Apple.