3GPP TSG-RAN WG4 Meeting #105                           	R4-2219980

Toulouse, France, November 14 – November 18, 2022

Source: 	Huawei, HiSilicon

Title: 	Initial discussion on the conformance testing specification TS 36.181 for the IoT NTN SAN RF

Agenda Item:	9.5.4

Document for:	Discussion

# Introduction

Referring to the NTN IoT WID in [1], TS 36.181 was considered as one of the new specifications to be introduced:

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| TS | 36.102 | Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception for satellite access | RAN#98 | RAN#98 | Led by RAN4. Rapporteur: Daniel Hsieh, Daniel.hsieh@mediatek.com Core part |
| TS | 36.108 | Evolved Universal Terrestrial Radio Access (E-UTRA); Satellite Access Node radio transmission and reception | RAN#98 | RAN#98 | Led by RAN4. Rapporteur: Fei Xue, xue.fei25@zte.com.cn Core part |
| TS | 36.181 | Evolved Universal Terrestrial Radio Access (E-UTRA); Satellite Access Node conformance testing | RAN#100 | RAN#100 | Led by RAN4. Rapporteur: Michal Szydelko, michal.szydelko@huawei.com Performance part |

According to the work-plan in [2], the Performance part of the WI will start at RAN4#106 (02/2023) meeting. In order to reassure fruitful progress of TS 36.181 work, as rapporteur of that specification we provide the very initial considerations for the TS 36.181 specification work.

# Discussion

# General aspects

In case of NR NTN, the following SAN types were introduced in TS 38.108 in Rel-17:

- SAN type 1-H

- SAN type 1-O

From the conformance testing point of view, the most important issue to distinguish if the differentiation of the conducted and radiated test methods, with related MU budget derivations.

Observation 1: SAN types considered in NTN IoT WI are the same as SAN types defined in NR NTN WI.

For the purpose of NTN/SAN testing approach alignment to avoid any unnecessary workload overhead, the following is proposed:

Proposal 1: the work on Rel-18 SAN RF conformance testing for NTN IoT for TS 36.181 shall follow progress on the Rel-17 SAN  RF conformance testing for NR NTN in TS 38.181.

Proposal 2: consider TS 38.181 SAN conformance specification as the baseline for the TS 36.181 specification drafting.

The related work-split will be handled during RAN4#106 (02/2023) meeting.

# TS 36.181 skeleton

Draft TS 36.181 specification skeleton is provided in the Annex for discussion. It was drafted based on the TS 38.181 specification, further refined to account for the current version of the TS 36.108, with minor corrections applied. Structure of sections 4 and 5 is to be reused from the core specification TS 36.108.

Proposal 3: align the skeleton of TS 36.181 with the structure of the TS 38.181, where possible.

NOTE:	Conducted and radiated performance requirements sections (i.e. section 8 and 11) are subject to refinements in demod session discussion.

Proposal 4: approve the draft TS 36.181 skeleton as provided in Annex A.

The structure of the proposed skeleton is subject to further refinements.

# Measurement Uncertainty derivations

In order to define test requirements, the minimum requirements from the core specifications TS 36.108 shall account for the Test Tolerance (where allowed, i.e. except the regulatory requirements), which are typically derived from the Measurements Uncertainties (MU) budget calculations.

Clearly, conducted MU values are different (smaller) than radiated MU values due to different test setups.

Therefore, similar to the core requirements derivation, for the test requirements derivation for the NB-IoT and eMTC, the MU values from the EUTRA BS conformance testing specification (TS 36.141) can be used as the baseline for conducted requirements. More specifically, the following sections in TS 36.141 are of most interest as the baseline for conducted MU/TT values:

- TS 36.141, section 4.1.2 Acceptable uncertainty of Test System: This section captures measurement uncertainties for testing BS that supports E-UTRA or E-UTRA with NB-IoT in-band/guard band operation or NB-IoT standalone operation, for both the Tx and Rx requirements (sections 4.1.2.1, and 4.1.2.2, respectively).

- TS 36.141, annex G: Based on MU values in 4.1.2, the Test Tolerance values are captured in this Annex. Test Tolerances in this annex apply to the Test System for testing BS that supports E-UTRA or E-UTRA with NB-IoT in-band/guard band operation or NB-IoT standalone operation.

Proposal 5: For the definition of the conducted SAN RF conformance testing, consider conducted MU/TT values from E-UTRA BS in TS 36.141 as the baseline.

In case of radiated requirements, the baseline is the work done for SAN radiated requirements testing in the NR NTN WI, with the consideration of E-UTRA specifics for the NB-IoT and eMTC requirements.

Proposal 6: For the definition of the radiated SAN RF conformance testing, consider OTA MU/TT values from NR NTN in TS 38.181 as the baseline, subject to further validation for NB-IoT and eMTC testing.

# Conclusions

Based on the above discussion, the following proposals were formulated:

Proposal 1: the work on Rel-18 SAN RF conformance testing for NTN IoT for TS 36.181 shall follow progress on the Rel-17 SAN  RF conformance testing for NR NTN in TS 38.181.

Proposal 2: consider TS 38.181 SAN conformance specification as the baseline for the TS 36.181 specification drafting.

Proposal 3: align the skeleton of TS 36.181 with the structure of the TS 38.181, where possible.

Proposal 4: approve the draft TS 36.181 skeleton as provided in Annex A.

Proposal 5: For the definition of the conducted SAN RF conformance testing, consider conducted MU/TT values from E-UTRA BS in TS 36.141 as the baseline.

Proposal 6: For the definition of the radiated SAN RF conformance testing, consider OTA MU/TT values from NR NTN in TS 38.181 as the baseline, subject to further validation for NB-IoT and eMTC testing.

# References

[1]	RP-221556	WID on NB-IoT/eMTC core & performance requirements for Non-Terrestrial Networks, WID, RAN#96

[2]	R4-2215117	Work Plan for NB-IoT/eMTC for NTN requirements, RAN4#104-e

[3]	TS 38.181 v.0.2.0

# Annex A: TS 36.108 skeleton

Foreword

1	Scope

2	References

3	Definitions, symbols and abbreviations

3.1	Definitions

3.2	Symbols

3.3	Abbreviations

4	General

4.1	Relationship with other core specifications

4.2	Relationship between minimum requirements and test requirements

4.3	Requirement reference points

4.3.1	SAN type 1-H

4.3.2	SAN type 1-O

4.4	Satellite Access Node classes

4.5	Regional requirements

4.6	Applicability of minimum requirements

5	Operating bands and channel arrangement

5.1	General

5.2	Operating bands

5.3	Satellite Access Node channel bandwidth

5.3.1	SAN channel bandwidth for eMTC

5.3.2	SAN channel bandwidth for NB-IoT

5.4	Channel arrangement

5.4.1	Channel spacing

5.4.2	Channel raster

5.4.3	Carrier frequency and EARFCN for eMTC

5.4.4	Carrier frequency and EARFCN for NB-IoT

5.5	Requirements for contiguous spectrum

6	Conducted transmitter characteristics

6.1	General

6.2	Satellite Access Node output power

6.2.1	General

6.2.2	Minimum requirement for SAN type 1-H

6.3	Output power dynamics

6.3.1	General

6.3.2	RE power control dynamic range

6.3.2.1	General

6.3.2.2	Minimum requirement for SAN type 1-H

6.3.3	Total power dynamic range

6.3.3.1	General

6.3.3.2	Minimum requirement for SAN type 1-H

6.4	Transmit ON/OFF power

6.5	Transmitted signal quality

6.5.1	Frequency error

6.5.1.1	General

6.5.1.2	Minimum requirement for SAN type 1-H

6.5.2	Modulation quality

6.5.2.1	General

6.5.2.2	Minimum Requirement for SAN type 1-H

6.5.2.3	EVM frame structure for measurement

6.5.3	Time alignment error

6.5.4	DL RS power

6.5.4.1	General

6.5.4.2	Minimum Requirement for SAN type 1-H

6.6	Unwanted emissions

6.6.1	General

6.6.2	Occupied bandwidth

6.6.2.1	General

6.6.2.2	Minimum requirement for SAN type 1-H

6.6.3	Adjacent Channel Leakage Power Ratio

6.6.3.1	General

6.6.3.2	Minimum requirement for SAN type 1-H

6.6.4	Operating band unwanted emissions

6.6.4.1	General

6.6.4.2	Minimum requirements for SAN type 1-H

6.6.5	Transmitter spurious emissions

6.6.5.1	General

6.6.5.2	Minimum requirements for SAN type 1-H

6.6.5.2.1	General transmitter spurious emissions requirements

6.6.5.2.2	Protection of the own Satellite Access Node receiver

6.6.5.2.3	Additional spurious emissions requirements

6.6.5.2.4	Co-location with other Satellite Access Nodes

6.7	Transmitter intermodulation

7	Conducted receiver characteristics

7.1	General

7.2	Reference sensitivity level

7.2.1	General

7.2.2	Minimum requirements for SAN type 1-H

7.3	Dynamic range

7.3.1	General

7.3.2	Minimum requirements for SAN type 1-H

7.4	In-band selectivity and blocking

7.4.1	Adjacent Channel Selectivity (ACS)

7.4.1.1	General

7.4.1.2	Minimum requirements for SAN type 1-H

7.4.2	In-band blocking

7.5	Out-of-band blocking

7.5.1	General

7.5.2	Minimum requirements for SAN type 1-H

7.6	Receiver spurious emissions

7.7	Receiver intermodulation

7.8	In-channel selectivity

7.8.1	General

7.8.2	Minimum requirements for SAN type 1-H

8	Conducted performance requirements

8.1	General

8.2	Performance requirements for PUSCH

8.3	Performance requirements for PUCCH

8.4	Performance requirements for PRACH

9	Radiated transmitter characteristics

9.1	General

9.2	Radiated transmit power

9.2.1	General

9.2.2	Minimum requirement for SAN type 1-H and SAN type 1-O

9.3	OTA Satellite Access Node output power

9.3.1	General

9.3.2	Minimum requirement for SAN type 1-O

9.4	OTA output power dynamics

9.4.1	General

9.4.2	OTA RE power control dynamic range

9.4.2.1	General

9.4.2.2	Minimum requirement for SAN type 1-O

9.4.3	OTA total power dynamic range

9.4.3.1	General

9.4.3.2	Minimum requirement for SAN type 1-O

9.5	OTA transmit ON/OFF power

9.6	OTA transmitted signal quality

9.6.1	OTA frequency error

9.6.1.1	General

9.6.1.2	Minimum requirement for SAN type 1-O

9.6.2	OTA modulation quality

9.6.2.1	General

9.6.2.2	Minimum requirement for SAN type 1-O

9.6.3	OTA time alignment error

9.6.4	DL RS power

9.6.4.1	General

9.6.4.2	Minimum requirement for SAN type 1-O

9.7	OTA unwanted emissions

9.7.1	General

9.7.2	OTA occupied bandwidth

9.7.2.1	General

9.7.2.2	Minimum requirement for SAN type 1-O

9.7.3	OTA Adjacent Channel Leakage Power Ratio (ACLR)

9.7.3.1	General

9.7.3.2	Minimum requirement for SAN type 1-O

9.7.4	OTA operating band unwanted emissions

9.7.4.1	General

9.7.4.2	Minimum requirement for SAN type 1-O

9.7.5	OTA transmitter spurious emissions

9.7.5.1	General

9.7.5.2	Minimum requirement for SAN type 1-O

9.7.5.2.1	General

9.7.5.2.2	General OTA transmitter spurious emissions requirements

9.7.5.2.3	Protection of the SAN receiver of own

9.8	OTA transmitter intermodulation

10	Radiated receiver characteristics

10.1	General

10.2	OTA sensitivity

10.2.1	General

10.2.2	Minimum requirement for SAN type 1-O

10.3	OTA reference sensitivity level

10.3.1	 General

10.3.2	 Minimum requirement for SAN type 1-O

10.4	OTA dynamic range

10.4.1	General

10.4.2	Minimum requirement for SAN type 1-O

10.5	OTA in-band selectivity and blocking

10.5.1	OTA adjacent channel selectivity

10.5.1.1	General

10.5.1.2	Minimum requirement for SAN type 1-O

10.5.2	OTA in-band blocking

10.6	OTA out-of-band blocking

10.6.1	General

10.6.2	Minimum requirement for SAN type 1-O

10.7	OTA receiver spurious emissions

10.8	OTA receiver intermodulation

10.9	OTA in-channel selectivity

10.9.1	General

10.9.2	Minimum requirement for SAN type 1-O

11	Radiated performance requirements

11.1	General

11.2	Performance requirements for PUSCH

11.3	Performance requirements for PUCCH

11.4	Performance requirements for PRACH

Annex A (normative): Reference measurement channels

A.1	Fixed Reference Channels for RF Rx requirements (QPSK, R=1/3)

A.2	Fixed Reference Channels for dynamic range (16QAM, R=2/3)

A.3	[Fixed Reference Channels for performance requirements]

Annex B (normative): Error Vector Magnitude

B.1	Reference point for measurement

B.2	Basic unit of measurement

B.3	Modified signal under test

B.4	Estimation of frequency offset

B.5	Estimation of time offset

B.5.1	General

B.5.2	Window length

B.6	Estimation of TX chain amplitude and frequency response parameters

B.7	Averaged EVM

Annex C (informative): Change history