3GPP TSG RAN WG #4 Meeting #105                                                             	         R4-2220021

Toulouse, France, Nov 14th  - 19th, 2022

Title:	Draft reply LS on ue-PowerClassPerBandPerBC-r17

Response to:	LS R4-2218009/R2-221103

Release:	Rel-17

Work Item:	NR_RF_FR1_enh

Source:	TSG RAN WG4

To:	TSG RAN WG4

Cc:

Contact person:	Aijun CAO

Aijun.cao@mediatek.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN4 would like to respectfully thank RAN2 for the below questions on newly power class per band per band combinations:

1) Whether R4 16-8 is applicable to only inter-band CA?

2) What is the interaction between R4 16-8 and the existing power class capabilities (i.e. ue-PowerClass/ue-PowerClass-v1610/ue-PowerClass-1700, powerClassNRPart-r16 (if R4 16-8 is also applicable to the cases other than inter-band CA) and powerClass/powerClass-v1610)?

RAN4 discussed the above two questions in RAN4#105 and reached a consensus on the answers to these two questions:

Answer to Question 1):

Though the R4 16-8 is originally intended for inter-band CA, other possible usage e.g., a band combination consisting of a single band is not precluded, however, it is not applicable to EN-DC or NE-DC band combinations.

Answer to Question 2):

The interaction between the existing power class capabilities and the R4 16-8 can be shown as:

- The existing power class capabilities ue-PowerClass/ue-PowerClass-v1610/ue-PowerClass-1700 represent the maximum output power for a single NR band operation, and it does not necessarily have to be the same as R4 16-8.

- The existing powerClass/powerClass-v1610 represent the power class for a band combination, and the R4 16-8 should not be higher than the power class of the band combination.

- The existing powerClassNRPart-r16 represent the power class of the NR band in an EN-DC band combination, since the R4 16-8 does not apply to EN-DC/NE-DC, powerClassNRPart-r16 is independent from the R4 16-8.

# 2	Actions

To TSG RAN WG2

ACTION: 	RAN4 respectfully asks RAN2 to consider the above response from RAN4 in the signalling design for power class per band per band combination.

# 3	Dates of next TSG RAN WG #4 meetings

TSG-RAN WG4 Meeting #106		Feb 27th  - Mar 3rd , 2023         Athens, GR

TSG-RAN WG4 Meeting #106-bis-e	April 17th  - 26th , 2023             E-meeting