3GPP TSG-RAN WG4 Meeting #105-e	R4-2220161

# Toulouse, France, 14th – 18th Nov, 2022

Title:	WF on NTN SAN demodulation requirements

Source:	Huawei, HiSilicon

Agenda item:			9.11.8

Document for:	Approval

Introduction

This WF capture all agreements and open issues for the following topics discussed in [105][319] NR_NTN_Demod_Part2.

- Satellite Access Node demodulation requirements

  - PUSCH requirements

  - PUCCH requirements

  - PRACH requirements

PUCCH requirements

Antenna configuration for PUCCH requirements

- Define both 1Rx and 2Rx for SAN PUCCH format 0 and 2 requirements with NLOS channel with the same test applicability rule for different antenna configurations as for PUSCH.

SNR values derivation for performance requirements

- Capture the following the derived SNR values with square brackets in the CRs to be endorsed for this meeting which are derived by applying the following steps in order:

  - Step 1: The existing SNR derivation procedure defined in [R4-1904713] [R4-19004714] is used, and the following SNR results are derived

    - All PUSCH requirements:

    - PUCCH requirements

    - PRACH requirements

  - Step 2: If the results cannot be derived by using the existing SNR derivation procedure [R4-1904713] [R4-19004714] after Step 1, i.e. the test cases with error code -103 as shown above, then just change the ideal span from 2dB to 2.5dB in the existing SNR derivation procedure, then the following SNR results with green are derived for those cases with error code -103:

  - Step 3: If SNR values still can’t be derived, remove the furthest results and average the remaining results, i.e. two companies results are allowed, but still with 2dB span threshold, then the following SNR results with green are derived for those cases with error code -103 after Step 2:

- Companies can provide updated and more results for next meeting to further align the results. If results are well aligned, the existing SNR derivation procedure defined in [R4-1904713] [R4-19004714] should be used, otherwise further discussion on how to treat those cases with not well aligned results is not precluded.

Simulation assumption (informative)

PUSCH

NTN normal PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled, Enabled |
| Channel bandwidth |  | 15kHz SCS: 5MHz 30kHz SCS: 10MHz |
| MCS |  | 4 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 group hopping and sequence hopping are disabled for transform precoding enabled |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Number of TX antennas |  | 1 |
| Number of Rx antennas |  | 1, 2 |
| Propagation conditions and correlation matrix |  | NTN-TDLA100-200 Low, NTN-TDLC5-200 Low (K_factor = 8.05) |
| Test metric |  | SNR@70%maximum throughput |

NTN UL timing adjustment

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Channel bandwidth |  | 15kHz SCS: 5MHz 30kHz SCS: 10MHz |
| MCS |  | 4 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 12 RB for each UE |
|  | Starting PRB index | Moving UE: 0 Stationary UE: 12 |
|  | Frequency hopping | Disabled |
| SRS resource allocation | Slots in which sounding RS is transmitted (Note 1) | slot #1 in radio frames |
|  | SRS resource allocation | CSRS = 5, BSRS =0, for 20 RB |
| Code block group based PUSCH transmission |  | Disabled |
| Number of TX antennas |  | 1 |
| Number of Rx antennas |  | 1, 2 |
| Propagation conditions and correlation matrix |  | NTN-TDLA100-200 Low |
| Test metric |  | SNR@70%maximum throughput |
| NOTE 1. The transmission of SRS is optional. The transmission comb is configured as KTC = 2. The SRS periodic is configured as TSRS = 10 for 15kHz SCS and 20 for 30kHz SCS respectively. |  |  |

| Parameter | Scenario X |
| --- | --- |
| Channel model | Stationary UE: AWGN Moving UE: NTN-TDLA100-200 Low |
| CP length | Normal |
| A | 15 kHz: 10 s 30 kHz: 5 s |
|  | 0.04 S-1 for 15kHz SCS and 0.08 S-1 for 30kHz SCS |

NTN PUSCH repetition type A

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Channel bandwidth |  | 15kHz SCS: 5MHz 30kHz SCS: 10MHz |
| MCS |  | 5 in Table 3 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 3, 0, 3 [Note 1] |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | n2 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Number of TX antennas |  | 1 |
| Number of Rx antennas |  | 1, 2 |
| Propagation conditions and correlation matrix |  | NTN-TDLA100-200 Low |
| Test metric |  | SNR@70%maximum throughput |
| Note 1: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

PUCCH

NTN PUCCH format 0

| Parameter | Test |
| --- | --- |
| PUCCH format | 0 |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 12 |
| Number of OFDM symbols | 2 |
| Number of TX antennas | 1 |
| Number of RX antennas | [1], 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | ACK missed detection probability <1% |

NTN PUCCH format 1

| Parameter | Test |
| --- | --- |
| PUCCH format | 1 |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Cyclic Prefix | Normal |
| Number of TX antennas | 1 |
| Number of RX antennas | 1, 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

NTN PUCCH format 2 (ACK missed detection)

| Parameter | Value |
| --- | --- |
| PUCCH format | 2 |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Cyclic Prefix | Normal |
| Number of TX antennas | 1 |
| Number of RX antennas | [1], 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

NTN PUCCH format 2 (UCI BLER)

| Parameter | Value |
| --- | --- |
| PUCCH format | 2 |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Cyclic Prefix | Normal |
| Number of TX antennas | 1 |
| Number of RX antennas | [1], 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | UCI block error probability<1% |

NTN PUCCH format 3

| Parameter | Value |
| --- | --- |
| PUCCH format | 3 |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| The number of UCI information bits | 16 |
| First symbol | 0 |
| Cyclic Prefix | Normal |
| Additional DM-RS configuration | No additional DM-RS, Additional DM-RS |
| Number of TX antennas | 1 |
| Number of RX antennas | 1, 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | UCI block error probability<1% |

NTN PUCCH format 4

| Parameter | Value |
| --- | --- |
| PUCCH format | 4 |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Cyclic Prefix | Normal |
| Additional DM-RS configuration | No additional DMRS, Additional DM-RS |
| Number of TX antennas | 1 |
| Number of RX antennas | 1, 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | UCI block error probability<1% |

NTN PUCCH format 1 (multi-slot)

| Parameter | Test |
| --- | --- |
| PUCCH format | 1 (multi-slot) |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |
| Cyclic Prefix | Normal |
| Number of TX antennas | 1 |
| Number of RX antennas | 1, 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

PRACH

| Parameter | Value |
| --- | --- |
| Preamble format | 0/B4/C2/2 |
| SCS | 1.25kHz for long preamble format 15kHz and 30kHz SCS for short preamble format (LRA=139) |
| Number of TX antennas | 1 |
| Number of RX antennas | 1, 2 |
| Propagation conditions and correlation matrix | AWGN, NTN-TDLA100-200 Low |
| Time error tolerance | 0.522μ+Tdelay where Tdelay is the largest delay of the propagation channel |
| Frequency offset | 0Hz for AWGN, 200Hz for fading channel |
| Test metric | false alarm probability<0.1%, detection probability>99% |

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
| 0 | 1.25 | 13 | 22 | 32 |
| 2 | 1.25 | 13 | 22 | 32 |
| B4,C2 | 15 | 23 | 0 | 0 |
|  | 30 | 46 | 0 | 0 |

Reference

- R4-2210662, WF on NTN SAN demodulation requirements, RAN4#103-e, Huawei, HiSilicon

- R4-2210329, Email discussion summary for [103-e][323] NR_NTN_Demod_Part2, RAN4#103-e, Moderator (Huawei)

- R4-2207198, WF on NTN SAN demodulation requirements, RAN4#102-e, Huawei, HiSilicon

- R4-2207464, WF on general and NTN UE demodulation requirements, RAN4#102-e, Qualcomm

- R4-2207169, Email discussion summary for [102-e][325] NR_NTN_Demod, RAN4#102-e, Moderator (Qualcomm Incorporated)

- R4-2203043, WF on NTN SAN demodulation requirements, RAN4#101bis-e, Huawei, HiSilicon