3GPP TSG-RAN WG4 Meeting #105	R4-2220232

Toulouse, France, November 14 - 18, 2022

Title: 	TP to TS 38.181: Annex E

Agenda Item:	6.2.2.1

Source: 	Ericsson

Document for:	Approval

## 1	Introduction

This contribution proposes text to be included in Annex E of TS 38.181

It is a revision of R4-2216493 and following changes were made:

- In-channel selectivity, and Adjacent channel selectivity included.

- Removed narrow-band blocking, OTA receiver intermodulation since they are not applicable for NTN.

- after revision of 2219836:

- - corrected table numbering

- - added conducted rx requirements

## 2	Text proposal

Annex E (normative):
Characteristics of the interfering signals

The interfering signal shall be a PUSCH containing data and DMRS symbols. Normal cyclic prefix is used. The data content shall be uncorrelated to the wanted signal and modulated according to clause 6 of TS 38.211 [8]. Mapping of PUSCH modulation to receiver requirement are specified in table E-1.

Table E-1: Modulation of the interfering signal

| Receiver requirement | Modulation |
| --- | --- |
| In-channel selectivity OTA in-channel selectivity | 16QAM |
| Adjacent channel selectivity OTA adjacent channel selectivity | QPSK |
| General blocking General OTA blocking | QPSK |