3GPP TSG-RAN WG4 Meeting #105	R4-2220255

Toulouse, France, 14 ‒ 18 Nov, 2022

Title: 	         WF for NCR system parameters and RF requirements

Agenda Item:	     8.25.4

Source: 	ZTE, CMCC

Document for:	Approval

Agreement

Issue 1-1:  Beamforming capability for FR1 NCR

- Agreement:

  - Beamforming capability can be supported for FR1 NCR from RAN4 perspective

Issue 1-2:  System parameter for NCR-MT

  - Proposal 1 agreed

Issue 1-3:  NCR-MT feature list

- Agreement: Postpone the discussion on NCR-MT feature list into future RAN4 meetings.

Issue 2-2-1  NCR-MT UL assumption

- Agreement:

  - NCR-MT uplink transmission is supported

Issue 2-2-2  RF requirements for NCR-MT transmitter requirements

- Agreement: For NCR-MT transmitter:

  - Baseline assumption: Legacy uplink transmitter requirements from Rel-17 RF repeater still be applicable

    - Further discuss the power control requirements for NCR-MT

    - Update on other requirements not precluded

  - For NCR Tx direction requirements, reuse the concept of OTA coverage range which also include OTA peak directions sets.

Issue 2-3  BC for NCR

- Agreement:

  - Taking IAB-MT approach as starting point, and further discuss how to address different BC capabilities.

Issue 2-4  Rx requirements for NCR-MT

- Agreement: Pro

  - Proposal 1: For NCR-MT Rx direction requirements, it’s suggested to specify following OTA Rx direction declarations. These declarations apply for both input signal and interference signal.

- A declared receiver target redirection range. This range is not needed if NCR don’t support redirection.

- [five] declared sensitivity RoAoA comprising the conformance testing directions. If don’t support redirection, only one RoAoA is suggested

- Receiver target reference direction

- Min EIS may only need for MT part

  - Proposal 3: for the RF requirement of NCR-MT receiver, to define the refense requirement for NCR-MT receiver.