3GPP TSG-RAN4 Meeting #105	R4-2220290

Toulouse, FR, November 14 – November 18, 2022

Title: 	TP for TS 38.181: Clause 4.1.2 Acceptable uncertainty of Test System

Source: 	CATT

Agenda item:	6.2.2.1

Document for:	Approval

- Introduction

This contribution provides a text proposal for TS 38.181: Clause 4.1.2 Acceptable uncertainty of Test System [1].

- Reference

[1] TS 38.181, 0.2.0

- Text proposal

---------------------------------------------------Start of Text proposal---------------------------------------------------------

4.1.2 Acceptable uncertainty of Test System

4.1.2.1	General

The maximum acceptable uncertainty of the Conducted Test System and OTA Test System are specified below for each test defined explicitly in the present specification, where appropriate. The maximum acceptable uncertainty of the Test System for test requirements included by reference is defined in the respective referred test specification.

The Test System shall enable the stimulus signals in the test case to be adjusted to within the specified tolerance and the equipment under test to be measured with an uncertainty not exceeding the specified values. All tolerances and uncertainties are absolute values, and are valid for a confidence level of 95 %, unless otherwise stated.

A confidence level of 95 % is the measurement uncertainty tolerance interval for a specific measurement that contains 95 % of the performance of a population of test equipment.

For conducted RF tests, it should be noted that the uncertainties in clause 4.1.2 apply to the Test System operating into a nominal 50 ohm load and do not include system effects due to mismatch between the DUT and the Test System.

For details on measurement uncertainty budget calculation, measurement methodology description (including calibration and measurement stage for each test range), MU budget format and its contributions, refer to TR 37.941 [13].

4.1.2.2	Measurement of transmitter

The maximum conducted Test System uncertainty for conducted transmitter tests minimum requirements is given in table 4.1.2.2-1. And the maximum OTA Test System uncertainty for OTA transmitter tests minimum requirements is given in table 4.1.2.2-2.

Table 4.1.2.2-1: Maximum Test System uncertainty for conducted transmitter tests

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2 Base Station output power | ±0.7 dB, f ≤ 3 GHz |  |
| 6.3 Output power dynamics | ± 0.4 dB |  |
| 6.5.2 Frequency error | ± 12 Hz |  |
| 6.5.3 EVM | ± 1% |  |
| 6.6.2 Occupied bandwidth | 5 MHz, 10 MHz SAN Channel BW: ±100 kHz 15 MHz, 20 MHz SAN Channel BW: ±300 kHz |  |
| 6.6.3 Adjacent Channel Leakage power Ratio (ACLR) | ACLR/ CACLR BW ≤ 20MHz: ±0.8 dB Absolute power ±2.0 dB, f ≤ 3 GHz CACLR BW ≤ 20MHz: ±0.8 dB CACLR absolute power ±2.0 dB , f ≤ 3 GHz |  |
| 6.6.4 Operating band unwanted emissions | ±1.5 dB, f ≤ 3 GHz |  |
| 6.6.5.5.1.1 Transmitter spurious emissions, Mandatory Requirements | 9 kHz < f ≤ 4 GHz: ±2.0 dB 4 GHz < f ≤ 15 GHz: ±4.0 dB |  |
| 6.6.5.5.1.2 Transmitter spurious emissions, Protection of SAN receiver | ±3.0 dB |  |

Table 4.1.2.2-2: Maximum OTA Test System uncertainty for FR1 OTA transmitter tests

| Clause | Maximum OTA Test System uncertainty |
| --- | --- |
| 9.2 Radiated transmit power | Normal condition: [±1.1] dB, f ≤ 3 GHz |
| 9.3 OTA base station output power | [±1.4] dB, f ≤ 3.0 GHz |
| 9.4.3 OTA total power dynamic range | [±0.4] dB |
| 9.6.2 OTA frequency error | [±12] Hz |
| 9.6.3 OTA modulation quality | [±1 %] |
| 9.7.2 OTA occupied bandwidth | [±100] kHz, BWChannel 5 MHz, 10 MHz [±300] kHz, BWChannel 15 MHz, 20 MHz |
| 9.7.3 OTA ACLR/CACLR | f ≤ 3.0 GHz [±1] dB, BW ≤ 20MHz Absolute power [±2.2] dB, f ≤ 3.0 GHz |
| 9.7.4 OTA operating band unwanted emissions | Absolute power [±1.8] dB, f ≤ 3.0 GHz |
| 9.7.5.2 OTA transmitter spurious emissions, mandatory requirements | [±2.3] dB, 30 MHz < f ≤ 6 GHz [±4.2] dB, 6 GHz < f ≤ 15 GHz |
| 9.7.5.3 OTA transmitter spurious emissions, protection of SAN receiver[(Note 1)] | [±3.1] dB, f ≤ 3 GHz |
| [NOTE 1: Fulfilling the criteria for CLTA selection and placement in clause 4.10 is deemed sufficient for the test purposes. When these criteria are met, the measurement uncertainty related to the selection of the co-location test antenna and its alignment as specified in the appropriate measurement uncertainty budget in TR 37.941 [13] shall be used for evaluating the test system uncertainty. ] NOTE 2: Test system uncertainty values are applicable for normal condition unless otherwise stated. |  |

4.1.2.3	Measurement of receiver

The maximum conducted Test System uncertainty for conducted receiver tests minimum requirements are given in table 4.1.2.3-1, and the maximum OTA Test System uncertainty for OTA receiver tests minimum requirements are given in table 4.1.2.3-2.

Table 4.1.2.3-1: Maximum Test System Uncertainty for conducted receiver tests

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 7.2 Reference sensitivity level | ±0.7 dB, f ≤ 3 GHz |  |
| 7.3 Dynamic range | ±0.3 dB |  |
| 7.4.1 Adjacent channel selectivity | ±1.4 dB, f ≤ 3 GHz | Overall system uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Additional impact of interferer leakage Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The interferer leakage effect is systematic, and is added arithmetically. Test System uncertainty = SQRT (wanted_level_error2 + interferer_level_error2) + leakage effect. f ≤ 3 GHz Wanted signal level ±0.7 dB Interferer signal level ±0.7 dB f ≤ 6 GHz Impact of interferer leakage 0.4 dB |
| 7.5.5.1 Out-of-band blocking (General requirements) | fwanted ≤ 3GHz 1MHz < finterferer ≤ 3 GHz: ±1.3 dB 3.0GHz < finterferer ≤ 4.2 GHz: ±1.5 dB 4.2GHz < finterferer ≤ 12.75 GHz: ±3.2 dB | Overall system uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = SQRT (wanted_level_error2 + interferer_level_error2) + Broadband noise effect. Out of band blocking, using CW interferer: Wanted signal level: ±0.7 dB up to 3 GHz Interferer signal level: ±1.0 dB up to 3 GHz Impact of interferer Broadband noise 0.1 dB |
| 7.8 In-channel selectivity | ±1.4 dB, f ≤ 3 GHz |  |
| NOTE 1: Unless otherwise noted, only the Test System stimulus error is considered here. The effect of errors in the throughput measurements due to finite test duration is not considered. |  |  |

Table 4.1.2.3-2: Maximum OTA Test System uncertainty for FR1 OTA receiver tests

| Clause | Maximum OTA Test System uncertainty |
| --- | --- |
| 10.2 OTA sensitivity | [±1.3] dB, f ≤ 3.0 GHz |
| 10.3 OTA reference sensitivity level | [±1.3] dB, f ≤ 3.0 GHz |
| 10.4 OTA dynamic range | [±0.3] dB |
| 10.5.1 OTA adjacent channel selectivity | [±1.7] dB, f ≤ 3.0 GHz |
| 10.6 OTA out-of-band blocking (General) | fwanted ≤ 3.0 GHz: [±2.0] dB, finterferer ≤ 3.0 GHz [±2.1] dB, 3.0 GHz < finterferer ≤ 6.0 GHz [±3.5] dB, 6.0 GHz < finterferer ≤ 12.75 GHz |
| 10.9 OTA in-channel selectivity | [±1.7] dB, f ≤ 3.0 GHz |
| NOTE 1: Test system uncertainty values are applicable for normal condition unless otherwise stated. |  |

---------------------------------------------------End of Text proposal---------------------------------------------------------