3GPP TSG-RAN WG4 Meeting # 105      	R4-2220361

Toulouse, France, November 14 – November 18, 2022

Title: 	WF on NR ATG RRM requirements

Agenda Item:	8.13.5

Source: 	CMCC

Document for:	Approval

- General

Issue 1-1-1: ISD assumption

Agreements:

- Investigate the RRM impactions of ISD [14]-200km.

  - Further capture the conclusion on square bracket which will come from RF session.

Issue 1-1-2: Maximum distance between ATG BS and Aircraft UE

Agreements

- Maximum distance between ATG BS and Aircraft UE for further RRM analysis is ≥ [200] km.

  - Upper bound is FFS.

Issue 1-1-3: Duplex mode and SCS

- For the test cases, further specify the combination of duplex mode and SCS at perf stage.

- For the core requirements, discuss following Options:

  - Option 1: For duplex mode/SCS configurations, no impacts on core requirements (HW, CATT)

  - Option 2: Consider all the combinations between FDD/TDD and 15kHz/30kHz (CMCC, ZTE)

  - Option 3: Determine either of the following options,

    - Option 3-1: Only consider 15kHz SCS for FDD 2GHz and 30kHz for TDD for requirement development. (Apple)

    - Option 3-2: Consider all the SCS’s of 15kHz, 30kHz and 60kHz for FDD and TDD (Apple)

  - Option 4: Consider 30kHz SCS as baseline assumption to analyses the impact of propagation delay difference in TDD (LGE)

  - Option 5: SCS type to be supported for ATG is discussed as part of the timing requirements. (Ericsson)

Agreements:

- For the test cases, further specify the combination of duplex mode and SCS at perf stage.

- For the core requirements, consider all the SCS’s of 15kHz, 30kHz [and 60kHz] for FDD and TDD

Issue 1-1-4: TDD pattern

Agreements:

- The TDD pattern is up to NW configuration. No impaction on core requirements is observed, and it can be further discussed in the stage of performance.

Issue 1-1-5: whether new UAI reporting method could be introduced

- Option 1: ATG WI should avoid to introduce new UAI reporting method. (CATT, CMCC, ZTE (no obvious motivation and feasibility guarantee to introduce the above UAI report))

- Option 2: There is no need to preclude new UAI and its reporting methods at this stage. Postpone the decision on assistance information (both UE-based and network-based) until the RRM requirement framework is clear. (LGE, Apple, Ericsson)

  - Option 2a: Assistance information related to unpredictable changes in flight operation mode are introduced: 1) UE detecting and informing the NW, 2) NW informing the UE (Ericsson)

- Option 3: Whether to introduce new UE assistant information will require RAN1/RAN2 work, which shall not be decided in WG. (HW)

Issue 1-1-6: Band table

Way forward:

- Postpone the discussion, wait the conclusion from RF session.

- Mobility

  - Mobility in RRC_IDLE/INACTIVE

Issue 2-1-1: Cell re-selection

Issue 2-1-1-1: Cell re-selection measurement capability

Agreements:

- The Rel-15 NR requirements on intra-frequency and inter-frequency measurement capability requirements are reused for ATG.

Issue 2-1-1-2: Cell re-selection mechanism

Agreements:

- Use legacy cell-reselection rules and the procedures as baseline for ATG cell re-selection mechanism.

  - FFS whether additional enhanced cell re-selection mechanisms shall be considered (e.g., location, distance, speed, based).

  - FFS: The UE is allowed to not measure on the neighbour cell measurements based on the distance. Details are FFS.

Issue 2-1-1-3: UE/network assistance information for Cell re-selection

- Option 1: Assistance information comprises at least changes in the UE altitude, flight path and speed, remaining service time for serving cell and side conditions can be FFS. (Ericsson)

- Option 2: FFS

Issue 2-1-2: Cell re-selection measurement requirements

- Option 1: Reuse the legacy R15 requirements on intra-frequency and inter-frequency measurement. (CATT, HW)

- Option 2: With the basic ISD assumption of [14]-200km, both legacy R15 cell-reselection requirement and HST cell-reselection requirement should be applied for ATG. (CMCC)

  - Option 2-1: The UE capability should be introduced for ATG similar as NTN. (CMCC)

- Option 3: Postpone the discussion on cell-reselection requirement until more progress is reached on cell re-selection mechanism (Apple, ZTE)

Issue 2-1-3: SDT

- Option 1: RAN4 is not going to define ATG specific requirements.

- Option 2: SDT is supported for A2G in Rel-18, and SDT requirements for ATG is FFS.

  - Mobility in RRC_CONNECTED

Issue 2-2-1: Handover

Issue 2-2-1-1: NR Handover mechanism

- Option 1: Legacy NR Handover mechanism can be reused by ATG. (CATT, CMCC, Apple (as the baseline), ZTE)

- Option 2: further discuss enhanced solution for ATG hand over mechanism (Apple)

Agreements:

- Legacy NR Handover mechanism can be reused by ATG

Issue 2-2-1-2: NR Handover requirement

- For the unknown case

  - Option 1: reusing legacy requirements (CATT (tolerable), CMCC, HW)

  - Option 2: Not to define handover requirements for an unknown cell. (CATT (preferred), ZTE)

  - Option 3: Postpone the discussion on hand over requirement until the hand over mechanism is concluded (Apple)

  - Option 4: For handover to unknown target cell, Tsearch is FFS (Ericsson)

Issue 2-2-2: Conditional Handover

- Option 1: Not to consider conditional handover for ATG UE at least in Rel-18 (CATT (preferred))

- Option 2: Introduce legacy CHO for ATG (CATT (tolerable), CMCC)

  - Option 2-1: The legacy R16 CHO delay requirements can be reused. (CATT (tolerable), CMCC)

- Option 3: Introduce location-based CHO for ATG (CMCC, Apple, LGE)

  - Option 3-1: The logic and signalling from R17 NTN could be reused, while the location-based CHO delay requirements should be revisited in ATG scenario. (CMCC)

  - Option 3-2: reuse legacy CHO requirement framework (Apple)

- Option 4: CHO is supported for A2G in Rel-18, and CHO requirements for ATG is FFS. (Ericsson)

Issue 2-2-3: SA: RRC Re-establishment

Way forward:

- Reuse the legacy requirement structure and definitions. Further discuss Tidentify_intra_NR and Tidentify_inter_NR in ATG condition.

Issue 2-2-3x: Large cell impact on RRC re-establishment

- FFS the large cell impact on RRC re-establishment requirements.

Issue 2-2-4: 2-step RA

Agreement:

- No need to have ATG specific requirements. (CATT, Apple)

Issue 2-2-5: SA: RRC Connection Release with Redirection

Way forward:

- Reuse the legacy requirement structure and definitions. Further discuss Tidentify-NR.

Issue 2-2-5x: Large cell impact on RRC Connection Release with Redirection

- FFS large cell impact on RRC connection release with redirection requirements.

- Timing and frequency adjustment

  - General issues

Issue 3-1-1: The mechanism of Koffset

- Option 1: No need to consider Koffset in ATG network.

- Option 2: Introduce the mechanism of Koffset in ATG system.

  - Option 2-1: For Koffset, considering the RTT for cell edge UE, for the case of 30kHz, maybe existing scheduling time line of n+k is not sufficient. While for the case of 15kHz, since the maximum RTT would not beyond one slot, the necessity of Koffset is not obvious.

Issue 3-1-2: BS location

- Option 1: Informing UE the BS location may not be needed for RRM requirements. (CATT)

- Option 2: PositionVelocity-r17 in TS 38.331 can be used to signal the BS location to ATG UE. (CMCC, HW, ZTE (Then UE can perform frequency pre-compensation based on the path between BS and itself.))

- Option 3: RAN4 can study following options to inform BS location. (LGE)

  - Alt 1. ReferenceLocation-r17.

  - Alt 2. PositionVelocity-r17.

  - Alt 3. ReferenceLocation-r17 or PositionVelocity-r17. It is up to NW implementation.

Agreement:

- PositionVelocity-r17 in TS 38.331 can be used to signal the BS location to ATG UE.

  - Timing and frequency pre-compensation by UE

Issue 3-2-1: Whether to introduce UE based Timing pre-compensation

Agreements

- Support UE-based timing pre-compensation for ATG networks

  - FFS for details and whether NTN-based solution can be reused

  - Timing requirements

Issue 3-3-1: UE transmit timing

Issue 3-3-1-1: Initial transmit timing requirements Te

- Option 1: The legacy R15 TN requirement for initial transmit timing requirement Te can be reused.

- Option 2: The Te should be revised, FFS the value.

Issue 3-3-1-2: Gradual timing adjustment

- Option 1: For ATG, the Tq_ATG and Tp_ATG should be 9.5*64*Tc in FR1 and UL SCS 15kHz and 30kHz SCS. (CMCC, Ericsson)

- Option 2: The maximum aggregate adjustment rate is changed to Tq per 100ms for ATG UE. (CATT)

- Option 3: Tp and Tq shall be updated for ATG UE. (Apple, ZTE)

Issue 3-3-2: Timing advance

Agreements:

- For timing Advance adjustment accuracy requirement, legacy value can be reused.

Way forward:

- Further discuss timing advance adjustment delay requirement.

Issue 3-3-4: deriveSSB-IndexFromCell and deriveSSB-IndexFromCell-inter tolerance

- Option 1: Legacy TN requirement can be reused. It is up to network to decide whether to enable ‘deriveSSB-IndexFromCell’ and ‘deriveSSB-IndexFromCell-inter’ or not (CATT, CMCC, Apple, HW, ZTE)

- Option 2: RAN4 to study to utilize the deriveSSB-IndexFromCell based on the propagation delay difference between serving and neighbor cell. (LGE)

Agreement:

- Legacy TN requirement can be reused. Whether ‘deriveSSB-IndexFromCell’ and/or ‘deriveSSB-IndexFromCell-inter’ are applicable to ATG scenarios is FFS

- Signalling characteristics

Issue 4-1-1: Radio Link Monitoring

- Option 1: Legacy requirements for RLM can be reused for ATG UE. (CATT, HW, ZTE)

- Option 2: Whether legacy RLM evaluation period requirements can be reused or new ATG specific evaluations are needed depends on channel model to be agreed for ATG in RF group. (Ericsson)

Agreement:

- Corresponding release 15 NR requirements for RLM can be reused for ATG UE.

Issue 4-1-2: Link Recovery Procedure

- Option 1: Legacy requirements for link recovery procedure can be reused for ATG UE. (CATT, HW, ZTE)

- Option 2: Whether legacy BFD and CBD requirements can be reused or new ATG specific evaluations are needed depends on channel model to be agreed for ATG in RF group. (Ericsson)

Agreement:

- Corresponding release 15 NR requirements for link recovery procedure can be reused for ATG UE.

Issue 4-1-3: Active BWP switching delay

Agreement:

- Reuse the legacy active BWP switching delay requirement of single carrier for Rel-18 ATG.

Issue 4-1-4: L1-RSRP measurement requirements

- Option 1: Reuse the legacy L1-RSRP measurement requirements for ATG. (CATT)

- Option 2: Whether the legacy L1-RSRP measurement requirements can be reused for ATG or new ATG specific evaluations are needed depends on the channel to be agreed for ATG in RF group. (Ericsson)

Agreement:

- Reuse the legacy L1-RSRP measurement requirements for ATG.

Issue 4-1-5: Active TCI state switching delay, Active downlink TCI state switching delay for unified TCI, Active uplink TCI state switching delay for unified TCI

Agreement:

- Reuse the legacy Active downlink TCI state switching delay for unified TCI and Active uplink TCI state switching delay for unified TCI requirement for ATG.

Issue 4-1-6: Pathloss reference signal switching delay

Agreement:

- Reusing the legacy Pathloss reference signal switching delay requirement for ATG

Issue 4-1-7: Uplink spatial relation switch delay

Agreement:

- Uplink spatial relation switch delay requirement will not be applicable for R18 ATG.

- Measurement

Issue 5-1-1: Measurement gap

- Option 1: Reuse the legacy MG requirements for R18 ATG. (CATT, CMCC, LGE, HW, ZTE)

- Option 2: It is proposed to consider extend the measurement gap length to include 6.5us/7us/7.5us for ATG. (Apple)

- Option 3: Only FR1 MG will be used in Rel-18 ATG network. (Ericsson)

Issue 5-1-2: Measurement capability

Agreement:

- The legacy measurement capability of monitoring of multiple layers using gap can be reused for Rel-18 ATG.

Issue 5-1-3: CSSF

Agreement:

- Take 2 searchers as the assumption when defining the CSSF requirement

Way forward

- : For inter-frequency without gap assumption

  - Option 1: inter-frequency measurement without GAP should be considered. (CATT, CMCC)

  - Option 2: inter-frequency without gap maybe not a typical scenarios to be considered. (HW)

- : For CSSF value

  - Option 1: For intra-frequency measurement outside measurement gap, CSSFoutside_gap,i =1. (CMCC, HW, Ericsson)

  - Option 2: For inter-frequency measurement outside measurement gap, CSSFoutside_gap,i =Y, Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG. (CMCC, Ericsson)

  - Option 3: If intra-frequency measurement is with measurement gap, CSSFoutside_gap,i = Y for inter-frequency measurement with no measurement gap, Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG (Ericsson)

  - Option 4: the CSSF under RedCap single carrier case can be a reference. (ZTE)

Issue 5-1-4: Intra-frequency measurements requirement

- Option 1: The legacy NR intra-frequency measurements requirement can be reused (CATT, CMCC, LGE, ZTE)

- Option 2: intra-frequency measurement with gap will not be considered for R18 ATG (CATT)

- Option 3: postpone the discussion on measurement requirement until we conclude measurement GAP, measurement capability an CSSF (Apple)

- Option 4: RAN4 not to consider deactivated SCell measurement, SCCs measurement, and PSCell measurement. (Ericsson)

Issue 5-1-5: Inter-frequency measurements requirement

- Option 1: The legacy NR inter-frequency measurements requirement can be reused for R18 ATG. (CATT, CMCC, LGE, HW, ZTE)

- Option 2: postpone the discussion on measurement requirement until we conclude measurement GAP, measurement capability an CSSF (Apple)

- Option 3: RAN4 to study the trade-off between Inter-frequency measurement with gap and the data throughput due to large cell coverage. (Ericsson)

Issue 5-1-6: Scheduling restriction

- Option 1: The scheduling restriction should be defined based on propagation delay between serving gNB and ATG UE. (LGE)

- Option 2: FFS

Issue 5-1-7: CSI-RS based L3 measurements

- Option 1: The legacy CSI-RS based L3 measurements requirement can be reused for R18 ATG (CATT)

- Option 2: CSI-RS measurement is unnecessary in ATG network (Ericsson)

Issue 5-1-8: Pre-configured measurement gap

- Option 1: No need to introduce the pre-configured measurement gap for R18 ATG. (CATT, ZTE)

- Option 2: Postpone pre-configured measurement gap to future release (Apple)

- Option 3: Pre-MG can be used in ATG network to improve the total system performance. (Ericsson)