3GPP TSG-RAN WG4 Meeting # 105															R4-2220363

Toulouse, France, November 14 – November 18, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | draft | rev |  | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of cell re-selection and PUR requirement for UE category NB-IoT for Satellite Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-Core |  |  |  |  |  | Date: |  |  | 2022-11-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce IDLE requirement for UE category NB-IoT for Satellite Access |  |  |  |  |  |  |  |  |
| Summary of change: |  | IDLE requirement for UE category NB-IoT for Satellite Access |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing IDLE requirement for UE category NB-IoT for Satellite Access |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 4.6A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change #1>

## 4.6A	Cell Selection and Re-selection Requirements for UE category NB1 for Satellite Access

The NB-IoT applicability of the requirements in section 4.6A is defined in Section 3.6.1. The requirements in this subclause apply if UE category NB1 is in normal and enhanced coverage area of the serving cell served by a satellite access node.

### 4.6A.1	Cell Selection

After a UE has switched on and a PLMN has been selected, the Cell selection process takes place, as described in TS36.304. This process allows the UE to select a suitable cell where to camp on in order to access available services. In this process the UE can use stored information (Stored information cell selection) or not (Initial cell selection).

### 4.6A.2	Cell Re-selection for UE category NB-IoT for Satellite Access

The cell reselection procedure allows the UE to select a more suitable cell and camp on it.

When the UE is in either Camped Normally state or Camped on Any Cell state on a cell, the UE shall attempt to detect, synchronise, and monitor intra-frequency and inter-frequency cells indicated by the serving NB-IoT cell. For intra-frequency and inter-frequency cells the serving NB-IoT cell may not provide explicit neighbour list but carrier frequency information and bandwidth information only. UE measurement activity is also controlled by measurement rules defined in TS36.304, allowing the UE to limit its measurement activity.

If SystemInformationBlockType32 [2] has been received and if the UE has determined that it is out of coverage, the UE is not required to perform cell measurements from the last slot of SI transmission which indicates that UE will be in out of coverage after t-Service [2] when the serving cell stop serving the area.

Editor’s note: For NGSO, FFS whether to clarify the UE shall meet the IDLE mode requirements after time period of X + ‘t-ServiceStart-r17’, where X is time until the UE acquires the system information carrying Ephemeris.

Editor’s note: For NGSO, FFS whether to revise the relaxed requirements upon, e.g. t-Service.

#### 4.6A.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage

The UE shall measure the NRSRP and NRSRQ level of the serving NB-IoT cell on the anchor carrier and evaluate the cell selection criterion S defined in clause 5.2.3.2 in [1] for the serving NB-IoT cell on the anchor carrier at least every DRX cycle.

The UE shall filter the NRSRP and NRSRQ measurements of the NB-IoT serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least DRX cycle/2.

If the UE is configured for receiving paging on the non-anchor carrier then the UE shall evaluate the cell selection criterion S defined in clause 5.2.3.2a in [1] for the serving NB-IoT cell on non-anchor carrier at least every DRX cycle.

The UE is allowed to measure NRSRP level of the serving NB-IoT cell, assuming that nrs-NonAnchor-config is enabled indicated by higher layer defined in clause 10.2.6 TS 36.211 [16], on non-anchor carrier provided that:

-	The relaxed monitoring criteria defined in TS 36.304 clause 5.2.4.12 are met,

-	Transmit power difference of the signals/channels between anchor- and non-anchor carriers is signalled to the UE, via the existing parameter nrs-PowerOffsetNonAnchor, and

-	UE is not configured with any positioning measurements.

When UE measures the NRSRP on non-anchor carrier, UE shall compare the measurements from anchor carrier and non-anchor carrier at least once every one hour by the following inequality:

| NRSRPanchor – (NRSRPnon-anchor -  nrs-PowerOffsetNonAnchor) | ≤ 10 dB

where NRSRPanchor is the NRSRP measurement on anchor carrier and NRSRPnon-anchor is the NRSRP measurement on non-anchor carrier. The measurement for comparison shall use at least 2 measurements for filtering. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least DRX cycle/2. If the measurement for comparison satisfy the inequality, UE is allowed to perform RRM measurements on the non-anchor carrier until the next comparison takes place or until the relaxed monitoring conditions are no longer met. UE shall perform NRSRP measurement on anchor carrier if the inequality is not satisfied until the next comparison takes place.

When all the conditions for measuring NRSRP on non-anchor carrier are satisfied, the UE shall filter the NRSRP of the serving NB-IoT cell using at least 2 measurements spaced by at least DRX cycle/2, where the measurements used for the filtering may include measurements on anchor carrier and on non-anchor carrier.

If the UE is not configured with eDRX_IDLE cycle and has evaluated according to Table 4.6A.2.1-1 in Nserv_NB -NC consecutive DRX cycles that the serving NB-IoT cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving NB-IoT cell, regardless of the measurement rules currently limiting UE measurement activities. If the UE is configured with eDRX_IDLE cycle and has evaluated according to Table 4.6A.2.1-2 in Nserv_NB-NC consecutive DRX cycles within a single PTW that the serving NB-IoT cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving NB-IoT cell, regardless of the measurement rules currently limiting UE measurement activities. Additionally, if the UE is configured with t-Service [2], the UE should start measurements of the neighbour cells indicated by the serving cell before t-Service is reached according to the requirements provided in clause [4.6A.2.2] and [4.6A.2.5].

If t-Service is not provided nor applicable, and if the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency and inter-frequency information indicated in the system information during the time T, the UE shall initiate cell selection procedures for the selected PLMN as defined in [1], where T=40 s if the UE is not configured with eDRX_IDLE cycle, and T=MAX(40 s, one eDRX_IDLE cycle) if the UE is configured with eDRX_IDLE cycle.

If t-Service is provided and applicable of the serving cell then the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 36.304 when any of the following conditions is fulfilled:

-	If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information within 40 s since time instance T1 provided that t-Service > T1 or

-	If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information within 40 s since the time instance t-Service.

-	Where, T1 is the time instance in seconds when the UE has determined that the serving cell does not fulfil the cell selection criterion S.

When the UE is provided with t-serviceStart-r17 and has discontinuous coverage capabilities, then after t-service-r17 is reached and the UE is out of coverage, the UE may delay or resume cell measurements/search till when the UE is in coverage.

Editor's Note: Definition of in coverage is FFS

Table 4.6A.2.1-1: Nserv_NB-NC

| DRX cycle length [s] | Nserv_NB-IoT-NC [number of DRX cycles] |
| --- | --- |
| 0.32 | 2 |
| 0.64 | 2 |
| 1.28 | 2 |
| 2.56 | 2 |
| 5.12 | 2 |
| 10.24 | 2 |

Table 4.6A.2.1-2: Nserv_NB-NC for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 2.56s periods) | Nserv_NB-IoT-NC [number of DRX cycles] |
| --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | ≥2.56 (1) | 2 |
|  | 0.64 | ≥2.56 (1) | 2 |
|  | 1.28 | ≥5.12 (2) | 2 |
|  | 2.56 | ≥7.68 (3) | 2 |
|  | 5.12 | ≥12.8 (5) | 2 |
|  | 10.24 | ≥23.04 (9) | 2 |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section X of TS 24.008 [34]. |  |  |  |

For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE has to meet the requirement corresponding to the second state.

#### 4.6A.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS

The UE which supports wakeUpSignal [2] shall meet the requirement defined for the DRX cycle length of N*DRX_cycle in Section 4.6A.2.1, provided the following conditions are met:

-	WUS has been configured in the serving NB-IoT cell using WUS-Config-NB-r15 [2], and

-	The serving cell measurement relaxation is signalled as n by the network using numDRX-CycleRelaxed-r15, and

-	Serving cell S criteria is met with at least 2 dB margin.

-	the relaxed monitoring criteria for neighbour cells in TS 36.304 [1] clause 5.2.4.12.1 is fulfilled, and

, where the relaxation factor N is given by Table 4.6A.2.1A-1. Otherwise the requirements defined for the configured DRX cycle length in Section 4.6A.2.1 shall apply.

The UE shall further meet the requirements in section 4.6A.2.1 during time period T0 after following occasions:

-	after the end of reception of latest paging message, or

-	from the moment UE has switched from RRC_CONNECTED state to RRC_IDLE state.

T0 = N*DRX cycle if the UE is not configured with eDRX_IDLE cycle where the value of N specified in Table 4.6A.2.1A-1;

T0 = one eDRX IDLE cycle if the UE is configured with eDRX_IDLE cycle;

Table 4.6A.2.1A-1: The relaxation factor N for a UE not configured with eDRX IDLE cycle

| DRX cycle length [s] | Value |
| --- | --- |
| 0.32 | Min(n , 8) |
| 0.64 | Min(n , 8) |
| 1.28 | Min(n , 8) |
| 2.56 | Min(n , 4) |
| 5.12 | Min(n , 2) |
| 10.24 | 1 |
| NOTE: n is signalled by the network by using numDRX-CycleRelaxed-r15 defined in TS 36.331 [2]. |  |

Table 4.6A.2.1A-2: The relaxation factor N for a UE configured with eDRX IDLE cycle

| DRX cycle length [s] | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2.56 ≤ PTW length [s] < 5.12 | 5.12 ≤ PTW length [s] < 7.68 | 7.68 ≤ PTW length [s] < 12.8 | 12.8 ≤ PTW length [s] < 23.04 | 23.04 ≤ PTW length [s] |
| 0.32 | Min(n , 2) | Min(n , 4) | Min(n , 8) | Min(n , 8) | Min(n , 8) |
| 0.64 | 1 | Min(n , 2) | Min(n , 4) | Min(n , 8) | Min(n , 8) |
| 1.28 | N/A | 1 | Min(n , 2) | Min(n , 4) | Min(n , 8) |
| 2.56 | N/A | N/A | 1 | Min(n , 2) | Min(n , 4) |
| 5.12 | N/A | N/A | N/A | 1 | Min(n , 2) |
| 10.24 | N/A | N/A | N/A | N/A | 1 |
| NOTE: n is signalled by the network by using numDRX-CycleRelaxed-r15 defined in TS 36.331 [2]. |  |  |  |  |  |

#### 4.6A.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage

The UE shall be able to identify new intra-frequency cells and perform NRSRP measurements of identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS36.304 within Ksatellite*Tdetect,NB_Intra_NC when Treselection= 0 .  An intra frequency cell is considered to be detectable according to NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot defined in Annex B.1.4 for a corresponding Band.

The UE shall measure NRSRP at least every Ksatellite*Tmeasure,NB_Intra_NC for intra-frequency cells that are identified and measured according to the measurement rules.

The UE shall filter NRSRP measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Ksatellite*Tmeasure,NB_Intra-NC/2

The UE shall not consider an NB-IoT neighbour cell in cell reselection if it is indicated as not allowed in the measurement control system information of the serving NB-IoT cell.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined [1] within Ksatellite*Tevaluate,NB_intra-NC when Treselection = 0, provided that the cell is at least XdB better ranked, where ‘X’ is specified in Table 4.6A.2.4-3. When evaluating cells for reselection, the side conditions for NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot apply to both serving and non-serving NB-IoT intra-frequency cells.

The paraemter Ksatellite is the scaling factor for measurements correspond to multiple NGSO satellites. Ksatellite = TBD

If Treselection timer has a non zero value and the intra-frequency cell is better ranked than the serving NB-IoT cell, the UE shall evaluate this intra-frequency cell for the Treselection time. If this cell remains better ranked within this duration, then the UE shall reselect that cell.

For UE not configured with eDRX_IDLE cycle, Tdetect,NB_Intra_NC, Tmeasure,NB_Intra_NC and Tevaluate, NB_intra_NC are specified in Table 4.6A.2.2-1. For UE configured with eDRX_IDLE cycle, Tdetect,NB_Intra -NC, Tmeasure,NB_Intra_NC and Tevaluate, NB_intra-NC are specified in Table 4.6A.2.2-2, where the requirements apply provided that the serving NB-IoT cell is configured with eDRX_IDLE and is the same in all PTWs during any of Tdetect,NB_Intra_NC, Tmeasure,NB_Intra_NC and Tevaluate, NB_intra_NC when multiple PTWs are used.

If the UE is configured with ‘t-Service-r17’ [2] in the serving cell and eDRX_IDLE, then the UE shall meet the requirements defined for DRX cycle length of [2.56] s for Tdetect,NB_Intra_NC, Tmeasure,NB_Intra_NC and Tevaluate, NB_intra_NC are specified in Table 4.6A.2.2-1 starting from at least [K] before ‘t-Service-r17’.

Table 4.6A.2.2-1 : Tdetect,NB_Intra -NC, Tmeasure,NB_Intra -NC and Tevaluate, NB_intra -NC

| DRX cycle length [s] | Tdetect,NB_Intra_NC [s] (number of DRX cycles) | Tmeasure,NB_Intra_NB_NC [s] (number of DRX cycles) | Tevaluate,NB_intra_NB_NC [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 26 (80) | 1.28 (4) | 5.12 (16) |
| 0.64 | 26 (40) | 1.28 (2) | 5.12 (8) |
| 1.28 | 51 (40) | 1.28 (1) | 6.5 (5) |
| 2.56 | 51 (20) | 2.56 (1) | 7.68 (3) |
| 5.12 | 102 (20) | 5.12 (1) | 10.24 (2) |
| 10.24 | 102 (10) | 10.24 (1) | 20.48 (2) |

Table 4.6A.2.2-2: Tdetect,NB_Intra_NC, Tmeasure,NB_Intra_NC and Tevaluate,NB_intra_NC for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 2.56s periods) | Tdetect,NB_Intra -NC [s] (number of DRX cycles) | Tmeasure,NB_Intra_NC [s] (number of DRX cycles) | Tevaluate,NB_intra_NC [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | ≥ 2.56 (1) | (20) | 1.28 (4) | 2.56 (8) |
|  | 0.64 | ≥ 2.56 (1) |  | 1.28 (2) | 2.56 (4) |
|  | 1.28 | ≥ 5.12 (2) |  | 1.28 (1) | 2.56 (2) |
|  | 2.56 | ≥ 7.68 (3) |  | 2.56 (1) | 5.12 (2) |
|  | 5.12 | ≥ 12.8 (5) |  | 5.12 (1) | 10.24 (2) |
|  | 10.24 | ≥ 23.04 (9) |  | 10.24 (1) | 20.48 (2) |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section X of TS 24.008 [34]. |  |  |  |  |  |

For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE has to meet the requirement corresponding to the second state.

If all the relaxed monitoring criteria defined in clause 5.2.4.12 [1] are fulfilled then the UE’s intra-frequency measurement is not required to meet Tdetect,NB_Intra_NC, Tmeasure,NB_Intra_NC and Tevaluate,NB_intra_NC as defined in Table 4.6A.2.2-1 and Table 4.6A.2.2-2.

If t-Service is broadcasted and applicable, UE shall be able to detect, measure, and evaluate neighbour cells before t-Service is reached, regardless of the rules currently limiting the UE measurement activities, and when to start the detection, measurement and evaluation on neighbour cells is up to UE implementation. This requirement does not apply when the time span from the last slot of SI transmission within SI modification period where the broadcasting of t-Service is started to the first slot when the cell is scheduled to stop serving the area according to the broadcasted information is less than Ttrigger.

Ttrigger = max(Tdetect,NB_Intra_NC , Pcarrier * Tdetect,NB_Inter_NC),

where

-	Pcarrier is the number of inter-frequency carriers for which carrier frequency information was provided by the serving NB-IoT cell,

-	Tdetect,NB_Intra_NC is defined in [4.6A.2.2], and Tdetect,NB_Inter_NC is defined in [4.6A.2.5].

The requirements in this clause apply provided that the valid information for the satellite serving the target cell has been provided by the serving cell.

#### 4.6A.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage

The UE shall measure the NRSRP and NRSRQ level of the serving NB-IoT cell on the anchor carrier and evaluate the cell selection criterion S defined in clause 5.2.3.2 in [1] for the serving NB-IoT cell on the anchor carrier at least every DRX cycle.

The UE shall filter the NRSRP and NRSRQ measurements of the serving NB-IoT cell using at least 4 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least DRX cycle/2.

If the UE is configured for receiving paging on the non-anchor carrier then the UE shall evaluate the cell selection criterion S defined in clause 5.2.3.2a in [1] for the serving NB-IoT cell on non-anchor carrier at least every DRX cycle.

The UE is allowed to measure NRSRP level of the serving NB-IoT cell, assuming that nrs-NonAnchor-config is enabled indicated by higher layer defined in clause 10.2.6 TS 36.211 [16], on non-anchor carrier provided that:

-	The relaxed monitoring criteria defined in TS 36.304 clause 5.2.4.12 are met,

-	Transmit power difference of the signals/channels between anchor- and non-anchor carriers is signalled to the UE, via the existing parameter nrs-PowerOffsetNonAnchor,

-	UE is not configured with any positioning measurements.

-	nB configured by higher layer is not equal to 4T.

When all the conditions for measuring NRSRP on non-anchor carrier are satisfied, the UE shall filter the NRSRP of the serving NB-IoT cell using at least 4 measurements, where the measurements used for the filtering may include measurements on anchor carrier and on non-anchor carrier.

If the UE is not configured with eDRX_IDLE cycle and has evaluated according to Table 4.6A.2.3-1 in Nserv_NB_EC consecutive DRX cycles that the serving NB-IoT cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving NB-IoT cell, regardless of the measurement rules currently limiting UE measurement activities. If the UE is configured with eDRX_IDLE cycle and has evaluated according to Table 4.6A.2.3-2 in Nserv_NB -EC consecutive DRX cycles within a single PTW that the serving NB-IoT cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving NB-IoT cell, regardless of the measurement rules currently limiting UE measurement activities. Additionally, if the UE is configured with t-Service [2], the UE should start measurements of the neighbour cells indicated by the serving cell before t-Service is reached according to the requirements provided in clause [4.6A.2.4] and [4.6A.2.6].

If t-Service is not provided nor applicable, and if the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency and inter-frequency information indicated in the system information during the time T, the UE shall initiate cell selection procedures for the selected PLMN as defined in [1], where T= 80 s if the UE is not configured with eDRX_IDLE cycle, and T=MAX(80 s, one eDRX_IDLE cycle) if the UE is configured with eDRX_IDLE cycle.

If t-Service is provided and applicable of the serving cell then the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 36.304 when any of the following conditions is fulfilled:

-	If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information within 80 s since time instance T1 provided that t-Service > T1 or

-	If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information within 80 s since the time instance t-Service.

-	Where, T1 is the time instance in seconds when the UE has determined that the serving cell does not fulfil the cell selection criterion S.

When the UE is provided with t-serviceStart-r17 and has discontinuous coverage capabilities, then after t-service-r17 is reached and the UE is out of coverage, the UE may delay or resume cell measurements/search till when the UE is in coverage.

Editor's Note: Definition of in coverage is FFS

Table 4.6A.2.3-1: Nserv_NB_EC

| DRX cycle length [s] | Nserv_NB -EC [number of DRX cycles] |
| --- | --- |
| 0.32 | 4 |
| 0.64 | 4 |
| 1.28 | 4 |
| 2.56 | 4 |
| 5.12 | 4 |
| 10.24 | 4 |

Table 4.6A.2.3-2: Nserv_NB_EC for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 2.56s periods) | Nserv_NB_EC [number of DRX cycles] |
| --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | ≥ 2.56 (1) | 4 |
|  | 0.64 | ≥ 5.12 (2) | 4 |
|  | 1.28 | ≥ 7.68 (3) | 4 |
|  | 2.56 | ≥ 12.8 (5) | 4 |
|  | 5.12 | ≥ 23.04 (9) | 4 |
|  | 10.24 | ≥ 43.52 (17) | 4 |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section X of TS 24.008 [34]. |  |  |  |

For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE has to meet the requirement corresponding to the second state.

#### 4.6A.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS

The UE which supports wakeUpSignal [2] shall meet the requirement defined for the DRX cycle length of N*DRX_cycle in Section 4.6A.2.3, provided the following conditions are met:

-	WUS has been configured in the serving NB-IoT cell using WUS-Config-NB-r15 [2], and

-	The serving cell measurement relaxation is signalled as n by the network using numDRX-CycleRelaxed-r15, and

-	Serving cell S criteria is met with at least 2 dB margin.

-	the relaxed monitoring criteria for neighbour cells in TS 36.304 [1] clause 5.2.4.12.1 is fulfilled, and

, where the relaxation factor N is given by Table 4.6A.2.3A-1. Otherwise the requirements defined for the configured DRX cycle length in Section 4.6A.2.3 shall apply.

The UE shall further meet the requirements in section 4.6A.2.3 during time period T0 after following occasions:

-	after the end of reception of latest paging message, or

-	from the moment UE has switched from RRC_CONNECTED state to RRC_IDLE state.

T0 = N*DRX cycle if the UE is not configured with eDRX_IDLE cycle where the value of N specified in Table 4.6A.2.3A-1;

T0 = one eDRX IDLE cycle if the UE is configured with eDRX_IDLE cycle;

Table 4.6A.2.3A-1: The relaxation factor N for a UE not configured with eDRX IDLE cycle

| DRX cycle length [s] | Value |
| --- | --- |
| 0.32 | Min(n , 8) |
| 0.64 | Min(n , 8) |
| 1.28 | Min(n , 8) |
| 2.56 | Min(n , 4) |
| 5.12 | Min(n , 2) |
| 10.24 | 1 |
| NOTE: n is signalled by the network by using numDRX-CycleRelaxed-r15 defined in TS 36.331 [2]. |  |

Table 4.6A.2.3A-2: The relaxation factor N for a UE configured with eDRX IDLE cycle

| DRX cycle length [s] | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2.56 ≤ PTW length [s] < 5.12 | 5.12≤ PTW length [s] < 7.68 | 7.68 ≤ PTW length [s] < 12.8 | 12.8 ≤ PTW length [s] < 23.04 | 23.04 ≤ PTW length [s] < 43.52 | 43.52 ≤ PTW length [s] |
| 0.32 | 1 | Min(n , 2) | Min(n , 4) | Min(n , 8) | Min(n , 8) | Min(n , 8) |
| 0.64 | N/A | 1 | Min(n , 2) | Min(n , 4) | Min(n , 8) | Min(n , 8) |
| 1.28 | N/A | N/A | 1 | Min(n , 2) | Min(n , 4) | Min(n , 8) |
| 2.56 | N/A | N/A | N/A | 1 | Min(n , 2) | Min(n , 4) |
| 5.12 | N/A | N/A | N/A | N/A | 1 | Min(n , 2) |
| 10.24 | N/A | N/A | N/A | N/A | N/A | 1 |
| NOTE: n is signalled by the network by using numDRX-CycleRelaxed-r15 defined in TS 36.331 [2]. |  |  |  |  |  |  |

#### 4.6A.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage

The UE shall be able to identify new intra-frequency cells and perform NRSRP measurements of identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS36.304 within Ksatellite*Tdetect,NB_Intra_EC when that Treselection= 0 .  An intra frequency cell is considered to be detectable according to NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot defined in Annex B.1.4 for a corresponding Band.

The UE shall measure NRSRP at least every Ksatellite*Tmeasure,NB_Intra_EC for intra-frequency cells that are identified and measured according to the measurement rules.

The UE shall filter NRSRP measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Ksatellite*Tmeasure,NB_Intra_ EC/2

The UE shall not consider a NB-IoT neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving NB-IoT cell.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined [1] within Ksatellite*Tevaluate,NB_intra_EC when Treselection = 0, provided that the cell is at least XdB better ranked, where ‘X’ is specified in Table 4.6A.2.4-3. When evaluating cells for reselection, the side conditions for NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot apply to both serving and non-serving NB-IoT intra-frequency cells.

The paraemter Ksatellite is the scaling factor for measurements correspond to multiple NGSO satellites. Ksatellite = TBD

If Treselection timer has a non zero value and the intra-frequency cell is better ranked than the serving NB-IoT cell, the UE shall evaluate this intra-frequency cell for the Treselection time. If this cell remains better ranked within this duration, then the UE shall reselect that cell.

For UE not configured with eDRX_IDLE cycle, Tdetect,NB_Intra_EC, Tmeasure,NB_Intra_EC and Tevaluate, NB_intra_EC are specified in Table 4.6A.2.4-1. For UE configured with eDRX_IDLE cycle, Tdetect,NB_Intra_EC, Tmeasure,NB_Intra_EC and Tevaluate, NB_intra_EC are specified in Table 4.6A.2.4-2, where the requirements apply provided that the serving NB-IoT cell is configured with eDRX_IDLE and is the same in all PTWs during any of Tdetect,NB_Intra_EC, Tmeasure,NB_Intra_EC and Tevaluate, NB_intra_EC when multiple PTWs are used.

If the UE is configured with ‘t-Service-r17’ [2] in the serving cell and eDRX_IDLE, then the UE shall meet the requirements defined for DRX cycle length of [2.56] s for Tdetect,NB_Intra_NC, Tmeasure,NB_Intra_NC and Tevaluate, NB_intra_NC are specified in Table 4.6A.2.4-1 starting from at least [K] before ‘t-Service-r17’.

Table 4.6A.2.4-1 : Tdetect,NB_Intra_EC, Tmeasure,NB_Intra_EC and Tevaluate, NB_intra_EC

| SCH Ês/Iot of neighboring cell: Q2 | DRX cycle length [s] | Tdetect,NB_Intra_ EC [s] (number of DRX cycles) | Tmeasure,NB_Intra_ EC [s] (number of DRX cycles) | Tevaluate,NB_intra_ EC [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- |
| -15≤ Q2 < -6 | 0.32 | 256 (800) | 1.28 (4) | 10.24 (32) |
|  | 0.64 | 266 (415) | 1.28 (2) | 10.24 (16) |
|  | 1.28 | 532 (415) | 1.28 (1) | 12.8 (10) |
|  | 2.56 | 532 (208) | 2.56 (1) | 15.36 (6) |
|  | 5.12 | 1063 (208) | 5.12 (1) | 20.48 (4) |
|  | 10.24 | 1063 (104) | 10.24 (1) | 30.72 (3) |
| Q2-6 | 0.32 | 26 (80) | 1.28 (4) | 10.24 (32) |
|  | 0.64 | 29 (45) | 1.28 (2) | 10.24 (16) |
|  | 1.28 | 58 (45) | 1.28 (1) | 12.8 (10) |
|  | 2.56 | 59 (23) | 2.56 (1) | 15.36 (6) |
|  | 5.12 | 113 (22) | 5.12 (1) | 20.48 (4) |
|  | 10.24 | 113 (11) | 10.24 (1) | 30.72 (3) |

Table 4.6A.2.4-2: Tdetect,NB_Intra_EC, Tmeasure,NB_Intra_EC and Tevaluate,NB_intra_EC for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 2.56s periods) | Tdetect,NB_Intra_EC [s] (number of DRX cycles) | Tmeasure,NB_Intra_EC [s] (number of DRX cycles) | Tevaluate,NB_intraEC [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | ≥12.8 (5) | (406) | 1.28 (4) | 10.24 (32) |
|  | 0.64 | ≥12.8 (5) |  | 1.28 (2) | 10.24 (16) |
|  | 1.28 | ≥15.36 (6) |  | 1.28 (1) | 12.8 (10) |
|  | 2.56 | ≥17.92 (7) |  | 2.56 (1) | 15.36 (6) |
|  | 5.12 | ≥23.04 (9) |  | 5.12 (1) | 20.48 (4) |
|  | 10.24 | ≥33.28 (13) |  | 10.24 (1) | 30.72 (3) |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section X of TS 24.008 [34]. |  |  |  |  |  |

Table 4.6A.2.4-3: Conditions on NSCH Ês/Iot of identified and of the neighbour cell

| NSCH Ês/Iot of already identified cell including serving cell: Q1 | Neighbouring cell NSCH Ês/Iot: Q2 | Cell Reselection Margin ‘X’ |
| --- | --- | --- |
| -15≤Q1<-6 | -15≤ Q2 < -6 | 8.3 |
| -15≤Q1<-6 | Q2 -6 | 8.3 |
| Q1 -6 | Q2 -6 | 4 |

For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE has to meet the requirement corresponding to the second state.

If all the relaxed monitoring criteria defined in clause 5.2.4.12 [1] are fulfilled then the UE’s intra-frequency measurement is not required to meet Tdetect,NB_Intra_EC, Tmeasure,NB_Intra_EC and Tevaluate,NB_intra_EC as defined in Table 4.6A.2.4-1 and Table 4.6A.2.4-2.

If t-Service is broadcasted and applicable, UE shall be able to detect, measure, and evaluate neighbour cells before t-Service is reached, regardless of the rules currently limiting the UE measurement activities, and when to start the detection, measurement and evaluation on neighbour cells is up to UE implementation. This requirement does not apply when the time span from the last slot of SI transmission within SI modification period where the broadcasting of t-Service is started to the first slot when the cell is scheduled to stop serving the area according to the broadcasted information is less than Ttrigger.

Ttrigger = max(Tdetect,NB_Intra_EC , Pcarrier * Tdetect,NB_Inter_EC),

where

-	Pcarrier is the number of inter-frequency carriers for which carrier frequency information was provided by the serving NB-IoT cell,

-	Tdetect,NB_Intra_EC is defined in [4.6A.2.4], and Tdetect,NB_Inter_EC is defined in [4.6A.2.6].

The requirements in this clause apply provided that the valid information for the satellite serving the target cell has been provided by the serving cell.

#### 4.6A.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage

The UE shall be able to identify new inter-frequency cells and perform NRSRP measurements of identified inter-frequency cells if carrier frequency information is provided by the serving NB-IoT cell, even if no explicit neighbour list with physical layer cell identities is provided.

If Srxlev ≤ SnonIntraSearchP then the UE shall search for and measure inter-frequency layers in preparation for possible reselection.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS36.304 within Ksatellite *Pcarrier * Tdetect,NB_Inter_NC, if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving NB-IoT cells when Treselection = 0 provided that the reselection criteria is met by a margin of at least Y dB, where Pcarrier is the number of inter-frequency carriers for which carrier frequency information was provided by the serving NB-IoT cell and ‘Y’ is specified by Table 4.6A.2.6-3 (when Q1 -6 dB). An inter-frequency cell is considered to be detectable according to NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot defined in Annex B.1.5 for a corresponding Band.

The UE shall filter NRSRP measurements of each measured inter-frequency cell using at least [2] measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Ksatellite *Tmeasure, Inter_NB-IoT_NC/2.

If an inter-frequency cell has been already detected but that has not been reselected to the filtering shall be such that the UE shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 36.304 within Ksatellite * Pcarrier * Tevaluate,NB_Inter_NC. When evaluating cells for reselection, the side conditions for NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot apply to both serving and inter-frequency cells.

The paraemter Ksatellite is the scaling factor for measurements correspond to multiple NGSO satellites. Ksatellite = TBD

If Treselection timer has a non zero value and the inter-frequency cell is better ranked than the serving NB-IoT cell, the UE shall evaluate this inter-frequency cell for the Treselection time. If this cell remains better ranked within this duration, then the UE shall reselect that cell.

For UE not configured with eDRX_IDLE cycle, Tdetect,NB_Inter_NC, Tmeasure,NB_Inter_NC and Tevaluate, NB_inter_NC are specified in Table 4.6A.2.5-1. For UE configured with eDRX_IDLE cycle, Tdetect,NB_Inter_NC, Tmeasure,NB_Inter_NC and Tevaluate, NB_inter_NC are specified in Table 4.6A.2.5-2, where the requirements apply provided that the serving NB-IoT cell is configured with eDRX_IDLE and is the same in all PTWs during any of Tdetect,NB_Inter_NC, Tmeasure,NB_Inter_NC and Tevaluate, NB_inter_NC when multiple PTWs are used.

If the UE is configured with ‘t-Service-r17’ [2] in the serving cell and eDRX_IDLE, then the UE shall meet the requirements defined for DRX cycle length of [2.56] s for Tdetect,NB_Inter_NC, Tmeasure,NB_Inter_NC and Tevaluate, NB_inter_NC are specified in Table 4.6A.2.5-1 starting from at least [K] before ‘t-Service-r17’.

Table 4.6A.2.5-1 : Tdetect,NB_Inter_NC, Tmeasure,NB_Inter_NC and Tevaluate,NB_Inter_NC

| DRX cycle length [s] | Tdetect,NB_Inter_ NC [s] (number of DRX cycles) | Tmeasure,NB_Inter_ NC [s] (number of DRX cycles) | Tevaluate,NB_Inter_ NC [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 26 (80) | 1.28 (4) | 5.12 (16) |
| 0.64 | 26 (40) | 1.28 (2) | 5.12 (8) |
| 1.28 | 51 (40) | 1.28 (1) | 6.5 (5) |
| 2.56 | 51 (20) | 2.56 (1) | 7.68 (3) |
| 5.12 | 102 (20) | 5.12 (1) | 10.24 (2) |
| 10.24 | 102 (10) | 10.24 (1) | 20.48 (2) |

Table 4.6A.2.5-2: Tdetect,NB_Inter_ NC, Tmeasure,NB_Inter_NC and Tevaluate, NB_inter_ NC for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] |  | DRX cycle length [s] |  | PTW length [s] (number of 2.56s periods) |  | Tdetect,NB_Inter_NC [s] (number of DRX cycles) | Tmeasure,NB_Inter_NC [s] (number of DRX cycles) | Tevaluate,NB_inter_ NC [s] (number of DRX cycles) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76 |  | 0.32 |  | ≥ 2.56 (1) |  | (20) |  | 1.28 (4) | 2.56 (8) |
|  |  | 0.64 |  | ≥ 2.56 (1) |  |  |  | 1.28 (2) | 2.56 (4) |
|  |  | 1.28 |  | ≥5.12 (2) |  |  |  | 1.28 (1) | 2.56 (2) |
|  |  | 2.56 |  | ≥7.68 (3) |  |  |  | 2.56 (1) | 5.12 (2) |
|  |  | 5.12 |  | ≥12.8 (5) |  |  |  | 5.12 (1) | 10.24 (2) |
|  |  | 10.24 |  | ≥23.04 (9) |  |  |  | 10.24 (1) | 20.48 (2) |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section X of TS 24.008 [34]. |  |  |  |  |  |  |  |  |  |

For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE has to meet the requirement corresponding to the second state.

If all the relaxed monitoring criteria defined in clause 5.2.4.12 [1] are fulfilled then the UE’s inter-frequency measurement is not required to meet Tdetect,NB_Intra_EC, Tmeasure,NB_Intra_EC and Tevaluate,NB_intra_EC as defined in Table 4.6A.2.5-1 and Table 4.6A.2.5-2.

If t-Service is broadcasted and applicable, UE shall be able to detect, measure, and evaluate neighbour cells before t-Service is reached, regardless of the rules currently limiting the UE measurement activities, and when to start the detection, measurement and evaluation on neighbour cells is up to UE implementation. This requirement does not apply when the time span from the last slot of SI transmission within SI modification period where the broadcasting of t-Service is started to the first slot when the cell is scheduled to stop serving the area according to the broadcasted information is less than Ttrigger.

Ttrigger = max(Tdetect,NB_Intra_NC , Pcarrier * Tdetect,NB_Inter_NC),

where

-	Pcarrier is the number of inter-frequency carriers for which carrier frequency information was provided by the serving NB-IoT cell,

-	Tdetect,NB_Intra_NC is defined in [4.6A.2.2], and Tdetect,NB_Inter_NC is defined in [4.6A.2.5].

The requirements in this clause apply provided that the valid information for the satellite serving the target cell has been provided by the serving cell.

#### 4.6A.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage

The UE shall be able to identify new inter-frequency cells and perform NRSRP measurements of identified inter-frequency cells if carrier frequency information is provided by the serving NB-IoT cell, even if no explicit neighbour list with physical layer cell identities is provided.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS36.304 within Ksatellite *Pcarrier * Tdetect,NB_Inter_EC. An inter-frequency cell is considered to be detectable according to NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot defined in Annex B.1.5 for a corresponding Band.

The UE shall not cause any interruption to the paging reception and acquisition of SI while performing measurement on serving or any neighbor cells.

The UE shall filter NRSRP measurements of each measured inter-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Ksatellite *Tmeasure,NB_Inter _EC/2.

If an inter-frequency cell has been already detected but that has not been reselected to the filtering shall be such that the UE shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 36.304 within Ksatellite * Pcarrier * Tevaluate,NB_Inter_EC. When evaluating cells for reselection, the side conditions for NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot apply to both serving and inter-frequency NB-IoT cells.

The paraemter Ksatellite is the scaling factor for measurements correspond to multiple NGSO satellites. Ksatellite = TBD

If Treselection timer has a non zero value and the inter-frequency cell is better ranked than the serving NB-IoT cell, the UE shall evaluate this inter-frequency cell for the Treselection time. If this cell remains better ranked within this duration, then the UE shall reselect that cell.

For a UE not configured with eDRX_IDLE cycle, Tdetect,NB_Inter_EC, Tmeasure,NB_Inter_EC and Tevaluate, NB_inter_EC are specified in Table 4.6A.2.6-1. For UE configured with eDRX_IDLE cycle, Tdetect,NB_Inter_EC, Tmeasure,NB_Inter_EC and Tevaluate, NB_inter_EC are specified in Table 4.6A.2.6-2 for the UE in enhanced coverage, where the requirements apply provided that the serving NB-IoT cell is configured with eDRX_IDLE and is the same in all PTWs during any of Tdetect,NB_Inter_EC, Tmeasure,NB_Inter_EC and Tevaluate, NB_inter_EC when multiple PTWs are used.

If the UE is configured with ‘t-Service-r17’ [2] in the serving cell and eDRX_IDLE, then the UE shall meet the requirements defined for DRX cycle length of [2.56] s for Tdetect,NB_Inter_NC, Tmeasure,NB_Inter_NC and Tevaluate, NB_inter_NC are specified in Table 4.6A.2.6-1 starting from at least [K] before ‘t-Service-r17’.

Table 4.6A.2.6-1 : Tdetect,NB_Inter_EC, Tmeasure,NB_Inter_EC and Tevaluate,NB_Inter_EC

| SCH Ês/Iot of neighboring cell: Q2 | DRX cycle length [s] | Tdetect,NB_Inter_ EC [s] (number of DRX cycles) | Tmeasure,NB_Inter_ EC [s] (number of DRX cycles) | Tevaluate,NB_Inter_ EC [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- |
| -15≤ Q2 < -6 | 0.32 | 256 (800) | 1.28 (4) | 10.24 (32) |
|  | 0.64 | 266 (415) | 1.28 (2) | 10.24 (16) |
|  | 1.28 | 532 (415) | 1.28 (1) | 12.8 (10) |
|  | 2.56 | 532 (208) | 2.56 (1) | 15.36 (6) |
|  | 5.12 | 1063 (208) | 5.12 (1) | 20.48 (4) |
|  | 10.24 | 1063 (104) | 10.24 (1) | 30.72 (3) |
| Q2-6 | 0.32 | 26 (80) | 1.28 (4) | 10.24 (32) |
|  | 0.64 | 29 (45) | 1.28 (2) | 10.24 (16) |
|  | 1.28 | 58 (45) | 1.28 (1) | 12.8 (10) |
|  | 2.56 | 59 (23) | 2.56 (1) | 15.36 (6) |
|  | 5.12 | 113 (22) | 5.12 (1) | 20.48 (4) |
|  | 10.24 | 113 (11) | 10.24 (1) | 30.72 (3) |

Table 4.6A.2.6-2: Tdetect,NB_Inter_EC, Tmeasure,NB_Inter_EC and Tevaluate, NB_inter_EC for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX cycle length [s] |  | PTW length [s] (number of 2.56s periods) | Tdetect,NB_Inter_EC [s] (number of DRX cycles) | Tmeasure,NB_Inter_EC [s] (number of DRX cycles) | Tevaluate,NB_interEC [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | ≥12.8 (5) |  | ([406]) | 1.28 (4) | 10.24 (32) |
|  | 0.64 | ≥12.8 (5) |  |  | 1.28 (2) | 10.24 (16) |
|  | 1.28 | ≥15.36 (6) |  |  | 1.28 (1) | 12.8 (10) |
|  | 2.56 | ≥17.92 (7) |  |  | 2.56 (1) | 15.36 (6) |
|  | 5.12 | ≥23.04 (9) |  |  | 5.12 (1) | 20.48 (4) |
|  | 10.24 | ≥33.28 (13) |  |  | 10.24 (1) | 30.72 (3) |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section X of TS 24.008 [34]. |  |  |  |  |  |  |

Table 4.6A.2.6-3: Conditions on NSCH Ês/Iot of identified and of the neighbour cell

| NSCH Ês/Iot of already identified cell including serving cell: Q1 | Neighbouring cell NSCH Ês/Iot: Q2 | Cell Reselection Margin ‘Y’ |
| --- | --- | --- |
| -15≤Q1<-6 | -15≤ Q2 < -6 | 9.3 |
| -15≤Q1<-6 | Q2-6 | 9.3 |
| Q1 -6 | Q2-6 | 5 |

For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE has to meet the requirement corresponding to the second state.

If all the relaxed monitoring criteria defined in clause 5.2.4.12 [1] are fulfilled then the UE’s inter-frequency measurement is not required to meet Tdetect,NB_Intra_EC, Tmeasure,NB_Intra_EC and Tevaluate,NB_intra_EC as defined in Table 4.6A.2.6-1 and Table 4.6A.2.6-2.

If t-Service is broadcasted and applicable, UE shall be able to detect, measure, and evaluate neighbour cells before t-Service is reached, regardless of the rules currently limiting the UE measurement activities, and when to start the detection, measurement and evaluation on neighbour cells is up to UE implementation. This requirement does not apply when the time span from the last slot of SI transmission within SI modification period where the broadcasting of t-Service is started to the first slot when the cell is scheduled to stop serving the area according to the broadcasted information is less than Ttrigger.

Ttrigger = max(Tdetect,NB_Intra_EC , Pcarrier * Tdetect,NB_Inter_EC),

where

-	Pcarrier is the number of inter-frequency carriers for which carrier frequency information was provided by the serving NB-IoT cell,

-	Tdetect,NB_Intra_EC is defined in [4.6A.2.4], and Tdetect,NB_Inter_EC is defined in [4.6A.2.6].

The requirements in this clause apply provided that the valid information for the satellite serving the target cell has been provided by the serving cell.

#### 4.6A.2.7	Maximum interruption in paging reception in normal coverage

UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. When the UE is configured with eDRX_IDLE cycle, the UE shall not miss any paging in a PTW provided the paging is sent in at least 2 DRX cycles before the end of that PTW.

At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving NB-IoT cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed

- TSI-NB1-EC + 100 ms, if the target cell belongs to the same satellite as the current one, and if the target cell is known.

- TSI-NB1-EC + 100 ms, If the target cell belongs to a different satellite than the current one and the target cell’s satellite is GEO , and if the target cell is known.

- TSI-NB1-EC + [250] ms, if the target cell belongs to a different satellite than the current one and the target cell’s satellite is non-GEO, and if the target cell is known.

The target cell is considered as known if it has been detectable during Tdetect,NR_Intra_NC or Tdetect,NR_Inter_NC, and the time span between SIB broadcasting cell stop time and the cell stop time is not less than Ttrigger. Otherwise, the target cell is considered as unknown, where Tdetect,NR_Intra_NC, Tdetect,NR_Inter_NC and Ttrigger are defined in [4.6A.2.2] and [4.6A.2.5]. A longer interruption can be expected if the target cell is unknown.

Editor’s note: For NGSO, FFS whether to clarify UE is allowed to drop paging during [2] DRX cycles immediately after ‘t-ServiceStart-r17’.

#### 4.6A.2.7A	Maximum interruption in paging reception in enhanced coverage

UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. When the UE is configured with eDRX_IDLE cycle, the UE shall not miss any paging in a PTW provided the paging is sent in at least 2 DRX cycles before the end of that PTW.

At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving NB-IoT cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed

- TSI-NB1-EC + 100 ms, if the target cell belongs to the same satellite as the current one, and if the target cell is known.

- TSI-NB1-EC + 100 ms, If the target cell belongs to a different satellite than the current one and the target cell’s satellite is GEO, and if the target cell is known.

- TSI-NB1-EC + [250] ms, if the target cell belongs to a different satellite than the current one and the target cell’s satellite is non-GEO, and if the target cell is known.

The target cell is considered as known if it has been detectable during Tdetect,NB_Intra_EC or Tdetect,NB_Inter_EC, and the time span between SIB broadcasting cell stop time and the cell stop time is not less than Ttrigger. Otherwise, the target cell is considered as unknown, where Tdetect,NB_Intra_EC, Tdetect,NB_Inter_EC and Ttrigger are defined in [4.6A.2.4] and [4.6A.2.6]. A longer interruption can be expected if the target cell is unknown.

Editor’s note: For NGSO, FFS whether to clarify UE is allowed to drop paging during [2] DRX cycles immediately after ‘t-ServiceStart-r17’.

#### 4.6A.2.8	UE measurement capability

For idle mode cell re-selection purposes, the UE shall be capable of monitoring at least:

-	Depending on UE capability, an intra-frequency carrier.

-	Depending on UE capability, at least 2 inter-frequency carriers.

In LEO deployments, the UE shall be capable of monitoring:

-	for intra-frequency carrier, the number of target satellites UE needs to monitor is  at least [2] including serving LEO satellite.

-	for inter-frequency carrier, the number of target satellites UE needs to monitor per carrier is at least [2] if one of the target satellites include the UE serving satellite; the number of target satellites UE needs to monitor is at least [1] otherwise

-	In addition to the requirements defined above, the UE which operates in LEO deployment shall be capable of monitoring a total of at least [2] satellites including the serving satellite.

#### 4.6A.2.9	WUS receptions for NB1

This clause contains requirements on the UE regarding WUS reception provided that the WUS has been configured in the serving NB-IoT cell.

The UE shall be capable of receiving the WUS signals of the serving NB-IoT cell provided that the minimum number of repetitions configured in the NB-IoT serving cell is according to Table 4.6A.2.9-1 for normal coverage and Table 4.6A.2.9-2 for enhanced coverage.

The requirements in this clause also apply when UE is configured to monitor 2 sequences within the same resource.

Table 4.6A.2.9-1: Conditions for WUS reception for UE normal coverage level

| DRX cycle length [s] | Required number of repetition of WUS signal with 1 transmit antenna | Required number of repetition of WUS signal with 2 transmit antennas |
| --- | --- | --- |
| ≤ 5.12 | 64 | 32 |
| >5.12 | 128 | 64 |

Table 4.6A.2.9-2: Conditions for WUS reception for UE enhanced coverage level

| DRX cycle length [s] | Required number of repetition of WUS signal with 1 transmit antenna | Required number of repetition of WUS signal with 2 transmit antennas |
| --- | --- | --- |
| ≤ 5.12 | 128 | 64 |
| > 5.12 | 256 | 128 |

### 4.6A.3	Requirements for transmission using preconfigured uplink resources for UE category NB-IoT for Satellite Access

#### 4.6A.3.1	Introduction

The requirements in this clause are applicable when the UE is configured with timing alignment (TA) validation for transmitting in uplink using preconfigured uplink resources (PUR) as specified in [TS 36.331].

#### 4.6A.3.2	Requirements on UE synchronization for transmission using PUR

The requirements in this clause are applicable for the UE in normal coverage or in enhanced coverage.

The UE is allowed to transmit using the preconfigured uplink resources provided that the UE is synchronized towards the serving cell prior to transmission. If the UE is not able to obtain the synchronization towards the serving cell then the UE shall drop the PUR transmission. The UE determines the PUR transmission occasion according to the received PUR configuration [TS 36.331].

#### 4.6A.3.3	Requirements on TA validation for transmission using PUR

The UE is allowed to transmit using PUR provided that TA is valid.  [ FFS for the validation conditions]

The UE shall determine the uplink timing,  TTA, for transmitting on PUR using the valid values of NTA, NTA,offset, NTA,adjcommon and   NTA,adjUE where TTA is defined in [16] which are the latest acquired values before PUR transmission.

<End of Change #1>