3GPP TSG-RAN WG4 Meeting #105	R4-2220417

Toulouse, France, November 14 – November 18, 2022

Title: 	WF on RRM requirements for NR Multi-carrier enhancements

Agenda Item:	8.20.3

Source: 	Huawei, HiSilicon

Document for:	Approval

Topic #1:	DL interruption for Tx switching across 3/4 bands with 2TAGs

<Agreement>: Issue 1-1: RTD for non-collocated inter-band scenario for Tx switching with 2 TAGs

Agreements on GTW:

  - Use RTD=6us side condition for non-collocated inter-band case to derive DL interruption length for 2TAGs case.

  - FFS if requirements for RTD > 6us is needed

  - Note: RTD value will be captured in the specification as an informational note in the table for DL interruption length for 2TAGs as an assumption to derive the values.

<Agreement>: Issue 1-2: The principle of deriving DL interruption length for 2TAGs

As R16/R17 the interruption length in symbols due to UL TX switching is define as,

Ceil((switching period+2*TA adjustment uncertainty+2*RTD-CP length)/symbol duration)+1

<Agreement>: Issue 1-3: DL interruption length for 2TAGs

Agreements on GTW:

- Use the following DL interruption length (in symbols) for R18 Tx switching across 3/4 bands with 2 TAGs for RTD = 6us

|  | NR Slot length (ms) | Uplink Tx switching period |  |  |
| --- | --- | --- | --- | --- |
|  |  | 35us | 140us | 210us |
| 0 | 1 | 2 | 3 | 4 |
| 1 | 0.5 | 3 | 6 | [7 or 8] |
| 2 | 0.25 | 4 | 10 | 14 |

<Agreement>: Issue 1-4: Applicable scenario of DL interruption requirements

It is recommended not to further discuss this issue in RRM session. RRM can directly use the conclusion of RANP or RF session.

Topic #2: UL outage time for TX switching with 2 TAGs

This topic is closed as the below agreement is achieved in RF session.

Agreement: outage time will be discussed in RF session.