3GPP TSG-RAN WG4 Meeting #105 	R4-2220436

# Toulouse, France, Nov. 14 – Nov. 18, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on general configurations in R17 feMIMO test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Perf |  |  |  |  |  | Date: |  |  | 2022-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. As discussed in P2 and P3 of R4-2219174, for the uplink TCI configuration in DL TCI swiching test cases, the target UL TCI is also switched, but the pathloss RS is maintained before the UL TCI swiching. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. The UL TCI configuration is clarified to include the case when CDP is considered in the test case, and PL-RS is confiugred as maintained. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case is not correct or not clear enough. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.16A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2217591 |  |  |  |  |  |  |  |  |

<Start of Change #1>

A.3.16A	Unified TCI State Configuration

A.3.16A.1	Introduction

This clause provides the configurations for unified TCI states towards either SSB or CSI-RS. The DLorJoint TCI states defined in this clause are configured in each test when applicable to indicate that certain DL (and UL, if joint DL/UL operation is configured) signals are QCL’ed with the referenceSignal configured in the TCI states. The UL TCI states defined in this clause are configured in each test when applicable to indicate that certain UL signals are QCL’ed with the referenceSignal configured in the TCI states.

A.3.16A.2	DLorJoint TCI states

Table A.3.16A.2-1: DLorJoint TCI States

| Parameter | DLorJoint TCI.State.0 | DLorJoint TCI.State.1 | DLorJoint TCI.State.2 | DLorJoint TCI.State.3 | DLorJoint TCI.State.4 |  | DLorJoint TCI.State.5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| tci-StateUnifiedId | Id0 | Id1 | Id2 | Id3 | Id4 |  | Id5 |
| qcl-Type1 | typeA | typeA | typeA | typeA | typeC |  | typeC |
| qcl-Type2Note1 | typeD | typeD | typeD | typeD | typeD |  | typeD |
| referenceSignal Note2 | Resource #4 in TRS resource set 1 Note3 | Resource #4 in TRS resource set 2 Note 5 | Resource #4 in TRS resource set 1 Note3 | Resource #4 in TRS resource set 2 Note3 | SSB0 |  | SSB1 from the cell with different PCI |
| pathlossReferenceRS | N/A | N/A | Resource #4 in TRS resource set 1 Note3 | Resource #4 in TRS resource set 1 Note3 | N/A |  | N/A |
| additionalPCI | N/A | configured Note4 | N/A | N/A | N/A |  | configured Note4 |
| Note 1: qcl-Type2 of typeD only where applicable. For RRM test cases, this will be only in FR2 Note 2: referenceSignal configurations towards which the TCI states are configured are defined in a test-specific manner. Note 3: Reference TRS resource sets are defined in A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. The TCI state of the TRS is the DLorJoint TCI.State.4. Note 4: Only one PCI than serving cell PCI is included in the additionalPCIList, and the additionalPCIIndex is configured as 0. Note 5: Reference TRS resource sets are defined in A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. The TCI state of the TRS is the DLorJoint TCI.State.5. |  |  |  |  |  |  |

A.3.16A.3	UL TCI states

Table A.3.16A.3-1: UL TCI States

| Parameter | UL TCI.State.0 | UL TCI.State.1 | UL TCI.State.2 | UL TCI.State.3 |
| --- | --- | --- | --- | --- |
| ul-TCIState-Id | Id0 | Id1 | Id2 | Id3 |
| referenceSignal Note1 | SSB0 | SSB1 | Resource #4 in TRS resource set 1 Note2 | Resource #4 in TRS resource set 2 Note2 |
| pathlossReferenceRS | Resource #4 in TRS resource set 1 Note2 | Resource #4 in TRS resource set 1 Note2 | Resource #4 in TRS resource set 1 Note2 | Resource #4 in TRS resource set 2 Note2 |
| additionalPCI | N/A | configured Note3 | N/A | N/A |
| Note 1: referenceSignal configurations towards which the UL TCI states are configured are defined in a test-specific manner. Note 2: Reference TRS resource sets are defined in A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. Note 3: Only one PCI than serving cell PCI is included in the additionalPCIList, and the additionalPCIIndex is configured as 0. |  |  |  |  |

<End of Change #1>