3GPP TSG-RAN WG4 Meeting # 105	R4-2220451

# Toulouse, France, November 14 – November 18, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on signaling characteristics test cases for FR2-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Perf |  |  |  |  |  | Date: |  |  | 2022-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The test cases for PSCell addition/release and TCI switching requirements for FR2-2 with CCA shall be defined based on WF R4-2217195 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce test cases for PSCell addition/release and TCI switching requirements for FR2-2 with CCA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The corresponding test cases are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.X.Y.Z.1(new) , A.X.Y.Z.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of change 1>

#### A.X.Y.Z.1	Addition and Release Delay of unknown NR PSCell in FR2-2 with CCA

##### A.X.Y.Z.1.1	Test Purpose and Environment

The purpose of this test is to verify the PSCell addition and release delay requirements defined in clauses 8.9.2 and 8.9.3, respectively, for the case where the PSCell is unknown to the UE at the time of addition.

The supported test configurations are given in Table A.X.Y.Z.1.1-1. The test scenario comprises two NR cells, Cell 1 and Cell 2, on radio channel 1 in FR1 and radio channel 2 in FR2-2 with CCA, respectively. Test parameters are given in Tables A.X.Y.Z.1.1-2, A.X.Y.Z.1.1-3 and A.X.Y.Z.1.1-4 below. The test consists of four time periods with durations T1, T2, T3 and T4, respectively.

At the start of T1, the UE shall be connected to Cell 1 (PCell) on radio channel 1 (PCC) and shall only monitor PCC and hence be unaware of Cell 2 (PSCell-to-be) on radio channel 2. At the end of T1, the test system shall send a RRC message instructing the UE to add PSCell (Cell 2), and further instructing the UE to report CSI periodically in the PSCell once it has been added. Reception by the UE of this RRC message defines the start of T2.

During T2, the UE shall identify PSCell and carry out random access towards the PSCell. Reception by the test system of the PRACH preamble defines the start of T3.

During T3, the UE shall send periodic CSI reports in PSCell. After having received at least one such report, the test system shall send a RRC message instructing the UE to release the PSCell. Reception by the UE of the RRC message defines the start of T4.

During T4, the UE shall release the PSCell.

Table A.X.Y.Z.1.1-1: Supported test configurations for FR2-2 PSCell with CCA

| Configuration | Description |  |
| --- | --- | --- |
| 1 | NR TDD, SSB SCS 120 kHz, data SCS 120 kHz, BW 100 MHz | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR TDD, SSB SCS 480 kHz, data SCS 480 kHz, BW 400 MHz | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR TDD, SSB SCS 960 kHz, data SCS 960 kHz, BW 400 MHz | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Table A.X.Y.Z.1.1-2: General test parameters for PSCell addition and release delay

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1 in FR1 |
| Neighbour cell |  | Cell 2 | Neighbour cell (PSCell-to-be) on RF channel number 2 in FR2-2 with CCA |
| DRX |  | OFF | For both PCell and PSCell once activated |
| DL CCA model |  | TBD |  |
| UL CCA model |  | TBD |  |
| PRACH configuration in Cell 2 |  | FR2 PRACH configuration 2 | PRACH configuration as specified in Clause A.3.8.3.2. |
| CSI reporting periodicity and offset configuration for Cell 2 | ms | [2] |  |
| T1 | s | 5 | During this time the PCell is known and Cell 2 is unknown. |
| T2 | s | 1 | During this time the UE adds the PSCell. |
| T3 | s | 1 | During this time the UE sends CSI reports for PSCell. |
| T4 | s | 1 | During this time the UE releases the PSCell. |

Table A.X.Y.Z.1.1-3: NR Cell specific test parameters for PSCell addition and release delay

| Parameter | Unit | Config | Cell 1 | Cell2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 |
| Frequency Range |  | 1,2,3 | FR1 | FR2 |  |  |  |
| Duplex mode |  | 1 | FDD | TDD |  |  |  |
|  |  | 2,3 | TDD |  |  |  |  |
| TDD configuration |  | 1 | – | TBD |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  |  |
| BWchannel | MHz | 1 | 10: NRB,c = 52 | 100: NRB,c = 66 |  |  |  |
|  |  | 2 | 10: NRB,c = 52 | 400: NRB,c = 66 |  |  |  |
|  |  | 3 | 40: NRB,c = 106 | 400: NRB,c = 33 |  |  |  |
| Data RBs allocated |  | 1 | 52 | 66 66 |  |  |  |
|  |  | 2 | 52 |  |  |  |  |
|  |  | 3 | 106 | 33 |  |  |  |
| PCCA_DL |  | 1,2,3 | – | TBD |  |  |  |
| PCCA_UL |  | 1,2,3 | – | TBD |  |  |  |
| Initial Downlink BWP configuration |  | 1,2,3 | DLBWP.0.1 | DLBWP.0.1 |  |  |  |
| Initial Uplink BWP configuration |  | 1,2,3 | ULBWP.0.1 | ULBWP.0.1 |  |  |  |
| Dedicated Downlink BWP configuration |  | 1,2,3 | DLBWP.1.1 | DLBWP.1.1 |  |  |  |
| Dedicated Uplink BWP configuration |  | 1,2,3 | ULBWP.1.1 | ULBWP.1.1 |  |  |  |
| PDSCH Reference Measurement Channel |  | 1 | SR.1.1 FDD | SR.3.3 TDD |  |  |  |
|  |  | 2 | SR.1.1 TDD | TBD |  |  |  |
|  |  | 3 | SR.2.1 TDD | TBD |  |  |  |
| TRS configuration |  | 1 | – | TRS.2.1 TDD |  |  |  |
|  |  | 2 | – | TBD |  |  |  |
|  |  | 3 | – | TBD |  |  |  |
| TCI state |  | 1,2,3 | – | TCI.State.0 |  |  |  |
| RMSI CORESET parameters |  | 1 | CR.1.1 FDD | CR.3.2 TDD |  |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |  |
| Dedicated CORESET parameters |  | 1 | CCR.1.1 FDD | CCR.3.7 TDD |  |  |  |
|  |  | 2 | CCR.1.1 TDD | TBD |  |  |  |
|  |  | 3 | CCR.2.1 TDD | TBD |  |  |  |
| OCNG PatternsNote1 |  | 1,2,3 | OP.1 | OP.3 |  |  |  |
| SSB configuration |  | 1 | SSB.1 FR1 | SSB.2 FR2 |  |  |  |
|  |  | 2 | SSB.1 FR1 | TBD |  |  |  |
|  |  | 3 | SSB.2 FR1 | TBD |  |  |  |
| SMTC configuration |  | 1,2,3 | SMTC.2 | SMTC.1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1 | 15 | 120 |  |  |  |
|  |  | 2 | 15 | 480 |  |  |  |
|  |  | 3 | 30 | 960 |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2,3 | 0 | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |  |
| Propagation Condition |  | 1,2,3 | AWGN | AWGN |  |  |  |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |  |  |  |

Table A.X.Y.Z.1.1-4: OTA related test parameters for PSCell addition and release delay

| Parameter | Unit | Config | Cell 1 | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 |
| Angle of arrival configuration |  | 1,2,3 | Link only, see clause A.3.7A | Setup 2a according to clause A.3.15.2.1 |  |  |  |
| Assumption for UE beams Note 3 |  |  |  | Rough |  |  |  |
| SSB_RP Note1, Note 2 | dBm/SCS | 1 |  | -∞ | -95.7 |  |  |
|  |  | 2 |  | -∞ | -89.7 |  |  |
|  |  | 3 |  | -∞ | -86.7 |  |  |
| BB Note1, Note 4 | dB | 1,2,3 |  | -∞ | 7 |  |  |
| Io Note 1, Note 2 | dBm/95.04 MHz | 1 |  | N/A | -58.9 |  |  |
|  | dBm/380.16 MHz | 2,3 |  | N/A | -52.9 |  |  |
| Note 1: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone. Note 3: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 4: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |

##### A.X.Y.Z.1.2	Test Requirements

The UE shall transmit the PRACH preamble to PSCell at Tconfig_PSCell_CCA into T2.

The UE shall transmit at least one periodic CSI report for PSCell during T3.

The UE shall stop transmitting CSI reports for PSCell at latest 20 ms into T4.

Tconfig_PSCell_CCA is defined in clause 8.9B.2.

All of the above test requirements shall be fulfilled in order for the observed PSCell addition and release delay to be counted as correct. The rate of correct events observed during repeated tests shall be at least 90%.

< End of change 1>

< Start of change 2>

A.X.Y.Z.2	MAC-CE based active TCI state switch in FR2-2 with CCA

##### A.X.Y.Z.2.1	NR PCell FR2-2 active TCI state switch for a known TCI state

A.X.Y.Z.2.1.1	Test Purpose and Environment

The purpose of this test is to verify the active TCI state switch delay requirement defined in clause 8.10A.3. Supported test configuration is shown in Table A.X.Y.Z.2.1.1-1.

The test scenario comprises of one NR PCell (Cell 1) as given in Table A.X.Y.Z.2.1.1-2. Cell-specific parameters of NR PCell are specified in Table A.X.Y.Z.2.1.1-3 below. The OTA related test parameters for FR2-2 are shown in Table A.X.Y.Z.2.1.1-4.

PDCCHs indicating new transmissions shall be sent continuously on PCell to ensure that the UE would have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC).

-	UE is configured with 2 different TCI states for PCell, PDCCH TCI state 0 (QCL’d to SSB0) and TCIstate 1 (QCL’d to SSB1), in Cell 1 before starting the test.

-	UE is indicated in TCI state 0 as the active PDCCH TCI state

The test consists of two time periods, T1 and T2. Figure A.X.Y.Z.2.1.1-1 and Figure A.X.Y.Z.2.1.1-2 show the Time multiplexed (allocation in Frequency is symbolic) downlink transmissions from each Angle of Arrival. During T1 only SSB to which PDCCH-TCI-state0 is QCL’d is transmitted. At the beginning of T2, the SSB corresponding to TCI state 1 starts transmitting. The UE is configured to provide periodic L1-RSRP reports. In slot n which is within 1280ms of UE providing L1-RSRP report with results for both SSB0 and SSB1, UE receives a MAC-CE command indicating a switch to TCI state 1. tci-PresentInDCI is not configured in the PDSCH configuration, i.e. TCI state for the PDSCH is identical to the PDCCH TCI state.

The test equipment verifies that UE can be scheduled on PCell on TCI state 0 till n+ THARQ +3 ms. The test equipment also verifies the TCI state switch time in PCell by scheduling the UE on TCI state 1 after n+ THARQ +3 ms + (Tfirst-SSB + TSSB-proc + TSSB*LMAC,known).

Table A.X.Y.Z.2.1.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR 480 kHz SSB SCS, 400 MHz bandwidth, TDD duplex mode |
| 3 | NR 960 kHz SSB SCS, 400 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR2-2 |  |

Table A.X.Y.Z.2.1.1-2: General test parameters for TCI state switch

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DL CCA model |  | TBD |  |
| UL CCA model |  | TBD |  |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |

Table A.X.Y.Z.2.1.1-3: NR Cell specific test parameters for TCI state switch

| Parameter |  | Test config | Unit | Cell 1 |
| --- | --- | --- | --- | --- |
| Frequency Range |  | 1,2,3 |  | FR2-2 |
| Duplex mode |  | 1,2,3 |  | TDD |
| TDD configuration |  | 1,2,3 |  | TDDConf.3.1 |
| BWchannel |  | 1 |  | 100 MHz: NRB,c = 66 |
|  |  | 2 |  | 400 MHz: NRB,c = 66 |
|  |  | 3 |  | 400 MHz: NRB,c = 33 |
| Data RBs allocated |  | 1-2 |  | 66 |
|  |  | 3 |  | 33 |
| PCCA_DL |  | 1,2,3 |  | TDD |
| PCCA_UL |  | 1,2,3 |  | TDD |
| Initial DL BWP Configuration |  | 1,2,3 |  | DLBWP.0.2 |
| Dedicated DL BWP Configuration |  | 1,2,3 |  | DLBWP.1.1 |
| Initial UL BWP Configuration |  | 1,2,3 |  | ULBWP.0.2 |
| Dedicated UL BWP Configuration |  | 1,2,3 |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  | 1 |  | SR.3. 2 TDD |
|  |  | 2 |  | TBD |
|  |  | 3 |  | TBD |
| RMSI CORESET parameters |  | 1 |  | CR.3.1 TDD |
|  |  | 2 |  | TBD |
|  |  | 3 |  | TBD |
| Dedicated CORESET parameters |  | 1 |  | CCR.3.1 TDD |
|  |  | 2 |  | TBD |
|  |  | 3 |  | TBD |
| OCNG Patterns |  | 1,2,3 |  | OP. 5 |
| SSB Configuration |  | 1 |  | SSB.1 FR2 |
|  |  | 2 |  | TBD |
|  |  | 3 |  | TBD |
| SMTC Configuration |  | 1,2,3 |  | SMTC.1 |
| TCI State 0 |  | 1,2,3 |  | TCI.State.0 |
| TCI State 1 |  | 1,2,3 |  | TCI.State.1 |
| TRS Configuration |  | 1 |  | TRS.2.1 TDD |
|  |  | 2 |  | TBD |
|  |  | 3 |  | TBD |
| Correlation Matrix and Antenna Configuration |  | 1,2,3 |  | 1x2 Low |
| EPRE ratio of PSS to SSS |  | 1,2,3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Propagation Condition |  |  |  | AWGN |
|  | Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.X.Y.Z.2.1.1-4: OTA related test parameters for TCI state switch

| Parameter |  | Test | Unit | Cell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Config |  | SSB0 |  | SSB1 |  |  |
|  |  |  |  | T1 | T2 | T1 |  | T2 |
| Angle of arrival configuration |  | 1,2,3 |  | Setup 3 according to clause A.3.15.3 |  |  |  |  |
|  |  |  |  | AoA1 |  |  | AoA2 |  |
| Assumption for UE beams Note 6 |  | 1,2,3 |  | Rough |  |  |  |  |
| Ês |  | 1 | dBm/SCS | -80.6 | -80.6 | -Infinity |  | -80.6 |
|  |  | 2 |  | -74.6 | -74.6 | -Infinity |  | -74.6 |
|  |  | 3 |  | -71.6 | -71.6 | -Infinity |  | -71.6 |
| SS B_RP Note 2 |  | 1 | dBm/ SCS | -80.6 | -80.6 | -Infinity |  | -80.6 |
|  |  | 2 |  | -74.6 | -74.6 | -Infinity |  | -74.6 |
|  |  | 3 |  | -71.6 | -71.6 | -Infinity |  | -71.6 |
| BB Note 7 |  | 1,2,3 | dB | 8.3 | 8.3 | -Infinity |  | 8.3 |
| IoNote2 |  | 1 | dBm/95.04 MHz Note4 | -56.0 | -56.0 | - Infinity |  | -56.0 |
|  |  | 2,3 | dBm/380.16 MHz | -49,98 | -49,98 | - Infinity |  | -49,98 |
|  | Note 1: Void Note 2: SS B_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the center of the quiet zone. Note 6: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 7: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |

Figure A.X.Y.Z.2.1.1-1: Time multiplexed downlink transmissions during T1

Figure A.X.Y.Z.2.1.1-2: Time multiplexed downlink transmissions during T2

A.X.Y.Z.2.1.2	Test Requirements

During T2, UE shall send L1-RSRP report with results for both SSB0 and SSB1.

After receiving MAC-CE command in slot n, UE shall:

-	be able to continue to receive on TCI state 0 till   n+ THARQ +3 ms

-	be able to start receiving on TCI state 1 after n+ THARQ +5 ms + Tfirst-SSB+ TSSB*LMAC,known which is defined in clause 8.10A.3

< End of change 2>