3GPP TSG-RAN WG4 Meeting #105	R4-2220485

France Toulouse, 14-18 November 2022

Source:					Huawei, Hisilicon

Title: 	TP for TR38.894 on the simultaneous Rx/Tx band combinations for CA_n34A-n41A, CA_n40A-n41A, CA_n7A-n40A

Agenda item:			7.26.2

Document for:	Approval

1	Introduction

This contribution provides the text proposals for TR 38.894 on the simultaneous Rx/Tx band combinations for CA_n34A-n41A, CA_n39A-n41A, CA_n40A-n41A and CA_n7A-n40A.

***********************************Start of the TP*************************************

5	Specific Band combinations

5.1	CA_n34A-n41A

5.1.1	Status of the band combination

5.1.1.1	Operating bands for CA

The operating bands for CA_n34-n41 are specified in Table 5.1.1.1-1.

Table 5.1.1.1-1: Inter-band CA operating bands involving FR1

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n34-n419 | n34, n41 |  |
| NOTE 8: Applicable when dynamic Tx switching is conducted. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. NOTE 9: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. |  |  |

For CA_n34-n41, the current spec only considers scenarios without simultaneous Rx/Tx operation.

5.1.1.2	Power class of the BC

The UE Power Class for uplink CA_n34A-n41A are specified in Table 5.1.1.2-1. The supported power class of the band combination would have impact on the MSD, if any, based on the following study.

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n34A-n41A |  |  |  |  | 23 | +2/-3 |  |  |

Table 5.1.1.2-1 UE Power Class for uplink CA_n34A-n41A

5.1.1.3	∆TIB and ∆RIB

The ΔTIB,c due to CA_n34-n41 are specified in Table 5.1.1.3-1.

Table 5.1.1.3-1: ΔTIB,c due to CA_n34-n41

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n34-n41 | n34 | 0.3 |
|  | n41 | 0.3 |

ΔRIB,c due to CA_n34-n41 is set to zero.

It is noted that ΔTIB,c and ΔRIB,c reflect the possible UE architecture to support the CA band combination.

5.1.2	MSD analysis

TBD

5.1.3	Requirements for simultaneous Rx/Tx

TBD

5.3	CA_n40A-n41A

5.3.1	Status of the band combination

5.3.1.1	Operating bands for CA

The operating bands for CA_n40-n41 are specified in Table 5.3.1.1-1.

Table 5.3.1.1-1: Inter-band CA operating bands involving FR1

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n40-n41 | n40, n41 | No |
| NOTE 8: Applicable when dynamic Tx switching is conducted. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. |  |  |

For CA_n40-n41, the current spec only considers scenarios without simultaneous Rx/Tx, which are limited by the applicable requirements of ΔTIB,c and ΔRIB,c.5.3.1.2	Power class of the BC

5.1.1.2	Power class of the BC

The UE Power Class for uplink CA_n40A-n41A are specified in Table 5.3.1.2-2. The supported power class of the band combination would have impact on the MSD, if any, based on the following study.

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n40A-n41A |  |  | 266,7 | +2/-3 | 23 | +2/-3 |  |  |
| NOTE 6: The UE supports PC3 within NR FDD band, and supports either PC3 or PC2 within NR TDD band. NOTE 7: The UE that supports PC3 within an NR TDD or FDD band and supports PC2 within a second NR TDD band may signal a [HigherPowerLimitCADC] capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2A.4.1.3. The power classes referenced are according to the reported [powerClassPerBand] if indicated or ue-PowerClass otherwise. |  |  |  |  |  |  |  |  |

Table 5.3.1.2-1 UE Power Class for uplink CA_n40A-n41A

5.3.1.3	∆TIB and ∆RIB

The ΔTIB,c due to CA_n40-n41 are specified in Table 5.3.1.3-1.

Table 5.3.1.3-1: ΔTIB,c due to CA_n40-n41

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n40-n41 | n40 | 0.53 |
|  | n41 | 0.53 |
| NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. |  |  |

ΔRIB,c due to CA_n40-n41 is set to zero.

It is noted that ΔTIB,c and ΔRIB,c reflect the possible UE architecture to support the CA band combination.

5.3.2	MSD analysis for simultaneous Rx/Tx

TBD

5.3.3	Requirements for simultaneous Rx/Tx

For ΔTIB,c due to CA_n40-n41, 0.5 is applied to UE supporting inter-band carrier aggregation with simultaneous Rx/Tx.

Other requirements are FFS.

5.4	CA_n7A-n40A

5.4.1	Status of the band combination

5.4.1.1	Operating bands for CA

The operating bands for CA_n7-n40 are specified in Table 5.4.1.1-1.

Table 5.4.1.1-1: Inter-band CA operating bands involving FR1

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n7-n40 | n7, n40 |  |
| NOTE 8: Applicable when dynamic Tx switching is conducted. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. |  |  |

For CA_n7-n40, the current spec has no limitation on scenarios with or without simultaneous Rx/Tx operation.

5.4.1.2	Power class of the BC

The UE Power Class for uplink CA_n7A-n40A are specified in Table 5.4.1-3. The supported power class of the band combination would have impact on the MSD, if any, based on the following study.

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n7A-n40A |  |  |  |  | 23 | +2/-3 |  |  |

Table 5.4.1.2-1 UE Power Class for uplink CA_n7A-n40A

5.4.1.3	∆TIB and ∆RIB

The ΔTIB,c due to CA_n7-n40 are specified in Table 5.4.1-4.

Table 5.4.1.3-1: ΔTIB,c due to CA_n7-n40

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n7-n40 | n7 | 0.5 |
|  | n40 | 0.6 |

The ΔRIB,c due to CA_n7-n40 are specified in Table 5.4.1-5.

Table 5.4.1.3-2: ΔRIB,c due to CA_n7-n40

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n7-n40 | n40 | 0.5 |

For inter-band carrier aggregation with uplink assigned to n7 and n40 given in Table 5.4.1-6 the reference sensitivity is defined only for the specific uplink and downlink test points specified in Table 5.4.1-6.

It is noted that ΔTIB,c and ΔRIB,c reflect the possible UE architecture to support the CA band combination.

5.4.1.4	Existing MSD requirement

MSD values due to crossband isolation specified in Rel-17 are in brackets, which should be further studied.

Table 5.4.1.4-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n7 | 2350 | 100 | 30 | 270 (RBstart=3) | 2622.5 | 5 | [21.9] | >ACLR2 |
| n40 | n7 | 2350 | 100 | 30 | 270 (RBstart=3) | 2645 | 50 | [13.5] | >ACLR2 |

5.2.2	MSD analysis

TBD

5.2.3	Requirements for simultaneous Rx/Tx

TBD

***********************************End of the TP*************************************

Reference

[1] R4-2216659	TR skeleton for TR 38.xxx Simultaneous RxTx inter-band combination in Rel-18

[2] R4-2215899,  RF requirements on CA_n39-n41 supporting simultaneous Rx/Tx, ZTE

[3] R4-2217770, Email discussion summary for [104-bis-e][117] LTE_NR_Other_WI, Moderator, Huawei

[4] R4-2220097, Topic summary for [105][117] LTE_NR_Other_WI,	Moderator, Huawei, HiSilicon

[5] R4-2218106, On simultaneous Rx-Tx for NR inter-band combinations, Apple

[6] R4-2218925, Discussion simultaneous Rx/Tx inter-band combinations for CA_n40-n41, MediaTek Inc.