3GPP TSG-RAN WG4 Meeting #105	R4-2220501

Toulouse, FR, November 14th ‒ 18th, 2022

Title: 	WF on UE RF for 5G Broadcast

Agenda Item:	9.4.5

Source: 	Qualcomm Incorporated

Document for:	Approval

# Band plan

Two options (not mutually exclusive)

- Define a single band covering the entire UHF frequency range 470 – [698/702] MHz

- The requirements are relaxed for this band (TBD which requirements and how much) and/or

- This band is assumed to be implemented with more than one filter in the UE (TBD how many filters and the characteristics of those filters)

- Define smaller bands

- At least one of the bands overlaps with Band n105 DL to reuse n105 Rx filter

- Specify the following bands 470-542 MHz, 540-606 MHz and 602-702 MHz.

- Other

Way forward:  For RAN4 #106, companies are invited to provide proposals and supporting technical data including filter performance for the options listed above.

# Reference sensitivity

Option 1 (ZTE):  to reuse the noise figure of legacy DVB-T receiver design and other implementation margin

Option 2 (Huawei, Nokia):  -100dBm/-97dBm/ -95.2dBm /-94dBm REFSENS can be derived for 5, 10, 15, 20MHz

Moderator comment:  Using the same formula as Huawei’s paper, but setting the bandwidth to 6, 7, and 8 MHz we have -99.2, -98.5, and -97.9 dBm.

Moderator comment:  Nokia suggests using SDO band as basis for refsens.  According to the moderator, this should be the same values as proposed by Huawei.

Way forward:  Reference sensitivity values of -99.2, -98.5, -97.9, and -97 dBm are agreed as the baseline for 6, 7, 8, and 10 MHz channels. It is FFS which bandwidth will be specified.

# ACS

Option 1 (Nokia): -25 dB with frequency offset at 6, 7, and 8 MHz

Option 2 (Qualcomm): -33 dB with frequency offset at 10 MHz (same as the existing 10 MHz spec)

Option 3 (SWR):  -33 dB with frequency offset at 6, 7, and 8 MHz

Way forward:  Companies are invited to study options 2 and 3.