3GPP TSG-RAN WG4 Meeting # 105												      R4-2220517

Toulouse, France, November 14th – November 18th, 2022

Agenda item:			8.7.4

Source:	Nokia

Title:	WF on beam correspondence requirements for initial access and RRC_inactive mode

Document for:	Approval

# Topic #1: Beam refinement assumption

## Sub-topic 1-1 Rough beam vs Fine beam

Issue 1-1: Rough beam vs Fine beam

- Proposals

  - Option 1: Beam refinement in RRC_INACTIVE and initial access is made in the same way as RRC_CONNECTED.

    - Option 1a: with the same SSB configuration as Rel-16 SSB BC (RRC_CONNECTED) case.

    - Option 1b: with some modification in SSB configuration.

  - Option 2: It is allowed not to refine beams in RRC_INACTIVE and initial access.

    - Option 2a: It is allowed to use only one antenna element.

    - Option 2b: Beam gain is 7 dB lower than RRC_CONNECTED.

      - The Msg1 EIRP spherical coverage requirement for PC3 is [7+xdB] higher than the EIRP spherical coverage requirement specified in 6.2.1 for connected mode.

  - Option 3: Somewhere in the middle.

    - Option 3a: Refinement is done but is not as good as RRC_CONNECTED.

    - Option 3b: Refinement is done in CG-SDT but is not in RA-SDT and initial access.

    - Option 3c: Refinement in DRX is not as good as continuous reception.

  - Option 4: Rough beam or Fine beam used in IA is up to UE implementation and requirements should be implementation agnostic.

Way forward/Agreements:

- Focus on BC requirements first.

# Topic #2:  BC requirement

## Sub-topic 2-1 Minimum peak EIRP and spherical coverage

Issue 2-1-1: minimum peak EIRP

- WF

  - Option 1: min peak EIRP is included.

    - Option 1a: EIRP is the same as RRC_CONNECTED

    - Option 1b: EIRP is lower in initial access and RRC_INACTIVE

  - Option 2: min peak EIRP is not included.

Issue 2-1-2: EIRP spherical coverage requirement for msg1

Way forward/Agreements:

- Agree that spherical coverage %-tile for PC3 is the same as connected, i.e., 50%-tile.

  - Discuss if Min EIRP at 50% is the same level as connected or relaxed from connected.

## Sub-topic 2-2 RAR

Issue 2-2: RAR

- Proposal

  - Option 1: RAR is included.

  - Option 2: RAR is not included.

- WF

  - Focus on msg1 requirement first. Then, discuss RAR later

## Sub-topic 2-3 msg3

Issue 2-3: msg3

- Proposals

  - Option 1: msg3 is included.

  - Option 2: msg3 is not included.

- WF

  - Option 2

## Sub-topic 2-4 msgA

Issue 2-4: msgA

- Proposals

  - Option 1: msgA is included.

    - Option 1a: msgA requirement is the same as msg1.

    - Option 1b: msgA requirement is the same as Rel-16.

  - Option 2: msgA is not included

- WF

  - Focus on msg1 requirement first. Then, discuss msgA later

## Sub-topic 2-5 applicability of Rel-16 SSB BC requirement

Issue 2-5: applicability of Rel-16 SSB BC requirement

- Proposals

  - Option 1: Applicable to IA, RA-SDT, and CG-SDT.

  - Option 2: Not applicable.

- WF

  - No further discussion. Discuss each requirement individually if it is agreed to specify.

## Sub-topic 2-6 requirement scenario (IA, RA-SDT, CG-SDT)

Issue 2-6: requirement scenario

- WF

  - Option 1: Core requirement is introduced to all cases, i.e., IA, RA-SDT, and CG-SDT

    - Option 1a: Core requirement is the same for all cases and one set of requirements is appliable to all.

    - Option 1b: Core requirement is specified for each case, IA, RA-SDT and CG-SDT.

  - Option 2: Core requirement is only introduced to initial access.

## Void

## Sub-topic 2-8 BC tolerance

Issue 2-8: BC tolerance

- Proposal

  - Option 1: BC tolerance is applicable.

    - Option 1a: The same as Rel-16.

    - Option 1b: New tolerance is introduced.

      - Option 1b-1: New tolerance for long/short DRX scenarios needs to be clear.

      - Option 1b-2: a beam correspondence tolerance X dB can be defined for IA, and the tolerance is applicable

  - Option 2: BC tolerance is not applicable.

- WF

  - Focus on msg1 requirement first. Then, discuss whether BC tolerance is needed later

## Sub-topic 2-9 UE capability

- WF

  - No new UE capability for beam correspondence is introduced.

## Sub-topic 2-10 side conditions

- WF

  - FFS: further study the side condition.

# Topic #3: Test Issues

## Sub-topic 3-1 Feasibility to achieve maximum output power

Issue 3-1-1: BC can be verified with well-defined parameters already available from legacy releases.

- WF

  - Option 1: Feasible by holding RAR.

  - Option 2: Feasible already from the first preamble.

  - Option 3:  The detail of how to enable the UE to transmit at max. output power is left to RAN5.

Issue 3-1-2: whether new test functionality is needed?

- WF

  - Option 1: Introduce a beam lock function to RRC_INACTIVE and initial access.

    - Option 1a: Ask RAN5 the feasibility of beam lock function

  - Option 2: Introduced a new RA response timer.

  - Option 3: If a beam lock function is not feasible, consider how to guarantee zero P-MPR during tests for BC requirements for RRC_INACTIVE and initial access for Issue 3-1-2

  - Option 4: No new test functionality is needed

## Sub-topic 3-2 Test scenario

Issue 3-2: Test scenario

- WF

  - Option 1: A new test scenario with short/long DRX to trigger SDT mode in RRC_INACTIVE is introduced

  - Option 2: A new test scenario with short/long DRX to trigger SDT mode in RRC_INACTIVE is not introduced

## Sub-topic 3-3 Polarization aspects

Issue 3-3: Polarization aspects

- WF

  - Option 1: Testability limitation on polarization aspect shall be addressed

    - Option 1a: EIRP compensation according to R4-2218559

    - Option 1b: Separate communication and measurement antenna

  - Option 2: No need or other solutions

## Sub-topic 3-4 PRACH requirement verification

- WF

  - FFS: further study the PRACH requirement related to beamlock in Rel-15