3GPP RAN WG4 Meeting #105		R4-2220558

Toulouse, France, November 14th – November 18th, 2022

Title:	WF on missing Scell MSD for intra-band contiguous EN-DC

Agenda item:	7.1.1

Source:	Skyworks Solutions Inc., (…)

Document for:	Approval

# 1	Background

- At RAN4 #104-e, TPs for DC_3-(n)7 [1] and DC_(n)3AA-n8A [2] were flagged due to missing MSD requirements to account for dual uplink IMD4 and IMD5 products respectively, affecting both the NR DL Pcell and the LTE DL Scell of a DL band that is configured for intra-band contiguous EN-DC operation.

- At RAN4 #104-bis-e,

  - MSD evaluation for DC_3A-(n)7AA and DC_(n)3AA-n8A was presented [3,4]

  - a Way Forward was agreed which provided guidelines on how to evaluate and how to capture the MSD test point in TS 38.101-3 Table 7.3B.2.3.5.2-1 [5].

- At RAN4 #105, MSD test points have been proposed for DC_66A-(n)5AA, DC_2A-(n)66AA, DC_48A-(n)12AA and DC_8A-(n)3AA [6].

- This WF summarizes the agreed MSD test points that need to be ported in the following Releases:

  - DC_48A-(n)12AA was introduced in Rel-16, the MSD test point needs to be ported to Rel-16,17,18.

  - DC_2A-(n)66AA, DC_8A-(n)3AA, DC_66A-(n)5AA were introduced in Rel-17, the MSD test points need to be ported to Rel-17,18.

  - DC_3A-(n)7AA, and DC_(n)3AA-n8A were requested in Rel-18. The MSD test point for DC_3A-(n)7AA needs to be ported in Rel-18. For DC_(n)3AA-n8A, the EN-DC MSD requirements from DC_8A-(n)3AA apply.

# Way forward

## 2.1	WF on DC_48A-(n)12AA MSD Test Point

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_48A-(n)12AA | 48 | 3557.5 | 10 | 50 | 3557.5 | N/A | N/A |
|  | 12 | N/A | 5 | N/A | 740.5 | 5.5 | IMD5 |
|  | n12 | 705.5 | 5 | 25 | 735.5 | 5.5 | IMD5 |

Note: This test point should be implemented in Release 16,17,18.

## 2.2	WF on DC_2A-(n)66AA MSD Test Point

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-(n)66AA | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | 66 | N/A | 5 | N/A | 2145 | 2.8 | IMD5 |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |

Note: This test point should be implemented in Release 17,18.

## 2.3	WF on DC_8A-(n)3AA MSD Test Point

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8A-(n)3AA | 8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  | 3 | N/A | 5 | N/A | 1835 | 4.5 | IMD5 |
|  | n3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | IMD5 |

Note: This test point should be implemented in Release 17,18.

## 2.4	WF on DC_66A-(n)5AA MSD Test Point

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A-(n)5AA | 66 | 1721 | 5 | 25 | 2121 | N/A | N/A |
|  | 5 | N/A | 5 | N/A | 878 | 25 | IMD2 |
|  | n5 | 838 | 5 | 25 | 883 | 30 | IMD2 |

Note: This test point should be implemented in Release 17,18.

## 2.5	WF on DC_3A-(n)7AA MSD Test Point

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-(n)7AA DC_3C-(n)7AA | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | 7 | N/A | 5 | N/A | 2647.5 | 6.9 | IMD4 |
|  | n7 | 2535 | 10 | 50 | 2655 | 10.2 | IMD4 |

Note: This test point should be implemented in Release 18.

## 2.6	WF on DC_(n)3AA-n8A MSD Test Point

The EN-DC MSD requirements from DC_8A-(n)3AA apply to DC_(n)3AA-n8A.

# 3	References

R4-2213094 “TP for TR 37.718-21-11: Including band combinations DC_3-(n)7”, 3GPP TSG-RAN WG4 Meeting #104-e, August 15th – 26th, 2022, Ericsson, Telstra.

R4-2213677 “TP for TR 37.718-11-21: DC_(n)3AA_n8A”, 3GPP TSG-RAN WG4 Meeting #104-e, August 15th – 26th, 2022, Huawei, HiSilicon.

R4-2215523 “MSD for DC_3A-(n)7AA”, 3GPP TSG-RAN WG4 Meeting #104-bis-e, October 10th – 19th, 2022, Skyworks Solutions, Inc.

R4-2215521 “MSD for DC_ (n)3AA-n8A”, 3GPP TSG-RAN WG4 Meeting #104-bis-e, October 10th – 19th, 2022, Skyworks Solutions, Inc.

R4-2217097 “Way forward on MSD to Scell for intra-band contiguous with IMD overlap”, 3GPP RAN WG4 Meeting #104-bis-e, Skyworks Solutions Inc., Murata Manufacturing Co Ltd.

R4-2219703 “Missing Three-band MSD to Scell for intra-band contiguous with IMD”, 3GPP TSG-RAN WG4 Meeting #105, Skyworks Solutions, Inc.