3GPP TSG-RAN4 Meeting #105	R4-2220572

# Toulouse, France, November 14th – 18th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | 0011 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.101-5 on 64QAM requirements related corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2022-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During the RAN4 #101bis-e meeting an agreement was reached to “Include 64QAM (DL and UL) for NTN satellite UE as optional feature with granularity [per UE]” (R4-2203036). Since the NTN UE RF specifications (38.101-5) refer to the terrestrial UE RF specificatoins (38.101-1) for MPR, transmit modulation quality, and max input level requirements, it is necessary to reflect the optionality of 64QAM in the specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify the following 64QAM related requirements: - Explicitly list which modulations are applicable for MPR in clause 6.2.2. - Explicitly list which modulations are applicable to the NTN transmission modulation quality requirement in clause 6.4.2. - Make clear the optional applicability of the 64 QAM FRC to the maximum input level requirement and introduce a placeholder reference to the 16 QAM FRC. - Introduce new FRC tables for 16QAM in Annex A. - Remove NOTE 3 in Table 7.4-1 as there is no derived power level which needs to be rounded. - Make NOTE 2 as a general note as there is no differentiation in maximum input level requirement under different modulations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The modulation applicability of the transmit modulation quality requirement is not aligned with the NTN UE modulation capabilities. - The max input level RMC is not aligned with the NTN UE modulation capabilities. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2, 7.4, A.x.y.z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changed sections >>>

### 6.2.2	UE maximum output power reduction

UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations. For UE power class 3, the allowed maximum power reduction (MPR) is defined as Table 6.2.2-1 in 3GPP TS 38.101-1[5] clause 6.2.2 for the modulations of QPSK, 16 QAM, and if supported by the UE, 64 QAM.

<<< Unchanged sections omitted >>>

### 6.4.2	Transmit modulation quality

The requirements for transmit modulation quality defined in 3GPP TS 38.101-1 [5] clause 6.4.2 shall apply for NTN satellite UE for the modulations of QPSK, 16 QAM, and if supported by the UE, 64 QAM.

<<< Unchanged sections omitted >>>

## 7.4	Maximum input level

Maximum input level is defined as the maximum mean power received at the UE antenna port, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel. The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in 3GPP TS 38.101-1 [5] Annexes A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in 3GPP TS 38.101-1 [5] Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.4-1.

Table 7.4-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 5, 10, 15, 20 |
| Power in Transmission Bandwidth Configuration | dBm | -40 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-2 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 in TS 38.101-1 [5] for 64 QAM if supported by the UE or A.x.y.z for 16 QAM otherwise. NOTE 3: Void |  |  |

<<< Unchanged sections omitted >>>

A.x.y.z FRC for maximum input level for 16QAM

Table A.x.y.z-1 Fixed reference channel for maximum input level receiver requirements (SCS 15 kHz, FDD, 16QAM)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration μ |  | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 25 | 52 | 79 | 106 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 8 | 8 | 8 | 8 |
| MCS Index |  | 13 | 13 | 13 | 13 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |
| Modulation |  | 16 QAM | 16 QAM | 16 QAM | 16 QAM |
| Target Coding Rate |  | 1/2 | 1/2 | 1/2 | 1/2 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0,1 | Bits | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 5120 | 10760 | 16392 | 22032 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0,1 | CBs | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | CBs | 2 | 4 | 5 | 7 |
| Binary Channel Bits per Slot |  |  |  |  |  |
| For Slots 0,1 | Bits | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 10800 | 22464 | 34128 | 45792 |
| Max. Throughput averaged over 1 frame | Mbps | 4.096 | 8.608 | 13.114 | 17.626 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1 of TS38.101-1 [5]. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |

Table A.x.y.z-2 Fixed reference channel for maximum input level receiver requirements (SCS 30 kHz, FDD, 16QAM)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration μ |  | 1 | 1 | 1 | 1 |
| Allocated resource blocks |  | 11 | 24 | 38 | 51 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 17 | 17 | 17 | 17 |
| MCS Index |  | 13 | 13 | 13 | 13 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |
| Modulation |  | 16 QAM | 16 QAM | 16 QAM | 16 QAM |
| Target Coding Rate |  | 1/2 | 1/2 | 1/2 | 1/2 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0,1,2 | Bits | N/A | N/A | N/A | N/A |
| For Slots 3,…,19 | Bits | 2088 | 4992 | 7808 | 10504 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0,1,2 | CBs | N/A | N/A | N/A | N/A |
| For Slots 3,…,19 | CBs | 1 | 2 | 3 | 3 |
| Binary Channel Bits per Slot |  |  |  |  |  |
| For Slots 0,1,2 | Bits | N/A | N/A | N/A | N/A |
| For Slots 3,…,19 | Bits | 4752 | 10368 | 16416 | 22032 |
| Max. Throughput averaged over 1 frame | Mbps | 3.550 | 8.486 | 13.274 | 17.857 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1 of TS38.101-1 [5]. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |

Table A.x.y.z-3 Fixed reference channel for maximum input level receiver requirements (SCS 60 kHz, FDD, 16QAM)

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 15 | 20 |
| Subcarrier spacing | kHz | 15 | 15 | 15 |
| Subcarrier spacing configuration μ |  | 1 | 1 | 1 |
| Allocated resource blocks |  | 11 | 18 | 24 |
| Subcarriers per resource block |  | 12 | 12 | 12 |
| Allocated slots per Frame |  | 36 | 36 | 36 |
| MCS Index |  | 13 | 13 | 13 |
| MCS Table for TBS determination | 64QAM |  |  |  |
| Modulation |  | 16 QAM | 16 QAM | 16 QAM |
| Target Coding Rate |  | 1/2 | 1/2 | 1/2 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |
| For Slots 0,1,2,3 | Bits | N/A | N/A | N/A |
| For Slots 4,…,39 | Bits | 2088 | 3624 | 4992 |
| Transport block CRC | Bits | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |
| For Slots 0,1,2,3 | CBs | N/A | N/A | N/A |
| For Slots 4,…,39 | CBs | 1 | 2 | 2 |
| Binary Channel Bits per Slot |  |  |  |  |
| For Slots 0,1,2,3 | Bits | N/A | N/A | N/A |
| For Slots 4,…,39 | Bits | 4752 | 7776 | 10368 |
| Max. Throughput averaged over 1 frame | Mbps | 7.517 | 13.046 | 17.971 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1 of TS38.101-1 [5]. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |

<<< End of changed sections >>>