3GPP TSG-RAN WG4 Meeting #106	R4-2300036

Athens, Greece, 27th February – 3rd March 2023

Agenda item:	9.27.2

Source: 		NTT DOCOMO, INC.

Title: 	Discussion on Rx spurious emissions requirement for NCR

Document for:	Approval

- Introduction

In the RAN4 meeting #104bis-e, the discussion of NCR RF requirements was conducted. NCR RF requirements can be divided into two parts, NCR-MT and NCR-Fwd. In last meeting RAN4#105, NCR-Fwd RF requirements baseline for NCR type 1-H and type 1-O were proposed by [1], but no conclusion has been reached.  In this contribution, we provide our views on RF requirements for NCR.

- Discussion

In RAN4#105, the starting point of NCR-Fwd requirements was proposed by [1] as below, further discussion was encouraged.

RAN4#105 [1] Proposal 9: for RF requirement for NCR Fwd link type 1-H and type 1-O,use the following table 2.5.1 as starting point for further discussion. Table 2.5.1. summary of the proposals for NCR-Fwd link type 1-H and type 1-O RF requirements; Applicability notes. Repeater output power; ### For NCR type 1-H: ### Minimum requirement for IAB type 1-H in section 6.2.2 of TS 38.174 could be applied. ### For NCR type 1-H and 1-O: ### Minimum requirement for IAB type 1-H and 1-O an in section 9.2.2 of TS 38.174 could be applied.. TRP output power; ### For NCR type 1-O: ### Minimum requirement for IAB type 1-H and 1-O an in section 9.3.2 of TS 38.174 could be applied.. Frequency stability; Reuse the type 1-C requirements and no additional requirement are expected;. Out of band gain; Reuse the type 1-C requirements and no additional requirement are expected;. Adjacent Channel Leakage Power Ratio; Reuse the type 1-C requirements and no additional requirement are expected;. Operating band unwanted emissions; For NCR type 1-H: The following scaling factor X = 10log10(NRXU,countedpercell) should be considered; For NCR type 1-O: The following scaling factor X should be considered, whether X is equal to 9dB or other value, this could be further discussed.. Transmitter spurious emissions; For NCR type 1-H: The following scaling factor X = 10log10(NRXU,countedpercell) should be considered; For NCR type 1-O: The following scaling factor X should be considered, whether X is equal to 9dB or other value, this could be further discussed.. Receiver spurious emissions; For NCR type 1-H: The following scaling factor X = 10log10(NRXU,countedpercell) should be considered; For NCR type 1-O: The following scaling factor X should be considered, whether X is equal to 9dB or other value, this could be further discussed.. Error Vector Magnitude; Reuse the type 1-C requirements and no additional requirement are expected;. Input intermodulation; ### For NCR type 1-H:. To reuse the type 1-C requirement for each TAB connector of NCR type 1-H; For NCR type 1-O:. Conducted requirement should be updated as RMS field-strength (V/m) for OTA conformance testing.. Output intermodulation; ### For NCR type 1-H:. To reuse the type 1-C requirement for each TAB connector of NCR type 1-H; For NCR type 1-O:. To reuse minimum requirement for BS type 1-O in section 9.8.2 of TS 38.104. Wanted signal should be updated to align with conducted part.. Adjacent Channel Rejection Ratio (ACRR); Reuse the type 1-C requirements and no additional requirement are expected; For NCR type 1-O, similar as FR2 NCR, some description of interfering signal outside pass-band and wanted signal within the pass-band should be added. Transmit ON/OFF power; ### For NCR type 1-H:. BS OFF power requirement specified in 6.4.1.3 in TS 38.104 could be reused; For NCR type 1-O:. BS OFF power requirement specified in section 9.5.2.2 in TS 38.104 could be reused. For transition period related requirement, repeater type 1-C requirement could be reused..

As state above table, the baseline of RF requirements for NCR 1-H and 1-O were proposed, and each requirements are reused from conventional requirements for BS, IAB and NR repeater. We consider this is agreeable as starting point for further consideration of the NCR-Fwd type 1-H and 1-O requirements. However, based on the discussion in Rel-17 NR repeater, we would like to clarify the applicability of Rx spurious emissions requirements for NCR type 1-O.

In the RAN4#98bis-e meeting [2], Rx spurious emissions for repeater type 2-O was not specified because UL Rx emissions cannot be separated from DL Tx emissions and vice versa in OTA measurements.

Observation 1: RAN4 concluded Rx spurious emissions limits were not necessary for repeater type 2-O in Rel-17.

Regarding NCR type 1-O, the test condition is the same with NR repeater type 2-O since both measurements would be conducted in OTA system. Therefore, the same reason for not defining Rx spurious emissions requirements for repeater type 2-O would be applicable to NCR Fwd type 1-O.

In terms of NCR type 1-H, it is reasonable to reuse the same requirements with BS and/or IAB.

Proposal 1: The specific requirement of Rx spurious emission is not necessary for NCR Fwd link type 1-O.

- Conclusion

In this contribution, we provide views on applicability of Rx spurious emissions for NCR-Fwd type 1-H and 1-O. The following an observation and a proposal are obtained.

Observation 1: RAN4 concluded Rx spurious emission was not necessary for type 2-O repeater in Rel-17.

Proposal 1: For RF requirement for NCR Fwd link type 1-O, the specific requirement of Rx spurious emission is not necessary.

References

- R4-2219375, “Further discussion on RF requirements for NCR”, ZTE, RAN4#105, November 2022

- R4-2106112, “WF for NR repeater RF requirements”, CMCC, RAN4#98bis-e, April 2021