3GPP TSG-RAN4 Meeting #106	R4-2300077

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.104 | CR | 0976 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR related to Introduction of support of NB1/NB2 for LTE TDD Band 54 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ligado Networks, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_bands_R18_M1_M2_NB1_NB2-Core |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adding Band 54 for NB1/NB2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Relevant clauses have been updated to introduce support for NB-IoT for Band 54 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NB-IoT for Band 54 will not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.141 CR 1032 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.5	Operating bands and Band Categories

MSR requirements are applicable for band definitions and band numbering as defined in the specifications TS 45.005 [5], TS25.104 [2], TS 25.105 [3], TS 36.104 [4] and TS 38.104 [17]. For the purpose of defining the BS requirements, the operating bands are divided into three band categories as follows:

-	Band Category 1 (BC1): Bands for NR FDD, E-UTRA FDD and/or UTRA FDD operation. Bands in this category are also used for NB-IoT operation (all modes)

-	Band Category 2 (BC2): Bands for NR FDD, E-UTRA FDD, UTRA FDD and/or GSM/EDGE operation. Bands in this category are also used for NB-IoT operation (all modes)

-	Band Category 3 (BC3): Bands for NR TDD, E-UTRA TDD and/or UTRA TDD operation. Bands in this category are also used for NB-IoT operation (all modes)

NOTE:	For UTRA TDD, requirements in the present document cover the 1.28 Mcps UTRA TDD option.

The paired and unpaired bands for the three Band Categories are shown in Table 4.5-1 and 4.5-2, together with the supported RATs and corresponding NR, E-UTRA, UTRA and GSM/EDGE band designations.

Table 4.5-1: Paired bands in NR, E-UTRA, UTRA and GSM/EDGE.

| MSR Band number | Supported RATs and Band Numbers |  |  |  |  | Uplink (UL) BS receive, UE transmit (MHz) | Downlink (DL) BS transmit, UE receive (MHz) | BC | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NR | E-UTRA | NB-IoT | UTRA | GSM/EDGE |  |  |  |  |
| 1 | n1 | 1 | X | I | - | 1920 – 1980 | 2110 – 2170 | 1 |  |
| 2 | n2 | 2 | X | II | PCS 1900 | 1850 – 1910 | 1930 – 1990 | 2 |  |
| 3 | n3 | 3 | X | III | DCS 1800 | 1710 – 1785 | 1805 – 1880 | 2 |  |
| 4 | - | 4 | X | IV | - | 1710 – 1755 | 2110 – 2155 | 1 |  |
| 5 | n5 | 5 | X | V | GSM 850 | 824 – 849 | 869 – 894 | 2 |  |
| 6 | - | - | - | VI | - | 830 – 840 | 875 – 885 | 1 |  |
| 7 | n7 | 7 | X | VII | - | 2500 – 2570 | 2620 – 2690 | 1 |  |
| 8 | n8 | 8 | X | VIII | E-GSM | 880 – 915 | 925 – 960 | 2 |  |
| 9 | - | 9 | - | IX | - | 1749.9 – 1784.9 | 1844.9 – 1879.9 | 1 |  |
| 10 | - | 10 | - | X | - | 1710 – 1770 | 2110 – 2170 | 1 |  |
| 11 | - | 11 | X | XI | - | 1427.9 – 1447.9 | 1475.9 – 1495.9 | 1 |  |
| 12 | n12 | 12 | X | XII | - | 699 – 716 | 729 – 746 | 1 |  |
| 13 | n13 | 13 | X | XIII | - | 777 – 787 | 746 – 756 | 1 |  |
| 14 | n14 | 14 | X | XIV | - | 788 – 798 | 758 – 768 | 1 |  |
| 15 | - | - | - | - | - | Reserved |  |  |  |
| 16 | - | - | - | - | - | Reserved |  |  |  |
| 17 | - | 17 | X | - | - | 704 – 716 | 734 – 746 | 1 |  |
| 18 | n18 | 18 | X | - | - | 815 – 830 | 860 – 875 | 1 |  |
| 19 | - | 19 | X | XIX | - | 830 – 845 | 875 – 890 | 1 |  |
| 20 | n20 | 20 | X | XX | - | 832 – 862 | 791 – 821 | 1 |  |
| 21 | - | 21 | X | XXI | - | 1447.9 – 1462.9 | 1495.9 – 1510.9 | 1 |  |
| 22 | - | 22 | - | XXII | - | 3410 – 3490 | 3510 – 3590 | 1 |  |
| 23 | - | 23 | - | - | - | 2000 – 2020 | 2180 – 2200 | 1 | Note 4 |
| 24 | n24 | 24 | X | - | - | 1626.5 – 1660.5 | 1525 – 1559 | 1 | Note 6 |
| 25 | n25 | 25 | X | XXV | - | 1850 – 1915 | 1930 – 1995 | 1 |  |
| 26 | n26 | 26 | X | XXVI | - | 814 – 849 | 859 – 894 | 1 |  |
| 27 | - | 27 | - | - | - | 807 – 824 | 852 – 869 | 1 |  |
| 28 | n28 | 28 | X | - | - | 703 – 748 | 758 – 803 | 1 |  |
| 29 | n29 | 29 | - | - | - | N/A | 717 – 728 | 1 | Note 1 |
| 30 | n30 | 30 | - | - | - | 2305 – 2315 | 2350 – 2360 | 1 |  |
| 31 | - | 31 | X | - | - | 452.5 – 457.5 | 462.5 – 467.5 | 1 |  |
| 32 | - | 32 | - | XXXII | - | N/A | 1452 – 1496 | 1 | Note1, Note 2 |
| 64 | - | - | - | - | - | Reserved |  |  |  |
| 65 | n65 | 65 | X | - | - | 1920 – 2010 | 2110 – 2200 | 1 |  |
| 66 | n66 | 66 | X | - | - | 1710 – 1780 | 2110 – 2200 | 1 | Note 7 |
| 67 | n67 | 67 | - | - | - | N/A | 738 – 758 | 1 | Note 1 |
| 68 | - | 68 | - | - | - | 698 – 728 | 753 – 783 | 1 |  |
| 69 | - | 69 | - | - | - | N/A | 2570 – 2620 | 1 | Note 1 |
| 70 | n70 | 70 | X | - | - | 1695 – 1710 | 1995 – 2020 | 1 | Note 5 |
| 71 | n71 | 71 | X | - | - | 663 – 698 | 617 – 652 | 1 |  |
| 72 | - | 72 | X | - | - | 451 – 456 | 461 – 466 | 1 |  |
| 73 | - | 73 | X | - | - | 450 – 455 | 460 – 465 | 1 |  |
| 74 | n74 | 74 | X | - | - | 1427 – 1470 | 1475 – 1518 | 1 |  |
| 75 | n75 | 75 | - | - | - | N/A | 1432 – 1517 | 1 | Note 1 |
| 76 | n76 | 76 | - | - | - | N/A | 1427 – 1432 | 1 | Note 1 |
| 85 | n85 | 85 | X | - | - | 698 – 716 | 728 – 746 | 1 |  |
| 87 | - | 87 | X | - | - | 410 – 415 | 420 – 425 | 1 |  |
| 88 | - | 88 | X | - | - | 412 – 417 | 422 – 427 | 1 |  |
| NOTE 1: For NR and/or E-UTRA, the band is restricted to operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 2: For UTRA, the band is restricted to operation when dual band is configured (e.g., DB-DC-HSDPA or dual band 4C-HSDPA). The down link frequenc(ies) of this band are paired with the uplink frequenc(ies) of the other FDD band (external) of the dual band configuration. NOTE 3: For E-UTRA, the range 2180-2200 MHz of the DL operating band is restricted to operation when carrier aggregation is configured. NOTE 4: Band 23 is not applicable. NOTE 5: For E-UTRA, the range 2010-2020 MHz of the DL operating band is restricted to operation when carrier aggregation is configured, and TX-RX separation is 300 MHz. For E-UTRA, the range 2005-2020 MHz of the DL operating band is restricted to operation when carrier aggregation is configured, and TX-RX separation is 295 MHz. NOTE 6: DL operation is restricted to 1526-1536 MHz frequency range. UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz per FCC Order DA 20-48. |  |  |  |  |  |  |  |  |  |

UTRA FDD can operate with DB-DC-HSDPA for the band configurations listed in subclause 5.2 c) of TS 25.104 [2].

NOTE:	For BS capable of multi-band operation, the supported operating bands may belong to different Band Categories.

Table 4.5-2: Unpaired bands in NR, E-UTRA and UTRA.

| MSR Band number | Supported RATs and Band Numbers |  |  |  | Uplink (UL) BS receive, UE transmit (MHz) | Downlink (DL) BS transmit, UE receive (MHz) | BC | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NR | E-UTRA | NB-IoT | UTRA |  |  |  |  |
| 33 | - | 33 | - | a) | 1900 – 1920 | 1900 – 1920 | 3 |  |
| 34 | n34 | 34 | - | a) | 2010 – 2025 | 2010 – 2025 | 3 |  |
| 35 | - | 35 | - | b) | 1850 – 1910 | 1850 – 1910 | 3 |  |
| 36 | - | 36 | - | b) | 1930 – 1990 | 1930 – 1990 | 3 |  |
| 37 | - | 37 | - | c) | 1910 – 1930 | 1910 – 1930 | 3 |  |
| 38 | n38 | 38 | - | d) | 2570 – 2620 | 2570 – 2620 | 3 |  |
| 39 | n39 | 39 | - | f) | 1880 – 1920 | 1880 – 1920 | 3 |  |
| 40 | n40 | 40 | - | e) | 2300 – 2400 | 2300 – 2400 | 3 |  |
| 41 | n41 | 41 | X | - | 2496 – 2690 | 2496 – 2690 | 3 | Note 1 |
| 42 | - | 42 | X | - | 3400 – 3600 | 3400 – 3600 | 3 |  |
| 43 | - | 43 | X | - | 3600 – 3800 | 3600 – 3800 | 3 |  |
| 44 | - | 44 | - | - | 703 – 803 | 703 – 803 | 3 |  |
| 45 | - | 45 | - | - | 1447 – 1467 | 1447 – 1467 | 3 |  |
| 48 | n48 | 48 | X | - | 3550 – 3700 | 3550 – 3700 | 3 |  |
| 50 | n50 | 50 | - | - | 1432 – 1517 | 1432 – 1517 | 3 |  |
| 51 | n51 | 51 | - | - | 1427 – 1432 | 1427 – 1432 | 3 |  |
| 52 | - | 52 | - | - | 3300 – 3400 | 3300 – 3400 | 3 |  |
| 53 | n53 | 53 | - | - | 2483.5 – 2495 | 2483.5 – 2495 | 3 |  |
| 54 | - | 54 | X | - | 1670 – 1675 | 1670 – 1675 | 3 |  |
| 77 | n77 | - | - | - | 3300 – 4200 | 3300 – 4200 | 3 |  |
| 78 | n78 | - | - | - | 3300 – 3800 | 3300 – 3800 | 3 |  |
| NOTE 1: Band 41 supports NB-IoT in certain regions. |  |  |  |  |  |  |  |  |

E-UTRA is designed to operate for the carrier aggregation bands defined in TS 36.101 [18]. The E-UTRA channel bandwidth BWChannel for a single carrier and the Aggregated Channel Bandwidth BWChannel_CA for E-UTRA carrier aggregation are specified in Clause 5.6 of TS 36.104 [4].

The NB-IoT channel bandwidth BWChannel is specified in Clause 5.6 of TS 36.104 [4].

The NR BS channel bandwidth and PRB utilization is specified in Clause 5.3 of TS 38.104 [17].

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------