3GPP TSG-RAN WG4 Meeting # 106												R4-23XXXXX

Athens, Greece, 27 February –03 March, 2023

Agenda item:			xx.xx.xx

Source:	Anterix

Title:	TP to XX.XXX: Operating band, channel arrangements, transmitter characteristics, output RF spectrum emissions

Document for:	Approval

# Introduction

At 3GPP RAN #98-e, a spectrum-related work item [1] was agreed to specify a new LTE band in the 900 MHz frequency range.  This document is a TP on analysis of UE requirements for introduction of the new E-UTRA band as defined in the WID and the consensus of responses from 3GPP members.

# Discussion

The objectives from the WID are copied below:

- Specify a new 4G LTE FDD operating band operating with

  - UL: 896 – 901 MHz and DL 935 – 940 MHz

  - To support 1.4 MHz, 3 MHz and 5 MHz channel bandwidths with 15 kHz SCS.

- Study and specify the co-existence requirements

  - Support FCC OOB emission mask in FCC-20-67A1 for all the defined channel bandwidths in the proposed band to ensure co-existence with adjacent narrowband and broadband networks to avoid interference.

  - Study and if needed, specify the protection for B5 and B26 including MPR.

  - Assume reusing band B8 duplexer

- Specify UE RF core requirements to operate in North America with primary operations in the US.

  - Assume power class 3 (23 dBm).

- Specify necessary BS RF core requirement.

- Specify the corresponding RRM requirements, i.e., add the new band for frequency bands grouping.

.

# Operating band, channel arrangements

The core objective of the WI is to standardize the 900MHz block defined by the FCC in the US to private broadband operations.

## Operating bands

The new 900 MHz band shall operate in the following frequency allocations.

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| xxx | 896 MHz | – | 901 MHz | 935 MHz | – | 940 MHz | FDD |

## UE channel bandwidth

The new band shall support standard E-UTRA channel bandwidths of 1.4 MHz, 3 MHz and 5 MHz.  The entire band is a 5 + 5 paired spectrum allocation.

| Channel bandwidth BWChannel [MHz] | 1.4 | 3 | 5 |
| --- | --- | --- | --- |
| Transmission bandwidth configuration NRB | 6 | 15 | 25 |

### M1 and M2 category

UE support for category M1 and M2 shall be supported and operate in both half duplex FDD mode and full-duplex FDD mode.

### NB1 and NB2 category

UE support for NB1 and NB2 shall be supported and operate in HDD-DD duplex mode.

## Channel raster

The channel raster is 100 kHz for all bands, which means that the carrier centre frequency must be an integer multiple of 100 kHz.

## TX-RX frequency separation

The intent is to reuse the band 8 duplexor with a default E-UTRA TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation of 39 MHz.

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| xxx | 39 MHz |

# Transmitter characteristics

## Transmit power

## UE maximum output power

PC3 transmit power will be used for the new 900 MHz band UE.

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| xxx |  |  |  |  | 23 | ±2 |  |  |

# Output RF spectrum emissions

## Regulatory Requirements

The FCC-20-67A1 emission mask rules create an asymmetrical emission mask specification as follows:

- UL:         43+10log(P)       -13dBm as measured in an RBW of 100kHz

- DL:         50+10log(P)       -20dBm as measured in an RBW of 100kHz

The FCC rules for 900 MHz Broadband Emissions Mask allow for 30kHz RBW.  This allows for measuring in at least one percent of the fundamental emission bandwidth in the 100 kHz immediately adjacent to the band edge i.e. to be able to measure in 30kHz RBW.  This rule aligns with 3GPP RF conformance device testing standards.

The FCC certification tests employ combinations of RBW’s 100 and 30kHz and active PRB’s.  The FCC SEM is intended for all the defined channel bandwidths in the proposed band to ensure co-existence with adjacent narrowband and broadband networks to avoid interference.

### Adjacent bands

The new 900 MHz band also must operate in coexistence with band 5 and band 26.  The band 26 deployments in the US market are nearly 30 MHz away in frequency and this does not pose a coexistence issue.  The cellular downlink (894 MHz) and 900 MHz uplink (896 MHz) are much closer in frequency.

Current FCC regulations have broadband (LTE/5G) operations in 897.5-900.5 MHz for uplink though and the 896-897.5 MHz spectrum is currently occupied by narrowband LMR.

The 894-896 MHz band is used for commercial and business aviation (OFDM based) for ATG  HPUE. It should be noted that the Federal Communications Commission DA 22-657 was granted to essentially increase power measured within Peak Average Ratio (PAR) limits.

The suggested approach is not to specify additional protection for B5/n5 and B26/n26 from 900MHz UL so that no A-MPR requirement is needed.  We agree with this approach and the following SEM should be used for the allowed channel bandwidths.

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | Measurement bandwidth |
| ± 0-1 | -10 | -13 | -15 | 30 kHz |
| ± 1-2.5 | -10 | -10 | -10 | 1 MHz |
| ± 2.5-2.8 | -25 | -10 | -10 | 1 MHz |
| ± 2.8-5 |  | -10 | -10 | 1 MHz |
| ± 5-6 |  | -25 | -13 | 1 MHz |
| ± 6-10 |  |  | -25 | 1 MHz |
| ± 10-15 |  |  |  | 1 MHz |
| ± 15-20 |  |  |  | 1 MHz |