3GPP TSG RAN WG4 Meeting #105                          R4- 2300297

Toulouse, FR, November 14th - November 18th, 2022

Title:	Discussion on LTM delay requirements

Source:	China Telecom

Agenda item:			9.23.3.3

Document for:	Discussion

- Introduction

In the RAN4 #104bis-e meeting, the L1/L2 based inter-cell mobility were discussed and WF [1] has been approved. In this paper, some issues on L1/L2 inter-cell mobility delay requirements are further discussed.

- Discussion

Issue 3-2-1: Starting point of cell switch delay for PCell/PSCell

In last meeting, companies had different views on starting point of cell switch delay for PCell/PSCell The Way forward during the last meeting is duplicated as below [1]:

< Wayforward >: FFS the following options - The starting point of cell switch delay for PCell/PSCell is that UE receives cell switch command.

In legacy L3 cell switch, the cell switch delay is the time from UE receives the cell switch command to UE starts transmission of the new uplink PRACH channel to the target cell.

For compatibility of LTM and legacy L3 cell switch, and convenient to handle. We think LTM could follow the starting point of cell switch as legacy L3 cell switch.

Proposal 1: The starting point of cell switch delay for PCell/PSCell is that UE receives cell switch command.

Issue 3-2-2: Ending point of RACH-based cell switch delay for PCell/PSCell

In last meeting, companies had different views on the ending point of RACH-based cell switch. The Way forward during the last meeting is duplicated as below [1]:

< Wayforward >: FFS the following options - For RACH-based cell switch, cell switch delay for PCell/PSCell ends at UE transmitting PRACH to the target cell.

Following the last issue, for compatibility of LTM and legacy L3 cell switch, and convenient to handle. We think LTM could follow the ending point of RACH-based cell switch as legacy L3 cell switch.

Proposal 2: For RACH-based cell switch, cell switch delay for PCell/PSCell ends at UE transmitting PRACH to the target cell.

Issue 3-2-3: Ending point of RACH-less cell switch delay for PCell/PSCell

In last meeting, companies had different views on the ending point of RACH-less cell switch. The Way forward during the last meeting is duplicated as below [1]:

< Wayforward >: FFS the following options - Option 1 (MTK): For RACH-less cell switch, cell switch delay for PCell/PSCell ends at UE transmitting SR on PUCCH or PUSCH on the target cell. - Option 2 (CTC, Apple, OPPO, Huawei): UE performs the first DL/UL reception/transmission on the indicated beam of the target cell. - Option 3 (Xiaomi): UE starts to transmit valid CSI report of target cell. - Option 4 (vivo): RAN4 will further discuss end point of delay requirements for L1L2-triggered mobility after RAN1 conclude whether/how to perform the TRS tracking, CSI acquisition before/after cell switch command, and RAN4 agrees there is related impact on UE RRM requirements. - Option 5 (Nokia): RAN4 to discuss if Tfirst-data is within the RAN4 scope. - Other options are not precluded

For RACH-less cell switch, UE needs not to transmit PRACH to the target cell. So we could not follow the definition as RACH-based cell switch. In this case, we need to find an occasion that is easy to be identified by network to indicate the ending point of RACH-less cell switch.

As RAN2 assumed, HO interruption time for L1/L2-based inter-cell mobility is the time from UE receives the cell switch command to UE performs the first DL/UL reception/transmission on the indicated beam of the target cell.

So we think the ending point of RACH-less cell switch delay is that UE performs the first DL/UL reception/transmission on the indicated beam of the target cell.

Proposal 3: For RACH-less cell switch, cell switch delay for PCell/PSCell ends at UE performs the first DL/UL reception/transmission on the indicated beam of the target cell.

Issue 3-3-1: RACH-based Cell switch delay for Pcell/PSCell

In last meeting, companies had different views on RACH-based cell switch delay. The Way forward during the last meeting is duplicated as below [1]:

< Wayforward >: FFS the following options - The baseline of RACH-based Cell switch delay requirements for PCell/PSCell is Tdelay = Tcmd + Tprocessing + Tsearch + T∆ + Tmargin + TIU, where TIU is the uncertainty in acquiring the first available PRACH occasion in the new cell. - FFS: the exact value of each component. Some components can be 0 in certain cases, if agreed. - FFS: add other components, e.g., execution time and/or TCI state switching time

In our opinion, for RACH-based, the starting point and ending point of LTM are same as legacy L3 mobility. So we think the components of LTM could reuse as L3 mobility in TS38.133. It could be shown as

Tdelay = Tcmd + Tsearch + Tprocessing + Tmargin + TIU + T∆,

where, Tcmd is the time for processing L1/L2-command, Tsearch is the time required to search the target cell, Tprocessing is the time for UE processing, T∆ is time for fine time tracking and acquiring full timing information of the target cell, Tmargin is the time for SSB or CSI-RS post-processing, TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell.

For other components, we think the specific mechanism and procedures of LTM is important to study it. So wait for RAN2 progress.

Proposal 4: For RACH-based case,

Tdelay = Tcmd + Tsearch + Tprocessing + Tmargin + TIU + T∆,

where, TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell.

Proposal 5: Wait for RAN2 progress to add other components.

Issue 3-3-2: RACH-less Cell switch delay for Pcell/PSCell

In last meeting, companies had different views on RACH-less cell switch delay. The Way forward during the last meeting is duplicated as below [1]:

< Wayforward >: FFS the following options - Option 1 (CTC, MTK): Tdelay = Tcmd + Tprocessing + Tsearch + T∆ + Tmargin + TIU, where TIU is the uncertainty in acquiring the first PUSCH transmission occasion or SR on PUCCH. - Option 2 (Xiaomi): Tdelay = Tcmd + Tprocessing + T∆ + Tmargin + TTCI_switch +TCSI_report, - Option 3 (OPPO): Tdelay = Tcmd + Tprocessing + Tsearch + T∆ + Tmargin + TIU, where TIU is the uncertainty in the first DL/UL reception/transmission on the indicated beam - Option 4 (Ericsson): Tdelay = Tcmd + Tprocessing

For RACH-less case, the cell switch delay could be shown as

Tdelay = Tcmd + Tsearch + Tprocessing + Tmargin + TIU + T∆,

where, Tcmd, Tsearch, Tprocessing, T∆ and Tmargin are same as those of RACH-based cell switch. TIU is the uncertainty in the first DL/UL reception/transmission on the indicated beam.

For Tsearch, it depends on whether to consider unknown cell case for RACH-less cell switch, i.e., issue 3-3-7. If not, Tsearch could be excluded.

Proposal 6: For RACH-less case, the cell switch delay could be shown as

Tdelay = Tcmd + Tsearch + Tprocessing + Tmargin + TIU + T∆,

where, Tcmd, Tsearch, Tprocessing, T∆ and Tmargin are same as those of RACH-based cell switch. TIU is the uncertainty in the first DL/UL reception/transmission on the indicated beam.

- If do not consider unknown cell case for RACH-less cell switch, Tsearch could be excluded.

Issue 3-3-3: Tcmd

In last meeting, companies had different views on Tcmd. The Way forward during the last meeting is duplicated as below [1]:

< Wayforward >: - Wait for RAN2 progress.

This issue relies on the specific mechanism and procedures of LTM, so we agree to wait for RAN2 progress.

Proposal 7:Wait for RAN2 progress to specify Tcmd.

Issue 3-3-4: Processing time: Tprocessing

In last meeting, companies had different views on Tprocessing. The Way forward during the last meeting is duplicated as below [1]:

< Wayforward >: FFS the following options - Option 1 (MTK): The baseline is: Tprocessing,2=20ms for intra-FR cell switch, Tprocessing,2=40ms for inter-FR cell switch - FFS: reduction on Tprocessing,2 under certain conditions - Option 2 (CTC, ZTE, Huawei): The time for UE process could been reduced. - Option 3 (Nokia): - RAN4 to differentiate processing times for intra-DU and inter-DU scenarios. - RAN4 to discuss how to specify delay requirements to account for L2 delay requirements for each user plane protocol layer. - LTM processing delays are not based on legacy Tprocessing_1 an Tprocessing_2 component requirements - RRC UE processing requirements are analyzed further to understand if RRC configuration can be performed before LTM switch command - Other options not precluded

As RAN2 assumed, the time for UE processing, before and after cell switch command, respectively. This may include L2/3 reconfiguration, RF retuning, baseband retuning, security update if needed, etc.

In our understanding, if some procedures have been done before UE receives the cell switch command, i.e. security update, the time for UE process could been reduced in certain case.

Proposal 8: The time for UE processing could been reduced if some procedures have been done before UE receive the cell switch command.

Issue 3-3-6: Cell search for RACH-based cell switch: Tsearch

In last meeting, companies had different views on Tsearch. The Way forward during the last meeting is duplicated as below [1]:

< Wayforward >: FFS the following options - For RACH-based cell switch, Tsearch equals to 0 when target cell is known or target cell is current active Scell - FFS: any other conditions

If UE could know the valid target cell and beam information in advance, the time for cell search could reach to 0ms. In this case, target cell is known for UE. Target cell is current active Scell equal to target cell is known.

Proposal 9: For RACH-based cell switch, Tsearch equals to 0 when target cell is known or target cell is current active Scell.

Issue 3-3-11: Tinterruption

In last meeting, companies had different views on Tinterruption. The Way forward during the last meeting is duplicated as below [1]:

< Wayforward >: FFS the following options - Option 1 (CTC, CMCC, ZTE): The components of L1/L2 cell switch interruption Tinterruption are the components of L1/L2 inter-cell mobility delay except Tcmd - Option 2 (Nokia): LTM cell switch interruption time should be minimized, and upper limit should be agreed not to exceed the existing L3 HO interruption time. The target should be to be as close to a beam switch delay as possible

When UE processes the cell switch command, it could remain the connection with source cell. So it should be excluded from Tinterruption. In which case, the components of L1/L2 cell switch interruption Tinterruption are the components of L1/L2 inter-cell mobility delay except Tcmd.

Proposal 10: Tinterruption are the components of L1/L2 inter-cell mobility delay except Tcmd.

Issue 3-4-1: known cell conditions

In last meeting, MTK proposed the views on known cell conditions. The Way forward during the last meeting is duplicated as below [1]:

< Wayforward >: FFS the following option - Option 1 (MTK): use the conditions for L3 HO with a bit modification: - The target cell is known if it has been meeting the following conditions: - During the last 5 seconds before the reception of the handover cell switch command: - the UE has sent a valid L1 [or L3] measurement report for the target cell and - One of the SSBs measured from the NR target cell being configured remains detectable according to the cell identification conditions specified in clause 9.3, - One of the SSBs measured from the target cell also remains detectable during the handover cell switch delay according to the cell identification conditions specified in clause 9.3. otherwise it is unknown.

We agree add the L1 measurement report for the target cell in known case.

Proposal 11: Agree with the proposal on known cell conditions of MTK in last meeting.

- Summary

In this paper, we provide our views on L1/L2 inter-cell mobility delay requirements. From this discussion we have derived the following observations and proposals:

Proposal1: The starting point of cell switch delay for PCell/PSCell is that UE receives cell switch command.

Proposal2: For RACH-based cell switch, cell switch delay for PCell/PSCell ends at UE transmitting PRACH to the target cell.

Proposal3: For RACH-less cell switch, cell switch delay for PCell/PSCell ends at UE performs the first DL/UL reception/transmission on the indicated beam of the target cell.

Proposal 4: For RACH-based case,

Tdelay = Tcmd + Tsearch + Tprocessing + Tmargin + TIU + T∆,

where, TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell.

Proposal 5: Wait for RAN2 progress to add other components.

Proposal 6: For RACH-less case, the cell switch delay could be shown as

Tdelay = Tcmd + Tsearch + Tprocessing + Tmargin + TIU + T∆,

where, Tcmd, Tsearch, Tprocessing, T∆ and Tmargin are same as those of RACH-based cell switch. TIU is the uncertainty in the first DL/UL reception/transmission on the indicated beam.

- If do not consider unknown cell case for RACH-less cell switch, Tsearch could be excluded.

Proposal 7:Wait for RAN2 progress to specify Tcmd.

Proposal 8: The time for UE processing could been reduced if some procedures have been done before UE receive the cell switch command.

Proposal 9: For RACH-based cell switch, Tsearch equals to 0 when target cell is known or target cell is current active Scell.

Proposal 10: Tinterruption are the components of L1/L2 inter-cell mobility delay except Tcmd.

Proposal 11: Agree with the proposal on known cell conditions of MTK in last meeting.

- References

[1] R4-2220403, WF on L1/L2 inter-cell mobility, MediaTek Inc.