3GPP TSG-RAN WG4 Meeting#106      	R4-23

Athens Greece, 27 Feb – 3 Mar, 2023

Title:	A-MPR for dedicated spectrum less than 5MHz for FR1

Agenda item:	9.14.3

Source:	Nokia

Document for:	Discussion

# 1	Background

In approved WI dedicated spectrum less than 5MHz for FR1[1] there are four example bands listed

- Specify necessary RAN4 requirements to support deploying NR in spectrum allocations from approximately 3 MHz up to below 5 MHz [RAN4], including in bands n100, n8, n26 and n28:

In this contribution we make initial assessment of possible A-MPR need.

# 2 	Discussion

## 2.1 n100

n100 has been specified already with out A-MPR hence introducing 3 MHz bandwidth do not cause a need to investigate n100.

## 2.2 n8

Band n8 does have A-MPR for Japan see figure 1,

Figure 1: n8 Japan portion (Japan 900)

Relevant emission requirements is presented below for NS_43 and NS_43U.

Table 6.5.3.3.5-1: Additional requirements for "NS_43" and “NS_43U”

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15 |  |
| 860 ≤ f ≤ 890 | -40 | 1 MHz |
| NOTE 1: Applicable for 5 MHz and 15 MHz channel BW confined between 900 MHz and 915 MHz and for 10 MHz channel BW confined between 905 MHz and 915 MHz |  |  |

AND

Table 6.5.2.4.2-1: UTRA ACLR requirement

|  | Power class 3 |
| --- | --- |
| UTRAACLR1 | 33 dB |
| UTRAACLR2 | 36 dB |

In simulations a 3 MHz carrier was placed at 900 - 903 MHz and results indicated that no A-MPR is needed. This is quite expected as current A-MPR definition is only valid for LCRB > 15 RB.

## 2.3	n28

Band n28 has A-MPR for NS_18 which is associated for operation in Korea to protect TV services, see Figure 2

Figure 2: n28 and TV protection

In simulations a 3 MHz carrier was placed at 703 - 706 MHz and results indicated that no A-MPR is needed. This is quite expected as current LTE specification do not define A-MPR for 3 MHz CHBW.

## 2.4	n26

We will provide results for this case in next meeting

# 3	Conclusion

In this contribution we have provided our A-MPR assessment for 3 MHz NR carrier for bands n8, n28 and n100. Our simulations indicate that no A-MPR is needed. We are calling others do similar study.

# 4	References

- [1] RP-222645, Revised WID: NR support for dedicated spectrum less than 5MHz for FR1, RAN#95e