3GPP TSG-RAN WG4 Meeting#106      	R4-2300429

Athens Greece, 27 Feb – 3 Mar, 2023

Title:	UE RF specification impact due to Introduction of 900 MHz LTE Band in the US

Agenda item:	10.4.3

Source:	Nokia

Document for:	Discussion

# 1	Background

In this contribution we discuss the necessary changes into 38.101-1 due to WI on Introduction of 900 MHz LTE Band in the US [1], see Figure 1.

Figure 1: Introduction of 900 MHz LTE Band in the US

# 2 	Discussion

In Table 1 we go trough all UE Tx and Rx requirements one by one and asses which requirements need modification with introduction of 900 MHz LTE Band in the US. We also present as reference how current band 8 is specified.

Table 1: Specification impact analysis

| NR UE Tx/Rx requirement | Current LTE specification for band 8 | Proposal for NR 900 MHz LTE Band in the US |
| --- | --- | --- |
| 6.2.2 UE maximum output power | 23 ±22 NOTE 2: 2 refers to the transmission bandwidths (Figure 5.6-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB | 23 ±22 Note 2 is needed as band 8 duplex-filter is assumed to be used, otherwise would not be needed. |
| 6.2.3 MPR | MPR is same for all bands | MPR can be applied to new band |
| 6.2.4 A-MPR | Band 8 does not have A-MPR | As the new band is for new region A-MPR is potentially needed to protect existing adjacent services such as band 5 and band 26. |
| 6.2.5 Configured transmitted power | Band agnostic | No specification impact |
| 6.3.2 Minimum output power | Band agnostic, -40 dBm. | No specification impact |
| 6.3.3 Transmit OFF power | Band agnostic, -50 dBm. | no specification impact |
| 6.3.4 Transmit ON/OFF time mask | Band agnostic | No specification impact |
| 6.3.5 Power control | Band agnostic | No specification impact |
| 6.5.1 Frequency error | Band agnostic | No specification impact |
| 6.5.2.1 EVM | Band agnostic | No specification impact |
| 6.5.2.2 Carrier leakage | Band agnostic | No specification impact |
| 6.5.2.3 In-band emissions | Band agnostic | No specification impact |
| 6.5.2.4 EVM equalizer spectrum flatness | Band agnostic | No specification impact |
| 6.6.1 Occupied bandwidth | Band agnostic | No specification impact |
| 6.6.2.1 Spectrum emission mask | General SEM for band 8 | General SEM is not applicable for new band |
| 6.6.2.2 Additional Spectrum emission mask |  | FCC SEM for new band |
| 6.6.2.3.1 E-UTRA ACLR | Band agnostic | No specification impact |
| 6.6.2.3.2 UTRA ACLR | Band agnostic | No specification impact |
| 6.5.3.1 General spurious emissions | Band agnostic | No specification impact |
| 6.5.3.2 Spurious UEtoUE co-ex | Protects bands relevant to band 8 | New requirement needed for protetion of US bands |
| 6.5.3.3 Additional spurious emissions | Not needed for band 8 | New requirement may be needed for protection of existing adjacent services such as band 5 and band 26. |
| 6.5.4 Transmit intermodulation | Band agnostic | No specification impact |
| 7.3.2 Reference sensitivity | Band 8 REFSENS for 5 MHz is -97 | Band 8 requirement can be used as same duplex-filter is assumed |
| 7.4 Max input level | Band agnostic | No specification impact |
| 7.5 Adjacent channel selectivity | Band agnostic | No specification impact |
| 7.6.2 In-band blocking | Band agnostic | New band needs to added into Table 7.6.2.1-2 |
| 7.6.3 Out of band blocking | Band agnostic | New band needs to added into Table 7.6.3.1-2 |
| 7.6.4 Narrow band blocking | Band agnostic | New band needs to added into Table 7.6.4-1 |
| Spurious response | Band agnostic | No specification impact |
| 7.7 Intermodulation characteristics | Band agnostic | No specification impact |

Observation: As a conclusion on table 1 we can note that most of the technical work needed is concentrating on defining the A-MPR for the new band to protect existing adjacent services.

# 3	Conclusion

In this contribution we have discussed the necessary changes into 36.101 due toiIntroduction of 900 MHz LTE Band in the US.

Observation: As a conclusion on table 1 we can note that most of the technical work needed is concentrating on defining the A-MPR for the new band to protect existing adjacent services.

# 4	References

- [1] RP-223513, New WID on Introduction of 900 MHz LTE Band in the US, RAN#98e