3GPP TSG-RAN WG4 Meeting # 105                	R4-2300698

Athens, Greece, February 27 – March 3, 2023

Source: 	Nokia, Nokia Shanghai Bell

Title: 	PDSCH simulation results for ext71 GHz

Agenda item: 	6.2.6.1.1

Document for: 	Discussion

Introduction

This paper presents out updated PDSCH simulation results using the TDL-D channel model for the definition of demodulation requirements related to the operation between 52.6 GHz and 71 GHz.

Simulation assumptions

Simulation assumptions are shown below and are derived considering the agreements on the WF [1].

The simulations are done using the proposed channel models defined in [1].

Table 1 PDSCH Simulation Assumptions agreed for 70% Peak Throughput [1]

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [KHz] | 120 KHz, 480 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (10 ns and 30 ns delay spread) TDLD (10 ns and 30 ns delay spread) |
| Antenna configuration | 2x2 ULA Low |
| Velocity | 3 km/h, 10 km/h |
| PA Model | None |
| gNB PN Model | None |
| UE PN Model | Up to UE implementation |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol index |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| MCS/TBS | From MCS Table 1 (TS38.214): Up to MCS 20 (64QAM). |
| NOH | 6 |
| Number of HARQ Processes | 8 for 120kHz and 16 for 480kHz SCS |

Table 2 PDSCH Simulation Assumptions agreed for 30% Peak Throughput [1]

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [KHz] | 120 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (30 ns delay spread) |
| Antenna configuration | 2x2 ULA Low |
| Velocity | 3 km/h, 10 km/h |
| PA Model | None |
| gNB PN Model | None |
| UE PN Model | Up to UE implementation |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol index |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| MCS/TBS | From MCS Table 1 (TS38.214) |
| NOH | 6 |
| Number of HARQ Processes | 16 |

PDSCH Simulation Results

Here we provide our updated simulation for PDSCH where the TDL-D model is used. Updated results are highlighted with green.

Table 3 Results for PDSCH ideal

| Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS | Rank | Channel Model | Antenna Configuration and Correlation Matrix | RB Allocation Size | Fraction of maximum throughput (%) | SNRBB (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 100/120 | MCS4 | 1 | TDLA30-650 | 2x2 ULA Low | 66 | 70 | -2.4 |
| 100/120 | MCS13 | 1 | TDLA30-200 | 2x2 ULA Low | 66 | 70 | 5.8 |
| 100/120 | MCS13 | 1 | TDLA30-650 | 2x2 ULA Low | 66 | 30 | -0.5 |
| 100/120 | MCS17 | 1 | TDLD30-200 | 2x2 ULA Low | 66 | 70 | 8.3 |
| 100/120 | MCS17 | 1 | TDLD30-200 | 2x2 ULA Low | 32 | 70 | 8.2 |
| 400/480 | MCS4 | 1 | TDLA10-200 | 2x2 ULA Low | 66 | 70 | -2.4 |
| 400/480 | MCS13 | 1 | TDLD10-200 | 2x2 ULA Low | 20 | 70 | 4.5 |

Table 3 Results for PDSCH with Impairment

| Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS | Rank | Channel Model | Antenna Configuration and Correlation Matrix | RB Allocation Size | Fraction of maximum throughput (%) | SNRBB (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 100/120 | MCS4 | 1 | TDLA30-650 | 2x2 ULA Low | 66 | 70 | 1.1 |
| 100/120 | MCS13 | 1 | TDLA30-200 | 2x2 ULA Low | 66 | 70 | 8.8 |
| 100/120 | MCS13 | 1 | TDLA30-650 | 2x2 ULA Low | 66 | 30 | 3.0 |
| 100/120 | MCS17 | 1 | TDLD30-200 | 2x2 ULA Low | 66 | 70 | 11.3 |
| 100/120 | MCS17 | 1 | TDLD30-200 | 2x2 ULA Low | 32 | 70 | 11.2 |
| 400/480 | MCS4 | 1 | TDLA10-200 | 2x2 ULA Low | 66 | 70 | -1.4 |
| 400/480 | MCS13 | 1 | TDLD10-200 | 2x2 ULA Low | 20 | 70 | 7.5 |

Conclusion

In this document we have provided our updated simulation results for PDSCH for simulations using the TDL-D channel model and based on the agreed simulation assumptions and test cases.

References

- R4-2220187 WF for FR2-2 UE demodulation part