3GPP TSG-RAN WG4 Meeting #106	R4-2301026

Athens Greek, 27th Febuary – 3rd March, 2023

Source:	Ericsson

Title:	Discussion on NR SAN demodulation requirement

Agenda item:	6.1.6.1

Document for:	Discussion

# Introduction

In RAN4#105 meeting, preliminary SNR values and conformance test procedure statements are endorsed. But the manufacture declaration and applicability rules were not discussed in detail. In this contribution, these remaining issues will be furtherly discussed.

# Discussion

## 2.1 Manufacture declaration

Based on current discussion, following SAN demodulation requirements are introduced.

- PUSCH with transform coding disabled

- PUSCH with transform coding enabled

- PUSCH UL timing adjustment

- PUSCH repetition type A

- PUCCH format 0/1/2/3/4

- PUCCH multi-slot

- PARCH false alarm and miss detection

Referring to legacy manufacture declaration in 38.141-1/2, following manufacture declarations could be introduced for SAN.

Table 2.1-1 SAN demodulation relevant declarations

| Declaration | Description | Applicability |  |
| --- | --- | --- | --- |
|  |  | SAN 1-H | SAN 1-O |
| TAB connectors used for performance requirement testing | To reduce test complexity, declaration of a representative (sub)set of TAB connectors to be used for performance requirement test purposes. At least one TAB connector mapped to each demodulation branch is declared. | x |  |
| PUSCH mapping type | Declaration of the supported PUSCH mapping type as specified in TS 38.211 [17], i.e., type A, type B or both. | x | x |
| PUSCH additional DM-RS positions | Declaration of the supported additional DM-RS position(s), i.e., pos0, pos1 or both. |  |  |
| PUCCH format | Declaration of the supported PUCCH format(s) as specified in TS 38.211 [17], i.e., format 0, format 1, format 2, format 3, format 4. | x | x |
| PRACH format and SCS | Declaration of the supported PRACH format(s) as specified in TS 38.211 [17], i.e., format: 0, 2, B4, C2. Declaration of the supported SCS(s) per supported PRACH format with short sequence, as specified in TS 38.211 [17], i.e., 15 kHz, 30 kHz or both. | x | x |
| Additional DM-RS for PUCCH format 3 | Declaration of the supported additional DM-RS for PUCCH format 3: without additional DM-RS, with additional DM-RS or both. | x | x |
| Additional DM-RS for PUCCH format 4 | Declaration of the supported additional DM-RS for PUCCH format 4: without additional DM-RS, with additional DM-RS or both. | x | x |
| PUCCH multi-slot | Declaration of multi-slot PUCCH support. | x | x |

Proposal 1: Take Table 2.1-1 for SAN manufacture declarations.

## 2.2 Applicability rule

According to the introduced requirements, most of relevant applicability rules in 38.141-1/2 could be reused for SAN demodulation. But the applicability rule for demodulation connectors needs further check.

In RAN4#104bis-e GTW, companies agreed on following applicability rule:

Unless otherwise stated, for a SAN supporting different numbers of antenna connectors (for SAN type 1-C) or TAB connectors (for SAN type 1-H) (see D.xxx in table yyy), the tests with low MIMO correlation level shall apply only for the highest number of supported connectors, and the specific connectors used for testing are based on manufacturer declaration.

There is no SAN type 1-C supported based on RF agreement, so the corresponding description should be removed.

Proposal 2: Take following applicability rule for SAN conducted demodulation requirements.

Unless otherwise stated, for a SAN supporting different numbers of TAB connectors (for SAN type 1-H) (see D.xxx in table yyy), the tests with low MIMO correlation level shall apply only for the highest number of supported connectors, and the specific connectors used for testing are based on manufacturer declaration.

For radiated requirements, the general rule for different demodulation branch was agreed and introduced to TS38.181 v1.0.1.

Radiated performance requirements are only specified for up to 2 demodulation branches.

If the SAN type 1-O uses polarization diversity and has the ability to maintain isolation between the signals for each of the demodulation branches, then radiated performance requirements can be tested for up to two demodulation branches (i.e. 1RX or 2RX test setups). When tested for two demodulation branches, each demodulation branch maps to one polarization.

If the SAN type 1-O does not use polarization diversity then radiated performance requirements can only be tested for a single demodulation branch (i.e. 1RX test setup).

The legacy applicability rule for demodulation branch is as below in 38.141-2.

Unless otherwise stated, for a BS declared to support more than 2 demodulation branches (for BS type 1-O and BS type 2-O), the performance requirement tests for 2 demodulation branches shall apply, and the mapping between connectors and demodulation branches is up to BS implementation.

Based on current agreement, only 1 and 2 demodulation branches are considered for SAN 1-O, so it seems no need to introduce similar applicability rule above for SAN. But on the other hand, the statement on more than 2 branches here seems still OK considering future possible implementation. In that case, similar applicability rule can be kept.

Proposal 3: Take following applicability rule for SAN radiated demodulation requirements.

Unless otherwise stated, for a SAN declared to support more than 2 demodulation branches (for SAN type 1-O), the performance requirement tests for 2 demodulation branches shall apply, and the mapping between connectors and demodulation branches is up to SAN implementation.

# Conclusions

Proposal 1: Take Table 2.1-1 for SAN manufacture declarations.

Proposal 2: Take following applicability rule for SAN conducted demodulation requirements.

Unless otherwise stated, for a SAN supporting different numbers of TAB connectors (for SAN type 1-H) (see D.xxx in table yyy), the tests with low MIMO correlation level shall apply only for the highest number of supported connectors, and the specific connectors used for testing are based on manufacturer declaration.

Proposal 3: Take following applicability rule for SAN radiated demodulation requirements.

Unless otherwise stated, for a SAN declared to support more than 2 demodulation branches (for SAN type 1-O), the performance requirement tests for 2 demodulation branches shall apply, and the mapping between connectors and demodulation branches is up to SAN implementation.

# References

[1]	R4-2220161, WF on NTN SAN demodulation requirements, Huawei, HiSilicon

[2]	TS38.181 v1.0.1