3GPP TSG-RAN WG4 Meeting # 106	R4-2301044

Athens, Greece, February 27 – March 3, 2023

Agenda Item: 9.8.3.5

Source: MediaTek Inc.

Title: Discussion on dual TCI state switch for multi-Rx UE

Document for: Discussion

# Introduction

According to WF [1] and discussion summary [2], RAN4 had some agreements in the last meeting while some issues were discussed without conclusion yet. In this meeting, this WI is divided into six agenda items to be discussed : (1) general issues, (2) L1 measurement, (3) RLM/BFD/CBD, (4) scheduling/measurement restriction, (5) dual TCI state switching and (6) receive timing difference. The discussion in this paper focus on the “dual TCI state switching”.

# Discussion

In the following sections, below issues will be discussed sequentially.

- Requirement applicability of known and unknown TCI state

- Detectable condition

## 2.1 Requirement applicability of known and unknown TCI state

For dual TCI state switching, there is one open issue regarding whether to consider unknown TCI state switching in the last meeting. The discussed cases are

(1) known TCI state + known TCI state

(2) known TCI state + unknown TCI state

(3) unknown TCI state + unknown TCI state.

To our understanding, dual TCI state switching is for simultaneous reception of PDSCH(s) for 4-layer MIMO, i.e., increase data throughput. If network indicates UE switches to unknown TCI states, the performance cannot be guaranteed, leading to degraded throughput and waste of UE power. This is very similar to CQI/RI reporting, i.e., typically network only schedules rank-4 PDSCH after UE reports RI=4 and the corresponding CQI. So, we do not see the needs and reason why network indicates UE switches to an unknown TCI state.

Proposal 1: For dual TCI state switching, only define the requirements for the case when two indicated TCI states are known.

## 2.2 Detectable condition

As below, there is one open issue regarding detectable condition in the last meeting.

#### Issue 2-3-4: Detectable condition of RS signals - Proposals - Option 1 (ZTE): - For detectable condition, all RSs in the same QCL chain for the target TCI state should remain detectable during the entire measurement/switch period. - Option 2 (Huawei): - For known target TCI state for switching, all RSs in the same TCI chain for the target TCI need to be detectable during switching period, which is not multi-Rx specific issue. - Option 3 (Ericsson): - In RRM requirements for simultaneous reception of RSs in different QCL type D infos, not only the target RSs but also the associated signals in the QCL infos shall remain detectable during the entire measurement periods, e.g., a detectability condition also for the associated signal needs to be added

We support proposal 1 and 3. Since the flied “repetition” cannot be configured for the tracking RS, UE has to take the other RS which is in the same TCI chain as the tracking RS to perform UE Rx beam selection. Therefore, it would be more reliable if all RSs in the same TCI chain remain detectable during the entire measurement period. Furthermore, as same QCL Type-D means that the 2 RS are transmitted from the same Tx spatial filter, we tend to believe that the detectability should be the same for all RS in the same TCI chain. So, the following proposal is suggested.

Proposal 2: For detectable condition, all RSs in the same TCI chain for the target TCI state should remain detectable during the entire measurement/switch period.

# Summary

In this paper, the discussion of R18 multi-Rx chains is provided. We have the following proposal:

Proposal 1: For dual TCI state switching, only define the requirements for the case when two indicated TCI states are known.

Proposal 2: For detectable condition, all RSs in the same TCI chain for the target TCI state should remain detectable during the entire measurement/switch period.

# Reference

- R4-2220402, WF on NR FR2 multi-Rx chain DL reception – Part 3, Ericsson, RAN4 #105

- R4-2220063, Topic summary for [105][217] FR2_multiRx_RRM_part3, Ericsson, RAN4 #105