# 3GPP TSG-RAN WG4 Meeting # 106		  R4-2301223

# Athens, Greece, Feb. 27-Mar. 03, 2023

# Source:	ZTE Corporation

# Title:	Discussion on UE RF for dedicated spectrum less than 5MHz for FR1

# Agenda Item:	9.14.3

# Document for:	Approval

- Introduction

During RAN#97-e meeting, a revised Work Item [1] has been approved on introduction of NR support for dedicated spectrum less than 5MHz for FR1. In this contribution, we want to share some initial views on UE RF requirements.

- Specify necessary RAN4 requirements to support deploying NR in spectrum allocations from approximately 3 MHz up to below 5 MHz [RAN4], including in bands n100, n8, n26 and n28:

  - Specify system parameters (including channel and sync rasters) for the associated dedicated spectrum.

  - Minimize impact on RF requirements:

    - Reuse 5 MHz channel bandwidth at least for FRMCS use case (assuming co-located NR and GSM-R with same operator).

    - Specify the required RF requirements for optional 3 MHz channel bandwidth in bands n100, n8, n26 and n28.

  - Specify RRM requirements while minimizing specification impact to support operation in dedicated spectrum allocations from approximately 3 MHz up to below 5 MHz.

- Discussion

In the following Table 2-1, we share some initial views on the UE RF requirements for dedicated spectrum less than 5MHz for FR1 and details could be found as following:

Table 2-1: Summary of the proposals for UE RF requirement for 900 MHz NR Band

| Tx RF requirements part |  |
| --- | --- |
| Output power dynamics | To add output power dynamics requirements for 3 MHz channel bandwidth. |
| Transmit signal quality | No requirement impacts. |
| Spectrum emission mask | To add spectrum emission mask requirement for 3 MHz channel bandwidth. |
| ACLR | To add ACLR requirement for 3 MHz channel bandwidth. |
| Receiver characteristics |  |
| Reference sensitivity | To add REFSENS requirement for 3 MHz channel bandwidth. |
| Maximum input level | To add maximum input level requirement for 3 MHz channel bandwidth. |
| ACS | To add ACS requirement for 3 MHz channel bandwidth. |
| Blocking characteristics | To add blocking requirement for 3 MHz channel bandwidth. |
| Spurious response | To add spurious response requirement for 3 MHz channel bandwidth. |
| Intermodulation | To add IMD requirement for 3 MHz channel bandwidth. |
| Spurious emissions | No requirement impacts. |

- Conclusion

In this contribution, we want to share some views on UE RF requirements and the proposal is made as following:

Proposal 1: To use the proposals in Table 2-1 for UE RF requirements for NR support for dedicated spectrum less than 5MHz for FR1.

- Reference

RP-222645, Revised WID: NR support for dedicated spectrum less than 5MHz for FR1, Nokia, Nokia Shanghai Bell