# 3GPP TSG-RAN WG4 Meeting # 106		  R4-2301227

# Athens, Greece, Feb. 27-Mar. 03, 2023

# Source:	ZTE Corporation

# Title:	Discussion on BS RF requirements for 900 MHz LTE Band

# Agenda Item:	10.4.4

# Document for:	Approval

- Introduction

During RAN#98-e meeting, a new Work Item [1] has been approved on introduction of 900 MHz LTE Band in the US. In this contribution, we want to share some initial views on BS RF requirements.

- Specify a new E-UTRA/LTE FDD operating band operating with

  - UL: 896 – 901 MHz and DL 935 – 940 MHz

  - To support 5 MHz, 3 MHz, 1.4 MHz and NB-IoT channel bandwidths with 15 kHz SCS and 3.75kHz SCS.

- Study and specify the co-existence requirements

  - Support FCC OOB emission mask in FCC-20-67A1 for all the defined channel bandwidths in the proposed band to ensure co-existence with adjacent narrowband and broadband networks to avoid interference.

  - Study and if needed, specify the protection for B5 and B26 including MPR.

  - Assume reusing band B8 duplexer

- Specify UE RF core requirements to operate in North America with primary operations in the US.

  - Assume power class 3 (23 dBm).

- Specify necessary BS RF core requirement.

- Discussion

  - Operating band and band numbering

As indicated in the objective, a new E-UTRA/LTE FDD operating band should be defined.

Table 2.1-1: E-UTRA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| [106] | 896 MHz | – | 901 MHz | 935 MHz | – | 940 MHz | FDD |

Proposal 1: To define the new LTE band as in table 2.1-1.

  - Carrier frequency and EARFCN

For E-UTRA channel numbers, we propose to use the similar descending manner to define as following:

Table 2.2-1: E-UTRA channel numbers

| E-UTRA Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low (MHz) | NOffs-DL | Range of NDL | FUL_low(MHz) | NOffs-UL | Range of NUL |
| [106] | 935 | 70656 | 70656 – 70705 | 896 | 134292 | 134292 – 134341 |

Proposal 2: For E-UTRA channel numbers, it is proposed to define as Table 2.2-1.

  - Summary

Based on the above, the summary of the corrections on TS 36.104 are given in Table 2.3-1.

Table 2.3-1. Summary of related RF requirement

| General part |  |
| --- | --- |
| Operating bands | A new band number for 900 MHz LTE Band is needed. |
| UE channel bandwidth | To support 5 MHz, 3 MHz and 1.4 MHz channel bandwidths. |
| Channel arrangement | The EARFCN should be needed for the new added FDD band. |
| Tx RF requirements part |  |
| Base station output power | All of these requirements are defined as band or channel bandwidth agnostic except for output power dynamics, therefore it’s not expected to have any impacts with the introduction of 900 MHz LTE Band. |
| Output power dynamics |  |
| Transmit ON/OFF power |  |
| Transmitted signal quality |  |
| ACLR | The Report and Order establishes an out-of-band emissions (OOBE) limit outside a licensee’s frequency band of operation to be attenuated by at least 50 + 10 log (p) dB for downlink operations in the 936.5-939.5 MHz band. |
| Operating band unwanted emissions |  |
| Transmitter spurious emissions | To add the additional spurious emissions requirements for 900 MHz LTE Band. To add the co-location with other base station for 900 MHz LTE Band. |
| Rx RF requirements part |  |
| REFSENS | No requirement impacts. |
| Dynamic range requirement | No requirement impacts. |
| ACS | No requirement impacts. |
| Blocking requirement | No requirement impacts. |
| RX IMD | No requirement impacts. |
| Rx spurious emission | No requirement impacts. |
| ICS | No requirement impacts. |

- Conclusion

In this contribution, we give some initial discussion on BS RF requirements for 900 MHz LTE band and the proposals are made as following:

Proposal 1: To define the new LTE band as in table 2.1-1.

Proposal 2: For E-UTRA channel numbers, it is proposed to define as Table 2.2-1.

Proposal 3: To use the proposals in Table 2.3-1 for 900 MHz LTE band BS RF requirements.

- Reference

RP-223530, New WID on Introduction of 900 MHz LTE Band in the US, Anterix