3GPP TSG-RAN WG4 Meeting # 106	R4- 2301322

# Athens, Greece, February 27 – March 3, 2023

Source:	LG Electronics

Title:	Discussion on timing for ATG RRM

Agenda item:	9.13.4.3

Document for:	Discussion

Introduction

In this contribution, we provide our views on timing requirement.

Discussion

In the last RAN4 meeting, RAN4 agreed the legacy TN requirement for deriveSSB-IndexFromCell can be reused, and deriveSSB-IndexFromCell is applicable to ATG is still open as follow.

Issue 3-3-4: deriveSSB-IndexFromCell and deriveSSB-IndexFromCell-inter tolerance - Option 1: Legacy TN requirement can be reused. It is up to network to decide whether to enable ‘deriveSSB-IndexFromCell’ and ‘deriveSSB-IndexFromCell-inter’ or not - Option 2: RAN4 to study to utilize the deriveSSB-IndexFromCell based on the propagation delay difference between serving and neighbor cell. Agreement: - Legacy TN requirement can be reused. Whether ‘deriveSSB-IndexFromCell’ and/or ‘deriveSSB-IndexFromCell-inter’ are applicable to ATG scenarios is FFS

For NR FR1 TDD, deriveSSB-IndexFromCell is always enabled, so UE can assume that the neighbor cell is synchronous with the serving cell. However, due to longer propagation delay for ATG deployment, deriveSSB-IndexFromCell might not be always feasible. So, unlike TN TDD case, network needs to enable or disable deriveSSB-IndexFromCell for ATG TDD deployment. In order for the network to determine whether to enable or disable deriveSSB-IndexFromCell, the distance information between the serving cell/ neighboring cell and ATG UE would be required.

- Observation 1: For ATG TDD deployment, ‘deriveSSB-IndexFromCell’ and/or ‘deriveSSB-IndexFromCell-inter’ are not always applicable due to longer propagation delay.

- Proposal 1: For ATG TDD deployment, it could be up to network to enable or disable ‘deriveSSB-IndexFromCell’ and/or ‘deriveSSB-IndexFromCell-inter’, but in order for network to determine whether to enable deriveSSB-IndexFromCell’ and/or ‘deriveSSB-IndexFromCell-inter’, assistance information for distance between serving cell/neighbour cell and ATG UE is needed.

Conclusion

In this contribution, we provide our views on timing requirement.

- Observation 1: For ATG TDD deployment, ‘deriveSSB-IndexFromCell’ and/or ‘deriveSSB-IndexFromCell-inter’ are not always applicable due to longer propagation delay.

- Proposal 1: For ATG TDD deployment, it could be up to network to enable or disable ‘deriveSSB-IndexFromCell’ and/or ‘deriveSSB-IndexFromCell-inter’, but in order for network to determine whether to enable deriveSSB-IndexFromCell’ and/or ‘deriveSSB-IndexFromCell-inter’, assistance information for distance between serving cell/neighbour cell and ATG UE is needed.

Reference

[1] R4-2220361 “WF on NR ATG RRM requirement”, CMCC