3GPP TSG-RAN WG4 Meeting #106	R4-2301398

Athens, Greece, 27 Feb – 3 Mar, 2023

Source:	ZTE Corporation

Title:	Discussion on general aspects for air-to-ground network

Agenda Item:	9.13.4.1

Document for:	Approval

- Introduction

In RAN plenary 96 meeting, a revised WID[1] for Rel-18 was approved to study the air-to-ground for NR. The objective of this WID is as follows.

The core part of this work item includes - Specify features to core specifications for ATG BS and UE [RAN4] - Scenario: - BS on the ground, and the CPE type of UE mounted in the aircraft - A direct radio link between BS on the ground and CPE type of UE mounted in the aircraft - Note: The deployment characteristics described in the justification section shall be taken as a basis for the technical discussion. - Specify core requirements for coexistence between ATG and IMT terrestrial network - Example bands include n1, n78 and n79. - Perform FR1 co-existence evaluation for ATG network (e.g. ACLR, ACS) - Identify key characteristics where it is necessary to differentiate ATG ground-based BS and UEs from conventional ground based BS and UEs - Aim to reuse existing requirements for BS and UE where possible, e.g., - Reuse TN BS requirements for ATG BS - Specify RF requirements for ATG UE/BS - Considering the results of co-existence simulations in terms of impact on emissions and RX requirements, cell sizes and link budgets, technology capabilities, likely BS and UE architectures and other relevant aspects. - Taking into account identified differences between ATG and fully ground based systems - Consider BS type 1-C/1-H/1-O and specify the requirements - Consider conductive requirements for UE - Specify RRM core requirements for ATG UE - Taking into account identified differences between ATG and fully ground based systems - Considering the different nature of ATG UEs and their view of the network, increased cell sizes and other relevant aspects - Specify new UE/BS type(s) for ATG network if necessary

In last meeting, RAN4 has spread general discussion on RRM aspects in ATG case. The following agreements were achieved in [2] around general aspects:

Issue 1-1-1: ISD assumption Agreements: - Investigate the RRM impactions of ISD [14]-200km. - Further capture the conclusion on square bracket which will come from RF session. Issue 1-1-2: Maximum distance between ATG BS and Aircraft UE Agreements - Maximum distance between ATG BS and Aircraft UE for further RRM analysis is ≥ [200] km. - Upper bound is FFS. Issue 1-1-3: Duplex mode and SCS - For the test cases, further specify the combination of duplex mode and SCS at perf stage. - For the core requirements, discuss following Options: - Option 1: For duplex mode/SCS configurations, no impacts on core requirements (HW, CATT) - Option 2: Consider all the combinations between FDD/TDD and 15kHz/30kHz (CMCC, ZTE) - Option 3: Determine either of the following options, - Option 3-1: Only consider 15kHz SCS for FDD 2GHz and 30kHz for TDD for requirement development. (Apple) - Option 3-2: Consider all the SCS’s of 15kHz, 30kHz and 60kHz for FDD and TDD (Apple) - Option 4: Consider 30kHz SCS as baseline assumption to analyses the impact of propagation delay difference in TDD (LGE) - Option 5: SCS type to be supported for ATG is discussed as part of the timing requirements. (Ericsson) Agreements: - For the test cases, further specify the combination of duplex mode and SCS at perf stage. - For the core requirements, consider all the SCS’s of 15kHz, 30kHz [and 60kHz] for FDD and TDD Issue 1-1-4: TDD pattern Agreements: - The TDD pattern is up to NW configuration. No impaction on core requirements is observed, and it can be further discussed in the stage of performance. Issue 1-1-5: whether new UAI reporting method could be introduced - Option 1: ATG WI should avoid to introduce new UAI reporting method. (CATT, CMCC, ZTE (no obvious motivation and feasibility guarantee to introduce the above UAI report)) - Option 2: There is no need to preclude new UAI and its reporting methods at this stage. Postpone the decision on assistance information (both UE-based and network-based) until the RRM requirement framework is clear. (LGE, Apple, Ericsson) - Option 2a: Assistance information related to unpredictable changes in flight operation mode are introduced: 1) UE detecting and informing the NW, 2) NW informing the UE (Ericsson) - Option 3: Whether to introduce new UE assistant information will require RAN1/RAN2 work, which shall not be decided in WG. (HW) Issue 1-1-6: Band table Way forward: - Postpone the discussion, wait the conclusion from RF session.

In this document, we will provide some further analysis on the remaining general issues of ATG based on the multiple new characteristics of ATG scenario.

- Discussion

Based on the characteristics of ATG system and some approved assumption in RF session, the following key points should be noted:

- Extremely large ISD, e.g. about 100km to 200 km

- Extremely high flight speed, e.g. up to 1200km/h

- Utilizing same frequency for deploying both ATG and TN(terrestrial network), e.g. n1, n78, n79 -- So only focus on FR1

- Much powerful on-board ATG terminal capacity

- R18 only focuses on SA mode and single connectivity with operation in a single band. DC, FR2, inter-RAT are not applicable to ATG. CA is possible in future release depending on demands

- Height of CPE: 3km--10km

We provide our analysis for multiple RRM aspects given all the above key points of ATG system.

- 2.1 Maximum distance between CPE and BS

In our opinion, the traditional TN deployment is typical, which can be used for ATG scenario. Additionally, considering the horizon beam based coverage has been approved in RF session, so we believe there are two possible deployment for ATG scenario, as shown in Figure 1 and Figure 2.

Figure 1: Traditional TN deployment

Figure 2: Horizon beam coverage deployment

- For traditional TN deployment, the maximum distance between CPE and BS can be assumed as below:

- For Deployment Method 1, considering the CPE altitude is relative very small compared with ISD, so

- ISD = [100, 200] km,

- Radius = ISD/3,

- Maximum distance between CPE and BS ≈ 2*Radius = [68, 134] km.

- For Deployment Method 2, considering the CPE altitude is relative very small compared with ISD, so

- ISD = [100, 200] km,

- Radius = ISD/√3,

- Maximum distance between CPE and BS ≈ Radius = [58, 115] km.

- For horizon beam coverage deployment, the maximum distance between CPE and BS can be assumed as below:

- ISD = [100, 200] km,

- In [3], some basic assumption was provided as follows:

- Base station vertical coverage range (degrees): 25°for non sub-array and 10°for sub-array,

- Mechanical up tilt (degrees): 14°for non sub-array and 6.5°for sub-array,

- For non sub-array case

- For ISD = 100 km, Maximum distance between CPE and BS ≈ 100 + 10/tan(14°+ 0.5*25°) = 120 km

- For ISD = 200 km, Maximum distance between CPE and BS ≈ 200 + 10/tan(14°+ 0.5*25°) = 220 km

- Maximum distance between CPE and BS ≈ [120, 220] km.

- For sub-array case

- For ISD = 100 km, Maximum distance between CPE and BS ≈ 100 + 10/tan(6.5°+ 0.5*10°) = 150 km

- For ISD = 200 km, Maximum distance between CPE and BS ≈ 200 + 10/tan(6.5°+ 0.5*10°) = 250 km

- Maximum distance between CPE and BS ≈ [150, 250] km.

- Observation 1: Under the assumption of traditional TN deployment, considering the ISD of 100-200 km, the maximum distance between CPE and BS is 68-134 km or 58-115 km.

- Observation 2: Under the assumption of horizon beam coverage deployment, considering the ISD of 100-200 km, the maximum distance between CPE and BS is 120-220 km for non sub-array case and 150-250 km for sub-array case.

- 2.2 UE assistance information(UE detecting and informing the NW)

During last meeting, regarding to UAI, the following options were captured in [2]:

- Option 1: ATG WI should avoid to introduce new UAI reporting method. - Option 2: There is no need to preclude new UAI and its reporting methods at this stage. Postpone the decision on assistance information (both UE-based and network-based) until the RRM requirement framework is clear. - Option 2a: Assistance information related to unpredictable changes in flight operation mode are introduced: 1) UE detecting and informing the NW, 2) NW informing the UE - Option 3: Whether to introduce new UE assistant information will require RAN1/RAN2 work, which shall not be decided in WG.

During previous several meetings, multiple types of UAI were discussed, including the UAI of altitude, location, velocity, flight path, propagation delay information and remaining service time for serving cell. It seems that all of these UAI are helpful to address the timing and frequency syn related issues caused by large ISD and high speed of moving.

However, regarding to all of these UAI, we raise a common concern: Under the high speed of up to 1200km/h, whether the report from UE can be received by the NW in time enough? If not, it would be meaningless.

Observation 3: Regarding to all of these UAI, a common concern is raised: Under the high speed of up to 1200km/h, whether the report from UE can be received by the NW in time enough? If not, it would be meaningless.

To our understanding, when we talking about whether UE need to report the UAI or not, a key point is whether UE performs pre-compensation. If UE would perform time/frequency pre-compensation, the UAI such as altitude, location, velocity and flight path are almost useless for NW.

Observation 4: If UE would perform time/frequency pre-compensation, the UAI such as altitude, location, velocity and flight path are almost useless for NW.

Referring to the timing related UAI, such as propagation delay estimation, such info can help NW to identify the overlapping between DL slot and UL slot due to large propagation delay so as to help NW identify the suitable TDD pattern. But the gain brought by such UAI depends on the common concern, whether the report is in time enough. When UE estimate the propagation delay, it should predict the value when NW receives such UAI report. The accuracy of such predict would determine the effect.

Observation 5: When UE estimate the propagation delay, it should predict the value when NW receives such UAI report. The accuracy of such predict would determine the effect.

Regarding to the service related UAI, such as the remaining service time, we are not sure how to estimate such info by UE, Since all scheduling decision is up to NW.

Observation 6: Regarding to the service related UAI, such as the remaining service time, we are not sure how to estimate such info by UE, Since all scheduling decision is up to NW.

In a whole, based on our above analysis, we can not see obvious motivation and feasibility guarantee to introduce the above UAI report.

Proposal 1: Based on our above analysis, we can not see obvious motivation and feasibility guarantee to introduce the above UAI report.

- Conclusion

In this contribution, we have the following proposals for the consideration of general RRM aspects for ATG system:

- Observation 1: Under the assumption of traditional TN deployment, considering the ISD of 100-200 km, the maximum distance between CPE and BS is 68-134 km or 58-115 km.

- Observation 2: Under the assumption of horizon beam coverage deployment, considering the ISD of 100-200 km, the maximum distance between CPE and BS is 120-220 km for non sub-array case and 150-250 km for sub-array case.

Observation 3: Regarding to all of these UAI, a common concern is raised: Under the high speed of up to 1200km/h, whether the report from UE can be received by the NW in time enough? If not, it would be meaningless.

Observation 4: If UE would perform time/frequency pre-compensation, the UAI such as altitude, location, velocity and flight path are almost useless for NW.

Observation 5: When UE estimate the propagation delay, it should predict the value when NW receives such UAI report. The accuracy of such predict would determine the effect.

Observation 6: Regarding to the service related UAI, such as the remaining service time, we are not sure how to estimate such info by UE, Since all scheduling decision is up to NW.

Proposal 1: Based on our above analysis, we can not see obvious motivation and feasibility guarantee to introduce the above UAI report.

- Reference

- RP-221369 “Revised WID on Air-to-ground network for NR”, CMCC

- R4-2220361 “WF on NR ATG RRM requirements”, CMCC

- R4-2220542 “WF on ATG co-existence evaluation”, CMCC