3GPP TSG-RAN4 Meeting #106	R4-2301485

# Athens, Greece, 27th Feb 2023 – 3rd March 2023

Source:			Ericsson

Title:			Spectrum less than 5 MHz - BS RF requirements

Agenda item:		9.14.4

Document for:		Approval

# Introduction

A new Work Item ([1]) was approved in RAN#95e to add NR support for dedicated spectrum less than 5 MHz for FR1, introducing new channel bandwidth(s) narrower than 5 MHz.

This contribution discusses the BS RF requirements, giving an overview of expected impacts when considering such introduction.

# Discussion

## Context

The system parameters are discussed in our other contribution [3].

As proposed in this contribution, only 3 MHz channel bandwidth is considered in the following sections. And the corresponding spectrum utilization is assumed to be 15 RBs.

## BS RF requirements

### General

Considering NR 3 MHz channel bandwidth spectrum utilization is the same as LTE one, we propose to align NR 3 MHz channel BW requirements to the corresponding LTE ones as starting point.

Proposal1: As starting point, align NR 3 MHz channel BW UE RF requirements with the corresponding LTE 3MHz channel BW requirements.

In the following sub-sections, we are providing an overview of BS RF requirements impact when introducing new 3 MHz channel bandwidth.

### BS Tx requirements

The following Table 1 gives an overview of the impact on BS Tx RF requirements when introducing 3 MHz channel bandwidth.

Table 1: BS RF Tx requirements overview - 3 MHz channel BW

| Requirements | Impact |
| --- | --- |
| BS output power | For band n100, the additional requirement shall be scaled for 3 MHz to keep PSD constant. |
| Output power dynamics |  |
| RE power control dynamic range | No impact expected. |
| Total power dynamic range | Requirement for 3 MHz channel BW shall be added . |
| NB-IoT RB power dynamic range for NB-IoT operation in NR in-band | Not applicable, NB-IoT in-band support is not mentioned in the objectives of this WI. |
| Transmit ON/OFF power | No impact expected. |
| Transmitted signal quality |  |
| Frequency error | No impact expected. |
| Modulation quality | No impact expected. |
| Time alignment error | No impact expected. |
| Unwanted emissions |  |
| Occupied bandwidth | No impact expected. |
| Adjacent Channel Leakage Power Ratio | Requirement for 3 MHz channel BW shall be added. |
| Operating band unwanted emissions | A new OBUE mask shall be specified, based on LTE 3 MHz one. For band n100, an additional requirement for 3 MHz channel BW should also be added. |
| Transmitter spurious emissions | No impact expected. |
| Transmitter intermodulation | No impact expected. |

### BS Rx requirements

The following Table 2 gives an overview of the impact on BS Rx RF requirements when introducing 3 MHz channel bandwidth.

Table 2: BS RF Rx requirements overview - 3 MHz channel BW

| Requirements | Impact |
| --- | --- |
| Reference sensitivity level | Requirement for 3 MHz channel BW shall be added. |
| Dynamic range | Requirement for 3 MHz channel BW shall be added. |
| ACS | Requirement for 3 MHz channel BW shall be added. |
| In-band blocking | Requirement for 3 MHz channel BW shall be added For band n100, an additional narrow band blocking requirement for 3 MHz channel BW shall also be added. |
| Out of band blocking | No impact expected. |
| Receiver spurious emission | No impact expected. |
| Receiver intermodulation | Requirement for 3 MHz channel BW (interferer) shall be added. For band n100, an additional narrowband intermodulation requirement for 3 MHz channel BW shall also be added. |
| In-channel selectivity | Requirement for 3 MHz channel BW (interferer) shall be added. |

# Conclusion

In this contribution we discussed the impacts on the BS RF parameters in the scope of the new WI on NR support for dedicated spectrum less than 5 MHz. We made the following proposals:

Proposal1: As starting point, align NR 3 MHz channel BW UE RF requirements with the corresponding LTE 3MHz channel BW requirements.

# References

- RP-222645, WID: NR support for dedicated spectrum less than 5MHz for FR1, Nokia, Nokia Shanghai Bell

- R1-2212919, LS on NR support for dedicated spectrum less than 5MHz for FR1, Nokia

- R4-2301483, Spectrum less than 5 MHz - System parameters, Ericsson