3GPP TSG-RAN4 Meeting #106	R4-2301515

# Athens, Greece, February 27th – March 3rd, 2023

Agenda item:			9.26.2.3

Source:	vivo

Title:	RF simulation results for transparent schemes for enhancement to reduce MPR

Document for:	Discussion

- Introduction

In RAN4#105 meeting, some RF simulation parameters and side conditions about power domain enhancements were discussed in the WF [1], captured as follows:

<Way forward/Agreement>: - For comparison purpose, the minimum set of number of RBs is 8,16, 32, 64, … < NRB for spectrum extension - The number above includes excess RBs. - Extension/reservation factors in evaluations are 0 – 0.375 for spectrum extension, which can be further limited to a certain range according to some other factors, e.g., modulations etc., if needed. - Extension/reservation factor (α) as Excess band size / Total allocation, where - Inband size: Occupied REs after DFT-block - Excess/reserved band size: The amount of spectrum extension. - Total allocation size (Inband size + Excess/reserved band size): Occupied REs after spectrum extension - Include a reference waveform(DFT-s-OFDM) which meet the existing requirements for gain evaluation. - Consider excess band as a part of the allocated UL transmission bandwidth for IBE. - Define ACLR requirement according to power class also with power boost, e.g., if power class 3 is boosted and to be equivalent to power class 2, the ACLR of power class 3 should apply. - RAN4 only focuses on pi/2 BPSK and QPSK.

In this contribution, we provide some simulation results for transparent schemes. A separate simulation parameters assumption document is also submitted in contribution [2]. The corresponding results for non-transparent schemes are shown in [3].

- Simulation parameters

Simulation parameters used in these simulations are shown in Table 1.

Table 1. Simulation parameters for FDSS

| Carrier frequency | 4GHz |
| --- | --- |
| Channel BW | 20 MHz |
| SCS | 15 kHz |
| Modulation | QPSK, pi/2 BPSK |
| Waveform | DFT-S-OFDM |
| Number of RBs | 16, 32, 64 |
| Channel | PUSCH, 14 OFDM symbols |
| Spectral shaping filter | - 3-tap (0.28 1 0.28) - 2-tap (1 0.28) - Truncated RRC - No filter (reference case) |
| Power class | PC 3 |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 4 |
| Number of DMRS symbols | 2 |
| Number of PUSCH data symbols | 12 |

- Simulation results

Based on the LS from RAN1 [4], transparent schemes include Frequency domain spectrum shaping w/o spectrum extension, named as ‘normal FDSS’ in this contribution. Also, reference waveform (DFT-s-OFDM) without filter is named as ‘no FDSS’.

In order to obtain the characteristics of the transparent scheme, two methods (i.e., [no FDSS], [normal FDSS]) are used in MPR simulation and the modulation modes are pi/2 BPSK and QPSK. It can be observed from Figure 1 that for both ‘no FDSS’ and ‘normal FDSS’, the number of allocated PRBs is the same, and there is no extension RB here.

Figure 1. Different FDSS schemes

The physical implementation challenge is not considered in the simulation, three kinds of filters are used: 3tap, 2tap, TRRC. The preliminary MPR simulation results are as follows, the comparison reference of power gain is ‘no FDSS’:

Table 2. Power gain for pi/2 BPSK and QPSK modulation, compared with no FDSS

| normal FDSS | pi/2 BPSK | QPSK |  |  |
| --- | --- | --- | --- | --- |
|  | 16RBs, inner allocation | 16RBs, inner allocation | 32RBs, inner allocation | 64RBs, outer allocation |
| 3-tap | 1.2 | 1 | 1 | 1.4 |
| 2-tap | 0.8 | 0.8 | / | / |
| TRRC | 1.1 | 0.2 | / | / |

It can be observed that for inner allocation (e.g., 16RB@RB46), power gain of normal FDSS for QPSK may be less obvious than pi/2 BPSK for power boosting. It can be noted that power gain from FDSS without spectrum extension (compared to reference waveform without filter) is typically less than 1.5 dB.

Observation 1: Power gain from FDSS without spectrum extension (compared to reference waveform without filter) is typically less than 1.5 dB.

At the same time, for QPSK modulation, FDSS w/o SE provides smaller gain than FDSS w SE, mainly compared to FDSS w SE with copying data, as shown in the contribution [3].

Observation 2: For QPSK modulation, transparent schemes provide smaller gain than non-transparent schemes.

On the other side, among the three filters, 3-tap filter seems to have better performance than the other two filters, providing the highest power gain. At the same time, comparing with different RBs, it can be seen that 32RBs provide similar performance to 16RBs, however 64RBs provide better performance than 16RBs and 32RBs. So it is possible that for QPSK with 3-tap filter, using FDSS without SE to improve power boosting is more effective for lager RBs to some extent.

Observation 3: For QPSK with 3-tap filter, FDSS without SE to increase power boost is more effective for lager RBs to some extent.

- Conclusion

Observation 1: Power gain from FDSS without spectrum extension (compared to reference waveform without filter) is typically less than 1.5 dB.

Observation 2: For QPSK modulation, transparent schemes provide smaller gain than non-transparent schemes.

Observation 3: For QPSK with 3-tap filter, FDSS without SE to increase power boost is more effective for lager RBs to some extent.

- References

- R4-2220551 WF on the MPR reduction for coverage enhancement, Nokia, Nokia Shanghai Bell, #RAN4 105.

- R4-2301514 Discussion on RF simulation parameters for enhancement to reduce MPR, vivo, #RAN4 106.

- R4-2301516 RF simulation results for non-transparent schemes for enhancement to reduce MPR, vivo, #RAN4 106.

- R4-2300007 LS to RAN4 for further information on RAN1 assumptions for LLS performance evaluation of MPR/PAR reduction solutions, RAN1, #RAN4 106.