# 3GPP TSG-RAN WG4 Meeting #106	R4-2301532

Athens, Greece, February 27 – March 3, 2023

Source: 	vivo

Title: 	Discussion and reply LS on interference modelling for duplex evolution

Agenda Item:	9.19.1

Document for:	Approval

Introduction

An incoming LS [1] was received from RAN1 on interference modelling and three issues need to be confirmed by RAN4. In this contribution, we provided our views for response to the LS.

Discussion

Agreement-1

Regarding the modelling of inter-site gNB-gNB co-channel inter-subband CLI agreed in RAN1#110bis for the case that both large scale fading and small scale fading are modelled for gNB-gNB co-channel channel model, the second part of inter-site gNB-gNB co-channel inter-subband CLI across all Rx chains at one UL RB, caused by receiver selectivity at victim gNB, can be modelled as

Iselectivity=z0n,z1n,…,zNR-1nT

  - zkn~N0,σz,n2, k=0,1,…,NR-1, is modelled as white Gaussian noise

  - σz,n2=PblockerICSBS×NDLRB

  - Pblocker=1NR*m∈Used DL RBsHCLImWmsm2

  - HCLIm is the NR×NT channel matrix between aggressor gNB and victim gNB at DL RB m, the analog beams of the aggressor gNB and the victim gNB can be taken into account by HCLIm,

  - Wm is the digital precoder at DL RB m at aggressor gNB, WmF=1,

  - sm is the symbol transmitted at DL RB m at aggressor gNB with transmission power for each layer as Ptxper-RB=Ptxmax/NDLRB.

  - NDLRB is the total number of DL RBs in the DL subbands,

  - RAN1 can assume ICSBS (in channel selectivity) is given by gNB ACS unless further RAN4 guidance is received.

    - Send LS to RAN4 to confirm RAN1 understanding and check whether ICSBS can be modelled depending on the value of the blocker interference, e.g.,

ICSBS= ICS1            Pblocker<P1 ICS2,   P1< Pblcoker<P2ICS3             Pblocker>P2

  - Note: Pblocker can be reported by companies

For now, RAN4 did not consider ICS model as the BS co-channel Rx model. It was agreed to use BS ACS for co-channel Rx model. BS ACS can be considered as frequency flat model, 46 dBc for FR1 BS and 24 dBc for FR2 BS. This model is not dependent on Pblocker. A reply LS [2] was sent to RAN1 for interference modelling and RAN4 recommended:

  - Iselectivity can be obtained based on the RX power and the ACS.

  - RAN4 has not yet preclude further study on the possibility of improved receiver impairment performance compared to gNB ACS.

RAN4 would like RAN1 to consider gNB ACS for co-channel Rx model.

Agreement-2

For SLS in RAN1, for co-site inter-sector co-channel inter-subband CLI modelling, reuse similar method as gNB self-interference modelling as follows.

Ico-siteper-RB=Ico-site-sector-1per-RB+Ico-site-sector-2per-RB

Ico-site-sector-xper-RB=Ptxsector-x, per-RBαco-site*Nused-DL-RBsector-xNDLRB

- Ptxsector-x, per-RB is DL Tx power of sector x per RB (in linear scale), Ptxsector-x, per-RB=Ptxsector-x, max/NDLRB.

- Ptxsector-x, max is the maximum DL Tx Power of sector x on the two DL subbands (in linear scale).

- NDLRB is the total number of DL RBs in the DL subbands.

- Nused-DL-RBsector-x is the number of DL RBs allocated for DL transmission of sector x.

- αco-site is the interference suppression capability of co-site inter-sector co-channel inter-subband CLI.

  - 10*log10αco-site=spatial isolationdB+10*log1011ACLRBS+1ACSBS+…

    - Note: ACLRBS and ACSBS are in linear scale. gNB ACLR (i.e.,ACLRBS) is provided as the candidate for TX leakage, and gNB ACS (i.e.,ACSBS) is provided as the candidate for Receiver impairment.

  - Companies shall report the value of αco-site assumed in the simulations with feasibility of how these values were derived.

  - Send LS to RAN4 confirming the model and asking the value ranges for spatial isolation, and values of  10*log10ACLRBS and  10*log10ACSBS.

For now, spatial isolation is not concluded and it will be discussed in the RSIC analysis framework [3]. Once it is settled, RAN4 will send the value ranges for spatial isolation. For BS ACLR/ACS, existing BS RF requirements can be referenced:

- FR1: BS ACLR (45 dB)    BS ACS (46 dB)

- FR2: BS ACLR (28 dB)    BS ACS (23 dB)

Agreement-3

For SLS in RAN1, regarding Tx leakage model of UE-UE co-channel inter-subband CLI modelling, Option 1 is used as starting point.

- Option 1: RAN1 to take in-band emission (IBE) defined in TS38.101-1 and TS38.101-2 as starting point.

- Send LS to RAN4 to ask them whether it can be modelled as an equivalent frequency flat model (e.g., IBEUE,ave) based on RAN4 IBE requirement, and if possible, what is the value of IBEUE,ave

For now, RAN4 does not consider to simplify the UE IBE model as an equivalent frequency flat model. In the Reply LS [2], RAN4 informed RAN1 that:

- The IBE-based model shall be used for TX modelling for UE-UE CLI for the co-channel case in RAN1 system-level simulation:

  - IBE models provided in clause 6.4.2.3 in TS38.101-1 and clause 6.4.2.3.4 in TS38.101-2 shall be followed.

  - The general and IQ Image part of in-band emission model shall be considered, while the carrier leakage part can be ignored.

Conclusion

This contribution discusses interference modelling issues for duplex evolution in the LS [1]. Our proposed reply are as in the discussion part.

- References

[1] R1-2212963, LS on interference modelling for duplex evolution, CMCC, RAN1#111.

[2] R4-2220243, LS response to RAN1 for interference modelling and Sub-band configuration, Samsung, RAN4#105.

[3] R4-2220244, WF for the feasibility from BS aspect, Samsung, RAN4#105.