3GPP TSG-RAN WG4 Meeting #106 	R4-2301707

# Athens, Greece, February 27 – March 3, 2023

Title:	[DRAFT] Reply LS to RAN1 on further conclusions about L1 intra- and inter- frequency measurement

Response to:	R1-2210727

Release:	Rel-18

Work Item:	NR_Mob_enh2-Core

Source:	RAN WG4

To:	RAN WG1

Cc:	RAN WG2

Contact Person:

Name:	Yanliang SUN

E-mail Address:	 Yanliang.sun@vivo.com

Attachments:

1. Overall Description:

In RAN4 #106 meetings, RAN4 have further discussed the questions from RAN1 in R1-2210727 and would like to provide the following responses on top of R4-2220733.

Question 1 (to RAN4): As mentioned in this agreement, while RAN1 assumes Rel-17 ICBM CSI measurement as starting point for L1 intra-frequency measurement for Rel-18 L1/L2 mobility, RAN1 wonders if the restriction on e.g., SFN offset alignment, BWP setting, i.e. non-serving cell SSB should be covered by serving cell active BWP, and Rx timing difference, etc, described in 9.13.2 of TS38.133 for intra-frequency L1 non-serving measurement can be relaxed or not.

RAN4 responses to Q1:

In the context of R18 LTM, from the perspective of RRM requirements, RAN4 has agreed to support two types of L1 measurements:

- Type 1: L1 measurement without prior precise downlink sync to the target cell.

  - Intermediate results of legacy L3 measurement is used in the L1L2 reporting

  - For FR2, rough Rx beam is assumed, and therefore sharing between L1 and L3 measurement is not needed.

  - UE performs the measurement only within SMTC, and therefore there is no need for SFN offset configuration.

  - For intra-frequency measurement, UE may perform the measurement no matter it is within active BWP or not. In case it is performed outside active BWP, interruption is expected as specified in 8.2.2.2.3 of TS 38.133.

  - For intra-frequency measurement, UE may perform the measurement as long as the Rx timing difference among all the serving CCs is less than MRTD specified in TS 38.133.

    - Note: Definition of ‘Intra-frequency L1 measurement’ is provided in R4-2220733.

- Type 2: L1 measurement with prior precise downlink sync to the target cell.

  - R17 L1 measurement is assumed, as described in 9.13.2 of TS 38.133

  - For FR2, fine Rx beam is assumed, and RAN4 will specify sharing factor between neighbour cell L1 measurement and L3 measurement.

  - UE performs the measurement based on SSB periodicity. Regarding the restriction on SFN offset alignment, it is NOT needed as long as the SSBs for L1-RSRP measurement and SMTCs are NOT fully non-overlapped.

  - UE performs the measurement only within the active BWP.

  - As R18 baseline, RRM requirements will be specified only assuming UE performs the measurement when Rx timing difference between the cells/RSs to be measured is less than CP.

    - RAN4 will further discuss whether/how to specify higher UE capability in R18.

Question 2 (to RAN4): As mentioned in this agreement, RAN1 would like to ask RAN4 to confirm our understanding that the supported scenarios not included in intra-frequency are regarded as inter-frequency for L1 measurement, which includes at least the following scenarios:

-	The frequency of the measured RS not covered by any of the active BWPs of SpCell and Scells configured for a UE, but covered by some of the configured BWPs of SpCell and Scells configured for a UE.

-	The frequency of the measured RS not covered by any of the configured BWPs of SpCell and Scells configured for a UE

Also, RAN1 would like to inform RAN4 of our understanding that the introduction of measurement gap and SMTC for L1 inter-frequency measurement, if any, is expected to be a RAN4 issue.

RAN4 responses to Q2:

RAN4 will support inter-frequency L1 measurement performed within measurement gaps in R18. RAN4 will further discuss whether/how to define new measurement gaps for the inter-frequency L1 measurements.

2. To RAN WG1 group.

ACTION: 	RAN4 respectfully asks RAN1 to take above conclusions in consideration.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 Meeting #106-bis-e	Apr. 17 – Apr. 26	Online

TSG-RAN4 Meeting #107	May 22 – May 26	Incheon, Korea