3GPP TSG-RAN WG4 Meeting#106                               R4-2301724

Athens, Greece, Feb 27 – Mar 3,2023

Agenda item:	8.30.2

Source: 	ZTE Corporation

Title: 	Discussion on the impact of APT60MHz on TS38.115-2

Document for:	Approval

Introduction

In RAN#98e meeting, the revised WID for APT600MHz has been approved to include more the impacted spec from the coexistence requirement perspective in RAN4. In this contribution, we want to share some further clarifications on the impacts on TS38.115-2 and suggestions on how to move forward in APT600MHz WID.

Discussion

Regarding the scope of TS38.115-2 spec, it is targeted for FR2 repeater only in Rel-17 and there are no additional spurious emission requirements and co-location with other base stations requirement between FR1 bands and FR2 bands. From this perspective, there are no spec impact on top of latest Rel-17 TS38.115-2.

Observation 1: there are no spec impact on the top of latest Rel-17 TS38.115-2.

However for Rel-18 NCR (network controlled repeater), it was agreed to introduce the beamforming capability for FR1 NCR in last RAN4#105 meeting, namely to introduce the FR1 repeater 1-H and 1-O and its corresponding requirements of FR1 repeater type 1-O would be specified in TS38.115-2. In other words, the additional spurious emission requirements and co-location with other base stations requirement for FR1 repeater type 1-O would be specified in TS38.115-2 under the Rel-18 NCR WID, therefore it’s still necessary to specify the corresponding requirement for new APT_600MHz band with FR1 repeater type 1-O in Rel-18. Unfortunately the perf parf of Rel-18 NCR will be finalized at the June, 2024 which is much later than target date March, 2023 of APT600MHz. Based on the situation and to avoid inter-relations for the pending Rel-18 work, it is proposed to consider the additional spurious emission requirements and co-location with other base stations requirement for APT600MHz with repeater type 1-O in TS38.115-2 in the NCR WID instead of APT600MHz WID.

Observation 2: it’s still necessary to specify the additional spurious emission requirements and co-location with other base stations requirement for APT_600MHz band with FR1 repeater type 1-O in Rel-18 TS38.115-2.

Proposal 1: to consider the additional spurious emission requirements and co-location with other base stations requirement for APT600MHz with repeater type 1-O in TS38.115-2 in the NCR WID instead of APT600MHz WID.

Conclusions

In this contribution, we want to share some further clarifications on the impacts on TS38.115-2 and proposal is made as following:

Observation 1: there are no spec impact on the top of latest Rel-17 TS38.115-2.

Observation 2: it’s still necessary to specify the additional spurious emission requirements and co-location with other base stations requirement for APT_600MHz band with FR1 repeater type 1-O in Rel-18 TS38.115-2.

Proposal 1: to consider the additional spurious emission requirements and co-location with other base stations requirement for APT600MHz with repeater type 1-O in TS38.115-2 in the NCR WID instead of APT600MHz WID.

References

- RP-223265,Revised WID on APT 600 MHz NR band, ZTE corporation, Approved.