3GPP TSG-RAN WG4 Meeting #106                               R4-2301755

Athens, Greece, February 27 – March 3, 2022

Agenda item:	8.34.4

Source: 	ZTE Corporation

Title: 	Discussion on SAN RF requirements for the satellite L-/S-band

Document for:	Approval

Introduction

In RAN#98e meeting, the work item [RP-223485] on Introduction of the satellite L-/S-band was approved. Part of the objective is as follows:

-	Specify a new NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz;

-	Support channel bandwidths and SCS as presented in Table 4.1-1 below;

-	Support channel raster points at step of 100kHz;

NOTE: Support for channel raster points at step of 5kHz can be considered later based on the outcome of the corresponding RAN WG4 for potential channel raster enhancements.

-	Support UE Power class 3 (+23dBm);

NOTE: Work on UE Power class 2 can commence based on the RAN#99 decision.

-	Introduce the corresponding SAN and UE RF core requirements;

-	Introduce the corresponding RRM requirements.

Table 4.1-1: Channel bandwidth and SCS system parameters.

| SCS (kHz) | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- |
| 15 | 5 | 10 | 15 |  |
| 30 |  | 10 | 15 |  |
| 60 |  | 10 | 15 |  |

- In this contribution, we want to share some initial views on SAN RF requirements for Introduction of the satellite L-/S-band.

Discussion

## 2.1. Operating bands

The new NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz should be numbered as n254 since that the satellite bands are numbered in descending order from n256.

Table 2.1-1: Satellite operating bands in FR1

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n254 | 1610 MHz-1626.5MHz | 2483.5MHz-2500MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from n256. |  |  |  |

Proposal 1: for the NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz should be numbered as Table 2.1-1.

## 2.2 SAN channel bandwidth per operating band

According to the work item [RP-223485] on Introduction of the satellite L-/S-band, the support channel bandwidths and SCS of band n254 can be defined as Table 2.2-1.

Table 2.2-1: SAN channel bandwidths and SCS per operating band in FR1

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 |
|  | 15 | 5 | 10 | 15 |  |
| n254 | 30 |  | 10 | 15 |  |
|  | 60 |  | 10 | 15 |  |

Proposal 2: for the NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz should be defined the SAN channel bandwidth per operating band as Table 2.2-1.

## 2.3 Channel raster entries for each operating band

According to the work item [RP-223485], the granularity of the channel raster is 100kHz and the granularity of the global frequency raster is 5kHz. Therefore, the NR-ARFCN can be defined as Table 2.3-1.

Table 2.3-1: Applicable NR-ARFCN per operating band in FR1

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n254 | 100 | 322000 – <20> – 325300 | 496700 – <20> – 500000 |

Proposal 3: for the NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz should be defined the NR-ARFCN as Table 2.3-1.

## 2.4 Summary

Based on the above, the summary of the corrections on TS38.108 are given in Table 2.4-1.

Table 2.4-1. Summary of related RF requirement

| General part |  |
| --- | --- |
| Operating bands | Add the new band number n254. |
| SAN channel bandwidth | Channel bandwidths of 5,10 and 15 MHz with supported subcarrier spacing of 15/30/60 kHz. |
| Channel arrangement | Add NR-ARFCN and sync raster entries for the new band n254. The sync raster entries should be added after the SS Block pattern is defined. |
| Tx RF requirements part |  |
| SAN output power | All of these requirements are defined as band or channel bandwidth agnostic except for output power dynamics, therefore it’s not expected to have any impacts with the introduction of new band n254. |
| Output power dynamics |  |
| Transmit ON/OFF power |  |
| Transmitted signal quality |  |
| OBW |  |
| Tx intermodulation |  |
| ACLR | No requirement impacts |
| Operating band unwanted emissions | No requirement impacts |
| Transmitter spurious emissions | No requirement impacts |
| Rx RF requirements part |  |
| REFSENS | No requirements impacts. |
| Dynamic range requirement | No requirement impacts. |
| ACS | No requirement impacts. |
| Blocking requirement | No requirement impacts. |
| OOBB | No requirement impacts. |
| Rx spurious emission | No requirement impacts. |
| RX intermodulation | No requirement impacts. |
| ICS | No requirement impacts. |

Conclusions

In this contribution, we want to share some further views on SAN RF requirements for sIntroduction of the satellite L-/S-band. Some proposals are made as following:

Proposal 1: for the NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz should be numbered as Table 2.1-1.

Proposal 2: for the NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz should be defined the SAN channel bandwidth per operating band as Table 2.2-1.

Proposal 3: for the NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz should be defined the NR-ARFCN as Table 2.3-1.

References

- RP-223485, “New WID on Introduction of the satellite L-/S-ban”, Globalstar, Apple.