3GPP TSG-RAN WG4 Meeting #106-e                                                    	R4-2301768

Electronic Meeting, 15– 26 August, 2022

Source:	Huawei, Hisilicon

Title:	TP for TR 37.718-21-11 DC_3A-7A-8A_n7A

Agenda item:			8.4.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.718-21-11 to include DC_3A-7A-8A_n7A with UL configuration DC_3A-n7A, DC_7A-n7A and DC_8A-n7A.

2. Text Proposal

<Start of Text Proposal>

5.x	DC_3-7-8_n7

5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink configuration |
| --- | --- |
| DC_3A-7A-8A_n7A | DC_3A_n7A DC_7A_n7A DC_8A_n7A |

5.x.2	Co-existence studies

For UE coexistence study of Band 3 + Band n7, it has been done with one of the fallback mode DC_3A-8A_n7A. Similarly, the coexistence study of Band 8 + Band n7 has been done with another fallback mode DC_7A-8A_n7A. They can all be referred to this TR 37.718-21-11.

As for the UL configuration of DC_7A_n7A, only single UL is required. Thus no IMD issue needs to be evaluated. Besides, there is no harmonic issue from DC_7A_n7A to Band 3 and Band 8 DL reception.

5.x.3	∆TIB and ∆RIB values

For DC_3-7-8_n7, the TIB,c and RIB,c values are given in the following tables. (Both are reusing the specified values for DC_3-7_n8 as in TS 38.101-3)

Table 5.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-8_n7 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 8 | 0.6 |
|  | n7 | 0.5 |

Table 5.x.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-8_n7 | 8 | 0.2 |

5.x.4	Reference sensitivity exceptions

Compared to its fallback modes, no additional MSD requirements for this band combination are needed.

<End of Text Proposal>