3GPP TSG-RAN WG4 Meeting #106		R4-2301774

Athens, Greece, 27 February – 3 March 2023

Agenda Item:	10.5.7.1

Source:	Ericsson

Title:	Simulation results of eMTC/NB-IoT UE demodulation requirements for NTN

Document for:	Information

# 1	Introduction

RAN4#105 agreed with the simulation assumption for NB-IoT/eMTC UE demodulation requirements for NTN [1]. This contribution provides our initial simulation results according to the simulation assumption.

# 2	Simulation results

## 2.1	PDSCH for UE Cat-M1

### 2.1.1	Simulation parameters

Table 1 shows the common parameters for PDSCH demodulation requirements, and Table 2 shows the test parameters for each test case. Table 3 shows the FRCs used in our simulation. Since we agreed to use TM1 for all the test cases, the channel bits become slight larger compared with the exiting FRCs (R.79/80/81 FDD) used for Cat-M1, because these FRCs assume 2 CRS ports. As it is observed from the assumption, we can set the same TBS to keep the same target coding rate; this means the effective code rate is slight lower compared with the existing Cat-M1 UE requirements.

Table 1	Common parameters for PDSCH demodulation requirements.

| Parameters | Value |
| --- | --- |
| CBW | FDD 1.4MHz |
| Transmission mode | TM1 |
| Antenna configuration | 1x1 |
| K_offset | 8ms |

Table 2	Test parameters for PDSCH demodulation requirements.

| Test number | MCS | PDSCH repetition | Propagation condition |
| --- | --- | --- | --- |
| 1 | 16QAM 1/2 | 1 | NTN-TDLC5-30 |
| 2 | QPSK 1/3 | 8 | NTN-TDLA100-X Option 1: X=200Hz Option 2: X=10Hz |
| 3 | QPSK 1/10 | 64 | NTN-TDLA100-10 |

Table 3	FRCs used for Cat-M1 PDSCH simulation.

| MCS | Number of RBs | Channel bits (bits) | TBS (bits) |
| --- | --- | --- | --- |
| 16QAM 1/2 | 3 | 1656 (cf. R.79 FDD: 1584) | 744 |
| QPSK 1/3 | 6 | 1656 (cf. R.80 FDD: 1440) | 504 |
| QPSK 1/10 | 6 | 1656 (cf. R.81 FDD: 1584) | 152 |
| Note: CFI=2, 1 CRS port, no DRMS |  |  |  |

### 2.1.2	Simulation results

Table 4 summarizes the SNR (dB) to achieve 70% of the maximum throughput.

Table 4	Summary of Cat-M1 UE PDSCH demodulation simulation results

| Test case | SNR (dB) to achieve 70% of maximum throughput |
| --- | --- |
| Test 1 | 6.7 |
| Test 2 (Option 1, X=200Hz) | -6.3 |
| Test 2 (Option 2, X=10Hz) | -8.1 |
| Test 3 | -17.8 |

## 2.2	NPDSCH for UE Cat-NB1/NB2

### 2.2.1	Simulation parameters

Table 5 shows the common parameters for NPDSCH demodulation requirements and Table 6 shows the test parameters for each test case. Table 7 shows the FRCs used in our simulation.

Table 5	Common parameters for NPDSCH demodulation requirements.

| Parameters | Value |
| --- | --- |
| CBW | FDD 200kHz |
| Operation mode | Stand-alone |
| Antenna configuration | 1x1 |
| K_offset | 8ms |

Table 6	Test parameters for NPDSCH demodulation requirements

| Test number | MCS | PDSCH repetition | Propagation condition | Carrier type |
| --- | --- | --- | --- | --- |
| 1 | QPSK 1/2 | 32 | NTN-TDLC5-X Option 1: X=200Hz Option 2: X=30Hz | Anchor |
| 2 | QPSK 1/3 | 256 | NTN-TDLA100-10 | Non-anchor |

Table 7	FRCs used for Cat-NB1/NB2 NPDSCH simulation.

| MCS | ITBS / ISF | Channel bits (bits) per SF | TBS (bits) |
| --- | --- | --- | --- |
| QPSK 1/2 (R.NB.6 FDD) | 9 / 3 | 320 | 616 |
| QPSK 1/3 (R.NB.6-1 FDD) | 6 / 3 | 320 | 392 |

### 2.1.2	Simulation results

Table 8 summarizes the SNR (dB) to achieve 70% of the maximum throughput.

Table 8	Summary of Cat-NB1/NB2 UE NPDSCH demodulation simulation results

| Test case | SNR (dB) to achieve 70% of maximum throughput |
| --- | --- |
| Test 1 (Option 1, X=200Hz) | -4.1 |
| Test 1 (Option 2, X=30Hz) | -6.6 |
| Test 2 | -14.8 |

# References

- R4-2220278, “WF on IoT-NTN UE demodulation and CQI reporting requirements”, MediaTek.