3GPP TSG-RAN WG4 Meeting #106	R4-2301890

Athens, Greece, February 27 – March 3, 2023

Agenda Item:	6.2.6.1.3

Source:	Ericsson

Title:	CSI reporting requirements in 52.6 – 71 GHz band

Document for:	Discussion

# 1	Introduction

In this paper, we present our conclusions on the CQI reporting requirements in 52.5 GHz – 71 GHz band, where some pending requirements’ definition have been gathered in the WF [1].

Rank to use for CQI Reporting requirements; - Agreement: Rank 1; SNR Test point for high modulation order - Agreement: [7/8] dB, companies can further check; SNR Test point for low modulation order - Agreement: 0/1 dB;

# 2	CSI Reporting Requirements

Based on the latest agreements, we only need to check the SNR test points for high modulation orders. Consequently, and based on our simulation results given below, we provide Proposal 1.

(a) CQI reporting under PN Set 1                   (b) CQI reporting under PN Set 2

Fig. 1 - Simulation results depicting the median CQI as function of SNR using 120 KHz/100 MHz (66 PRBs) Rank 1 under PN effects.

Proposal 1: Define the CQI reporting definition test for 2Rx UE with CQI table 1 (64QAM), see Appendix A.1, by reusing the existing test setup and metrics for SCS 120 kHz/100 MHz with Rank 1.

- Consider PN with CPE compensation only.

- Tx EVM = 6%.

- Test points:

- SNR = 7/8 dB for high modulation order.

- SNR = 0/1 dB for low modulation order.

# 4	Summary

In this paper, we tried to contribute with the following proposal for further discussions:

Proposal 1: Define the CQI reporting definition test for 2Rx UE with CQI table 1 (64QAM), see Appendix A.1, by reusing the existing test setup and metrics for SCS 120 kHz/100 MHz with Rank 1.

- Consider PN with CPE compensation only.

- Tx EVM = 6%.

- Test points:

- SNR = 7/8 dB for high modulation order.

- SNR = 0/1 dB for low modulation order.

# References

- R4-2220187, “WF on FR2-2 UE demodulation requirements”, Qualcomm.

# Appendix

## A.1	CQI Tables

We consider Table A.4-1 (TS 38.101-4) mapping CQI index to the information bit payload, given by

Table A.4-1: Mapping of CQI Index to Information Bit payload (CQI table 1)

| TBS Scheme |  |  |  | TBS.1-1 | TBS.1-2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS table |  |  |  | 64QAM |  |  |  |  |  |
| Number of allocated PDSCH resource blocks |  |  |  | 66 | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 |  |  |  |  |
| Number of PDSCH MIMO layers |  |  |  | 1 | 2 |  |  |  |  |
| Number of DMRS REs (Note 1) |  |  |  | 24 | 24 |  |  |  |  |
| Overhead for TBS determination |  |  |  | 6 | 6 |  |  |  |  |
| Available RE-s |  |  |  | 7590 | 7590 |  |  |  |  |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  |  |  |  |  |
| 0 | OOR | OOR | OOR | N/A | N/A |  |  |  |  |
| 1 | 0.2344 | 0 | QPSK | 1800 | 3624 |  |  |  |  |
| 2 | 0.2344 | 0 |  | 1800 | 3624 |  |  |  |  |
| 3 | 0.3770 | 2 |  | 2856 | 5640 |  |  |  |  |
| 4 | 0.6016 | 4 |  | 4480 | 8968 |  |  |  |  |
| 5 | 0.8770 | 6 |  | 6528 | 13064 |  |  |  |  |
| 6 | 1.1758 | 8 |  | 8712 | 17928 |  |  |  |  |
| 7 | 1.4766 | 11 | 16QAM | 11016 | 22032 |  |  |  |  |
| 8 | 1.9141 | 13 |  | 14343 | 28680 |  |  |  |  |
| 9 | 2.4063 | 15 |  | 17928 | 35856 |  |  |  |  |
| 10 | 2.7305 | 18 | 64QAM | 20496 | 40976 |  |  |  |  |
| 11 | 3.3223 | 20 |  | 25104 | 50184 |  |  |  |  |
| 12 | 3.9023 | 22 |  | 29192 | 58384 |  |  |  |  |
| 13 | 4.5234 | 24 |  | 33816 | 67584 |  |  |  |  |
| 14 | 5.1152 | 26 |  | 38936 | 77896 |  |  |  |  |
| 15 | 5.5547 | 28 |  | 42016 | 83976 |  |  |  |  |

where the CQI indexes can be mapped to the corresponding MCS as given by Table 5.1.3.1-1 (TS 38.214)

| CQI Index | MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- | --- |
| 1 - 2 | 0 | 2 | 120 | 0.2344 |
| 3 | 2 | 2 | 193 | 0.3770 |
| 4 | 4 | 2 | 308 | 0.6016 |
| 5 | 6 | 2 | 449 | 0.8770 |
| 6 | 8 | 2 | 602 | 1.1758 |
| 7 | 11 | 4 | 378 | 1.4766 |
| 8 | 13 | 4 | 490 | 1.9141 |
| 9 | 15 | 4 | 616 | 2.4063 |
| 10 | 18 | 6 | 466 | 2.7305 |
| 11 | 20 | 6 | 567 | 3.3223 |
| 12 | 22 | 6 | 666 | 3.9023 |
| 13 | 24 | 6 | 772 | 4.5234 |
| 14 | 26 | 6 | 873 | 5.1152 |
| 15 | 28 | 6 | 948 | 5.5547 |

## A.2	CQI reporting in static condition

### A.2.1	Test metric

- The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

- If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

| For FR2-2 Parameter |  |  | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 100 |  |  |  |
| Subcarrier spacing |  |  | kHz | 120 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| TDD Slot Configuration |  |  |  | FR2.120-2 Annex A.1.3 |  |  |  |
| SNRBB |  |  | dB | 0 | 1 | 7 | 8 |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 2x2 with static channel specified in Annex B.1 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 8 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |  |  |  |
|  | CSI-RS periodicity and offset |  | Slot | 8/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | fd-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 6 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 8/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 1 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (8, 13) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 8/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 1 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 8 |  |  |  |
| csi-ReportingBand |  |  |  | 111111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 8/3 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type | typeI-SinglePanel | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 8.375 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-1, TBS.1-1 |  |  |  |