3GPP TSG-RAN WG4 Meeting #106                                R4-2302090

Athens, Greece, 27 February– 3 March, 2023

Title:	Replied LS on applicability of requirements for RedCap UE

Response to:	R5-228034

Release:	Release 17

Work Item:	NR_redcap_plus_ARCH-UEConTest

Source:	TSG RAN WG4

To:	TSG RAN WG5

Cc:

Contact person:	Peng, Zhang

Zhangpeng169@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- 1	Overall description

RAN4 thanks RAN5 for raising these questions. After discussion, RAN4 would like to clarify applicability of Rx requirements for RedCap UE as below.

According to the current specification TS 38.101-1, there is a general description to clarify how to verify the Receiver characteristics for RedCap UE as below.

- 7.1I	General

For a Redcap UE the requirements in Section 7 shall be verified with the channel bandwidth up to 20MHz and REFSENS specified in clause 7.3I.

Since clause 7.3C belongs to Section 7, for a RedCap UE indicating SUL band combinations, the requirements in clause 7.3C in 38.101-1 shall be verified with the channel bandwidth up to 20MHz and REFSENS specified in clause 7.3I.

For some specific details, it’s recommended to follow this general principle as below when specifying conformance test cases for RedCap UE indicating SUL band combinations.

1) Reference sensitivity side conditions (UL/DL configuration) specified in clause 7.3C should be considered under the conditions that channel bandwidth up to 20MHz and REFSENS specified in clause 7.3I.

2) Sensitivity allowance specified in Table 7.3C.2-2 and Table 7.3C.2-4 should be considered under the conditions that channel bandwidth up to 20MHz and REFSENS specified in clause 7.3I.

3) SUL band combination with HD-FDD band should meet the requirements specified in clause 7.3C under the conditions that channel bandwidth up to 20MHz and REFSENS specified in clause 7.3I for corresponding HD-FDD band.

- 2	Actions

To TSG RAN WG5

ACTION: 	RAN4 would like to respectfully request RAN5 to take the clarifications into account as above in future meetings.

- 3	Dates of next TSG-RAN WG4 meetings

TSG-RAN4 Meeting#106-bis-e 	   17 – 26 April 2023	e-Meeting

TSG-RAN4 Meeting#107 	      	22 – 26 May 2023	Incheon, KR

- 4	References