3GPP TSG-RAN WG4 Meeting #106                                R4-2302099

Athens, Greece, 27 February– 3 March, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on MSD evaluation and deltaTibRib for CA_n8-n20-n28

Agenda Item:	9.2.4.2

Document for:	Approval

Introduction

In last meeting, WF [1] on CA_n8-n20-n28 was approved. The following way forwards and agreements were captured.

Agreement:

- Use 3 antenna UE architecture as the baseline for evaluation.

  - Do not preclude 2 and 4 antennas in the study item

Given that three antenna architecture is still the baseline:

Way forward:

- Provide MSD analysis for the TR (Technical Report)

- Assuming it is the worst case, analysis is prioritized for 3 antenna architecture based on:

  - n28UL/n20+n28DL/n20UL triplexer on antenna 1

  - n8UL/n8DL duplexer on antenna 2

  - n20+n28DL/n8DL duplexer on antenna 3

  - For comparison purpose, analysis of alternate two or three antenna solutions are not precluded, and if a worse case is found it can be discussed whether it should be used for the MSD feasibility.

- 10db antenna isolation is used for MSD calculations

- The normal Tx/Rx 50dB isolation is used between UL and DL of the same FDD bands for duplexer/triplexer/quadplexer. If higher insertion loss is needed to meet the 50dB it can be accounted in DeltaT/R

- For different FDD bands within triplexer/quadplexer companies are encouraged to provide feasible isolation between ULs and DLs

- For diversity path, the DL filter provide at least 40dB of Rejection of the UL of the same band and companies are encouraged to provide feasible rejection for UL of other bands.

Companies are encouraged to provide input on the feasibility of such values

In this paper, we’d like to provide our technical analysis about MSD evaluation and deltaTibRib for CA_n8-n20-n28.

Discussion on MSD evaluation for CA_n8-n20-n28

The component linearity assumptions are listed in table 1, and the component attenuation and isolation values are listed in table 2.

Table 1 General linearity parameters

| Component | IP3(dBm) |
| --- | --- |
| Antenna switch | 68 |
| Diplexer | 86 |
| Triplexer/Duplexer | 74 |
| PA forward mixing | 30 |
| PA reverse mixing | 28 |
| LNA | -6 |

Table 2 Attenuation and isolation values

| Attenuation and Isolation Parameter | Value (dB) | Comment |
| --- | --- | --- |
| Antenna ISO | 10 |  |
| PA gain | 25 |  |
| PCB isolation Paout-Pain | 60 | PCB isolation (PA forward mixing) |
| n28 Tx attenuation at Band n8 Rx | 27 |  |
| n28 Rx attenuation at Band n8 Tx | 25 |  |
| n20 Tx attenuation at n8 RX | 43 |  |
| n20 Tx attenuation at n8 TX | 46 |  |
| n8 Tx attenuation at n20 TX | 30 |  |
| n8 Tx attenuation at n28 RX | 30 |  |
| The Triplexer isolation between band 20 Tx and band n28A Tx | 50 |  |
| The Triplexer isolation between band n28A/20 Rx and (band n28A Tx or band n20 Tx) | 50 |  |

Table 3 and Table 4 summarize the MSD analysis results for DL_n8A-n20A-n28A_UL_n20A-n28A and DL_n8A-n20A-n28A_UL_n8A-n20A.

Table 3 MSD calculation for victim Rx of band 8 when UL_n20A-n28A

| Component | IMD3(dBm) |  |
| --- | --- | --- |
|  | Main_@Antenna | Div_@Antenna |
| n28 PA Forward mixing interference | -98.4 | -98.4 |
| B20 PA Forward mixing interference | -78.2 | -78.2 |
| n28 PA Reserve mixing interference | -122 | -122 |
| B20 PA Reserve mixing interference | -86.5 | -86.5 |
| n28 Triplexer | -195 | -195 |
| B20 Triplexer | -140 | -140 |
| B8 LNA via filters | -77 | -77 |
| B8 LNA via PCB coupling | -86 | -86 |
| Diplexer (L-H) | -121 | -121 |
| Antenna switch | -83 | -83 |
| Total | -73.47 | -73.47 |

Table 4 MSD calculation for victim Rx of band 28 when UL_n8A-n20A

| Component | IMD3(dBm) |  |
| --- | --- | --- |
|  | Main_@Antenna | Div_@Antenna |
| n8 PA Forward mixing interference | -103 |  |
| n20 PA Forward mixing interference | -69.5 |  |
| n8 PA Reserve mixing interference | -124 |  |
| n20 PA Reserve mixing interference | -94 |  |
| n20 Triplexer | -126 |  |
| n8 Duplexer | -206 |  |
| n28 LNA via filters | -75 | -103.5 |
| n28 LNA via PCB coupling | -92 | -92 |
| Diplexer (L-H) | -121.5 | -142 |
| Antenna switch | -84 | -135.5 |
| From main path |  | -78.3 |
| Total | -68.3 | -78.1 |

Proposal 1: It’s proposed to specify band n8 MSD due to IMD3 of Tx band n20 + band n28 and band n28 MSD due to IMD3 of Tx band n8 + band n20.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n8-n20-n28 | n20 | 834.5 | 5 | 25 | 793.5 | N/A | FDD | N/A |
|  | n28 | 715.5 | 5 | 25 | 770.5 | N/A | FDD | N/A |
|  | n8 | 908.5 | 5 | 25 | 953.5 | 23.5 | FDD | IMD3 |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | FDD | N/A |
|  | n20 | 834.5 | 5 | 25 | 793.5 | N/A | FDD | N/A |
|  | n28 | 726.5 | 5 | 25 | 781.5 | 23.0 | FDD | IMD3 |

Discussion on Delta TibRib for CA_n8-n20-n28

Referring to clause 5.67.3 from TR 36.715-03-01 [2], ΔTIB,c values for CA_8-20-28 are derived from existing combination CA_8A-20A and CA_20A-28A and ΔRIB,c for CA_8-20-28 values are derived from existing combination CA_8A-20A and CA_20A-28A.

In addition, in the contribution [3], the simulation results related to additional insertion loss were provided by some vendors. And the following formulas were proposed to decide the final ΔTIB,c and ΔRIB,c.

∆TIB = (Average Tx)/2 + 0.1

∆RIB = (Average Rx – 0.6)/2

So the following values were proposed in the contribution [3].

Table 5: TIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_8A-28A | 8 | 0.6 = (0.93/2)+0.1 |
|  | 28 | 0.5 = (0.73/2)+0.1 |

Table 6: RIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_8A-28A | 8 | 0.2 = (0.97-0.6)/2 |
|  | 28 | 0.1 = (0.8-0.6)/2 |

Since we have agreed to use 3 antennas UE architecture as the baseline for evaluation and RF front end of CA_n8-n28 and CA_n20-n28 are reused for 3 antennas UE architecture of CA_n8-n20-n28, the following ΔTIB,c and ΔRIB,c values are derived from the existing CA_n8-n28 and CA_n20-n28.

Proposal 2: It’s proposed to specify ΔTIB,c and ΔRIB,c values for CA_n8-n20-n28 as below based on 3 antennas UE architecture.

Table 7: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n8-n20-n28 | n8 | 0.6 |
|  | n20 | 0.5 |
|  | n28 | 0.5 |

Table 8: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n8-n20-n28 | n8 | 0.2 |
|  | n20 | 0 |
|  | n28 | 0.1 |

Summary

Proposal 1: It’s proposed to specify band n8 MSD due to IMD3 of Tx band n20 + band n28 and band n28 MSD due to IMD3 of Tx band n8 + band n20.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n8-n20-n28 | n20 | 834.5 | 5 | 25 | 793.5 | N/A | FDD | N/A |
|  | n28 | 715.5 | 5 | 25 | 770.5 | N/A | FDD | N/A |
|  | n8 | 908.5 | 5 | 25 | 953.5 | 23.5 | FDD | IMD3 |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | FDD | N/A |
|  | n20 | 834.5 | 5 | 25 | 793.5 | N/A | FDD | N/A |
|  | n28 | 726.5 | 5 | 25 | 781.5 | 23.0 | FDD | IMD3 |

Proposal 2: It’s proposed to specify ΔTIB,c and ΔRIB,c values for CA_n8-n20-n28 as below based on 3 antennas UE architecture.

| Inter-band CA Configuration | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n8-n20-n28 | n8 | 0.6 |
|  | n20 | 0.5 |
|  | n28 | 0.5 |

| Inter-band CA Configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n8-n20-n28 | n8 | 0.2 |
|  | n20 | 0 |
|  | n28 | 0.1 |

- References

[1] R4-2220809, WF on CA_n8-n20-n28, Murata Manufacturing Co, Ltd, Skyworks Solutions

[2] TR 36.715-03-01.

[3] R4-162926, TP for TR36.714-02-01: Finalization of B8+B28 (2DL/1UL), SoftBank Corp.