3GPP TSG-RAN WG4 Meeting #106                                											R4-2302103

Athens, Greece, 27 February– 3 March, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on REFSENS test burden reduction

Agenda Item:	9.3.3

Document for:	Approval

Introduction

In RAN4#104 meeting, a contribution [1] clarified the similarity and dependency of Tx/Rx RF requirements for different features. For MSD due to IMD for two bands, proposal was shown below.

Proposal 4: Since ENDC combos DL_A_nB_UL_A_nB and DL_B_nA_UL_B_nA can share the same mechanism and principles with NR CA band combination DL_nA-nB_UL_nA-nB which have MSD requirements due to two bands IMD interference, it’s recommended to test one of them in order to reduce the test burden.

Since this proposal is too general, companies provided comments with specific cases during the discussion. In this meeting, we’d like to check and compare the Table 7.3A.5-1 of TS 38.101-1 and Table 7.3B.2.3.5.1-1 of TS 38.101-3 to further clarify the similarity and dependency of this kind of RF requirements for different features. In this paper, we’d like to propose a solution to reduce the test burden for MSD requirements due to IMD (two bands).

Discussion on REFSENS exception due to inter-modulation distortion for inter-band combinations (two bands) for PC3

For REFSENS exception due to inter-modulation distortion for inter-band combinations (two bands) for PC3, we summarized PC3 MSD requirements for ENDC and NR CA as below based on the latest specification TS 38.101-1-i00 and TS 38.101-3-i00. For each row, the band combinations have the same frequency range for each band and same RF implementation in spite of different features (EN-DC or NR CA). Except for those highlighted band combinations, most of band combination have the same MSD requirements due to IMD interference, no matter whether ENDC or NR CA is supported. As same RF implementation can be reused for one band combination with different features, e.g. CA_n1-n3/DC_1_n3/DC_3_n1, there is no need to test the MSD due to IMD interference for every individual band combination in the same row of table 1. It’s proposed to choose one band combination for the requirements for each row. It’s recommended to choose one band combination randomly for MOP testing.

Proposal 1: The following text proposals are provided to be captured into TR 38.846 based on the analysis.

MSD due to IMD for NR CA, NR DC, EN-DC and NE-DC band combinations with two bands are shown in table 1 based on the TS 38.101-1-i00 and TS 38.101-3-i00. One band combination can be chosen to verify the requirements for some band combinations in same row in table 1, as a result of reducing test burden. It’s suggested to randomly choose one band combination that UE support for MSD testing. The proposed test reduction could be considered for an informative annex in the TS since the final decision should be taken by RAN5 based on the industry certification testing needs.

Table 1 Band combination with different features in the same frequency range for PC3 MSD due to IMD interference

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  | Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order | NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1_n3 | 1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 | CA_n1-n3 | n1 | 1950 | 5 | 25 | 2140 | 23 | FDD | IMD3 |
|  | n3 | 1760 | 5 | 25 | 1855 | N/A | N/A |  | n3 | 1760 | 5 | 25 | 1855 | N/A | TDD | N/A |
| DC_1A_n8A | 1 | 1965 | 5 | 25 | 2155 | 6.0 | IMD4 | CA_n1-n8 | n1 | 1965 | 5 | 25 | 2155 | 6.0 | FDD | IMD4 |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | N/A |  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | FDD | N/A |
| DC_1A_n77A, DC_1A_SUL_n77A-n84A, DC_1A_n77(2A), | 1 | 1950 | 5 | 25 | 2140 | 29.8 | IMD23 | CA_n1-n77 | 1 | 1950 | 5 | 25 | 2140 | 29.8 | FDD | IMD24 |
|  | n77 | 4090 | 10 | 50 | 4090 | N/A | N/A |  | n77 | 4090 | 10 | 50 | 4090 | N/A | TDD | N/A |
| DC_1A_n77A, DC_1A_SUL_n77A-n84A, DC_1A_n77(2A), DC_1A_n77(3A), DC_1A_n78A, DC_1A_SUL_n78A-n84A, DC_1A_n78(2A) DC_1A_n78(A-C) | 1 | 1950 | 5 | 25 | 2140 | 8.0 | IMD43 | CA_n1-n77 | 1 | 1950 | 5 | 25 | 2140 | 8.0 | FDD | IMD44 |
|  | n77, n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
| DC_2A_n48A | 2 | 1852.5 | 5 | 25 | 1932.5 | 12 | IMD4 | CA_n2-n48 | n2 | 1852.5 | 5 | 25 | 1932.5 | 12 | FDD | IMD4 |
|  | n48 | 3625 | 20 | 100 | 3625 | N/A | N/A |  | n48 | 3625 | 20 | 100 | 3625 | N/A | TDD | N/A |
| DC_2A_n66A, DC_2A-2A_n66A DC_2A_n66(2A) | 2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 | CA_n2-n66 | n2 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |  | n66 | 1775 | 5 | 25 | 2175 | N/A | FDD | N/A |
| DC_2A_n66A, DC_2A-2A_n66A DC_2A_n66(2A) | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A | CA_n2-n66 | n2 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |  | n66 | 1750 | 5 | 25 | 2150 | 4 | FDD | IMD5 |
| DC_2A_n77A DC_2A_n77(2A) DC_2A-2A_n77A DC_2A_n77(2A) DC_2A-2A_n77(2A) | 2 | 1855 | 5 | 25 | 1935 | 26 | IMD2 | CA_n2-n77 | n2 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | N/A |  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | 2 | 1900 | 5 | 25 | 1980 | 8.0 | IMD4 | CA_n2-n77 | n2 | 1900 | 5 | 25 | 1980 | 8.0 | FDD | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | 2 | 1885 | 5 | 25 | 1965 | 5 | IMD5 | CA_n2-n77 | n2 | 1885 | 5 | 25 | 1965 | 5 | FDD | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | N/A |  | n77 | 3810 | 10 | 50 | 3810 | N/A | TDD | N/A |
| DC_2A_n78A DC_2A_n78(2A) DC_2A-2A_n78(2A) | 2 | 1855 | 5 | 25 | 1935 | 26 | IMD23 | CA_n2-n78 | n2 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD24 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| DC_3_n1 | 3 | 1760 | 5 | 25 | 1855 | N/A | N/A | CA_n1-n3 | n1 | 1950 | 5 | 25 | 2140 | 23 | FDD | IMD3 |
|  | n1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |  | n3 | 1760 | 5 | 25 | 1855 | N/A | TDD | N/A |
| DC_3_n5 | 3 | 1771 | 10 | 50 | 1866 | 4 | IMD4 | CA_n3-n5 | n3 | 1771 | 10 | 50 | 1866 | 4 | FDD | IMD4 |
|  | n5 | 838 | 5 | 25 | 883 | N/A | N/A |  | n5 | 838 | 5 | 25 | 883 | N/A | FDD | N/A |
|  | 3 | 1721 | 10 | 50 | 1816 | N/A | N/A | CA_n3-n5 | n3 | 1721 | 10 | 50 | 1816 | N/A | FDD | N/A |
|  | n5 | 838 | 5 | 25 | 883 | 24 | IMD23 |  | n5 | 838 | 5 | 25 | 883 | 24 | FDD | IMD23 |
| DC_3A_n7A DC_3C_n7A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A | CA_n3-n7 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n7 | 2535 | 10 | 50 | 2655 | 10.2 | IMD4 |  | n7 | 2535 | 10 | 50 | 2655 | 10.2 | FDD | IMD4 |
| DC_3_n8 | n8 | 900 | 5 | 25 | 945 | 8 | IMD43 | CA_n3-n8 | n3 | 1755 | 10 | 50 | 1850 | N/A | FDD | N/A |
|  | 3 | 1755 | 10 | 50 | 1850 | N/A | N/A |  | n8 | 900 | 5 | 25 | 945 | 8 | FDD | IMD44 |
|  | n8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A | CA_n3-n8 | n3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | FDD | IMD5 |
|  | 3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | IMD5 |  | n8 | 897.5 | 5 | 25 | 942.5 | N/A | FDD | N/A |
| DC_3A_n26A | 3 | 1771 | 10 | 50 | 1866 | 4 | IMD4 | CA_n3-n26 | n3 | 1771 | 5 | 25 | 1866 | 4 | FDD | IMD4 |
|  | n26 | 838 | 5 | 25 | 883 | N/A | N/A |  | n26 | 838 | 5 | 25 | 883 | N/A | FDD | N/A |
|  | 3 | 1721 | 10 | 50 | 1816 | N/A | N/A | CA_n3-n26 | n3 | 1721 | 5 | 25 | 1816 | N/A | FDD | N/A |
|  | n26 | 838 | 5 | 25 | 883 | 24 | IMD23 |  | n26 | 838 | 5 | 25 | 883 | 26 | FDD | IMD211 |
| DC_3A_n38A | 3 | 1712.8 | 5 | 25 | 1807.8 | 8.2 | IMD4 | CA_n3-n38 | n3 | 1713 | 5 | 25 | 1808 | 8.2 | FDD | IMD4 |
|  | n38 | 2616.7 | 10 | 50 | 2616.7 | N/A | N/A |  | n38 | 2617 | 5 | 25 | 2617 | N/A | TDD | N/A |
| DC_3A_n41A DC_3C_n41A DC_3A_SUL_n41A-n80A, DC_3C_SUL_n41A-n80A | 3 | 1740 | 5 | 25 | 1835 | 8.2 | IMD4 | CA_n3-n41 | n3 | 1740 | 5 | 25 | 1835 | 8.2 | FDD | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | N/A |  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | TDD | N/A |
| DC_3A_n77A, DC_3A_n77(2A), DC_3A_n77(3A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3A_n78(A-C) DC_3C_n78A DC_3C_n78(2A) | 3 | 1740 | 5 | 25 | 1835 | 26 | IMD23 | CA_n3-n77 CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD24 |
|  | n77, n78 | 3575 | 10 | 50 | 3575 | N/A | N/A |  | n77, n78 | 3575 | 10 | 50 | 3575 | N/A | TDD | N/A |
| DC_3A_n77A, DC_3A_n77(2A), DC_3C_n77A, DC_3C_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | 1765 | 5 | 25 | 1860 | 8.0 | IMD43 | CA_n3-n77 CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD24 |
|  | n77, n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |  | n77, n78 | 3575 | 10 | 50 | 3575 | N/A | TDD | N/A |
| DC_5A_n3A | 5 | 838 | 5 | 25 | 883 | N/A | N/A | CA_n3-n5 | n3 | 1771 | 10 | 50 | 1866 | 4 | FDD | IMD4 |
|  | n3 | 1771 | 10 | 50 | 1866 | 4 | IMD4 |  | n5 | 838 | 5 | 25 | 883 | N/A | FDD | N/A |
|  | 5 | 838 | 5 | 25 | 883 | 24 | IMD23 | CA_n3-n5 | n3 | 1721 | 10 | 50 | 1816 | N/A | FDD | N/A |
|  | n3 | 1721 | 10 | 50 | 1816 | N/A | N/A |  | n5 | 838 | 5 | 25 | 883 | 24 | FDD | IMD23 |
| DC_5_n7 | n7 | 2547 | 10 | 50 | 2667 | N/A | N/A | CA_n5-n7 | n5 | 834 | 5 | 25 | 879 | 12 | FDD | IMD34 |
|  | 5 | 834 | 5 | 25 | 879 | 12 | IMD33 |  | n7 | 2547 | 10 | 50 | 2667 | N/A | FDD | N/A |
| DC_5A_n66A | 5 | 838 | 5 | 25 | 883 | 30 | IMD23 | CA_n5-n66 | n5 | 838 | 5 | 25 | 883 | 30 | FDD | IMD24 |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A | N/A |  | n66 | 1721 | 5 | 25 | 2121 | N/A | FDD | N/A |
| DC_5A_n77A8 DC_5A_n77(2A)8 DC_5A_n77(3A)8 | 5 | 844 | 5 | 25 | 889 | 8.3 | IMD4 | CA_n5-n7713 | n5 | 844 | 5 | 25 | 889 | 8.3 | FDD | IMD4 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | N/A |  | n77 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
|  | 5 | 826.5 | 5 | 25 | 871.5 | 5.5 | IMD5 | CA_n5-n7713 | n5 | 829 | 5 | 25 | 874 | 5.5 | FDD | IMD5 |
|  | n77 | 4177.5 | 10 | 50 | 4177.5 | N/A | N/A |  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD | N/A |
| DC_5A_n78A DC_5A_n78(2A) DC_5A_n78(A-C) DC_5A_n78C | 5 | 844 | 5 | 25 | 889 | 8.3 | IMD4 | CA_n5-n78 | n5 | 844 | 5 | 25 | 889 | 8.3 | FDD | IMD4 |
|  | n78 | 3421 | 10 | 50 | 3421 | N/A | N/A |  | n78 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
| DC_7_n3 | 7 | 2535 | 10 | 50 | 2655 | 13 | IMD4 | CA_n3-n7 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  | n7 | 2535 | 10 | 50 | 2655 | 10.2 | FDD | IMD4 |
| DC_7_n5 | 7 | 2547 | 10 | 50 | 2667 | N/A | N/A | CA_n5-n7 | n5 | 834 | 5 | 25 | 879 | 12 | FDD | IMD34 |
|  | n5 | 834 | 5 | 25 | 879 | 12 | IMD33 |  | n7 | 2547 | 10 | 50 | 2667 | N/A | FDD | N/A |
| DC_7A_n26A DC_7C_n26A | 7 | 2547 | 10 | 50 | 2667 | N/A | N/A | CA_n7-n26 | n7 | 2556 | 5 | 25 | 2676 | N/A | FDD | N/A |
|  | n26 | 834 | 5 | 25 | 879 | 12 | IMD33 |  | n26 | 837 | 5 | 25 | 882 | 16.0 | FDD | IMD311 |
|  | 7 | 2567.5 | 5 | 25 | 2687.5 | 2.5 | IMD5 | CA_n7-n26 | n7 | 2567.5 | 5 | 25 | 2687.5 | 2.5 | FDD | IMD5 |
|  | n26 | 816.5 | 5 | 25 | 861.5 | N/A | N/A |  | n26 | 816.5 | 5 | 25 | 861.5 | N/A | FDD | N/A |
| DC_7_n40 | 7 | 2510 | 5 | 25 | 2630 | 23 | IMD3 | CA_n7-n40 | n7 | 2510 | 5 | 25 | 2630 | 23 | FDD | IMD3 |
|  | n40 | 2390 | 5 | 25 | 2390 | N/A | N/A |  | n40 | 2390 | 5 | 25 | 2390 | N/A | TDD | N/A |
| DC_7A_n66A DC_7A-7A_n66A DC_7C_n66A | 7 | 2535 | 10 | 50 | 2655 | 15 | IMD4 | CA_n7-n66 | n7 | 2535 | 10 | 50 | 2655 | 15 | FDD | IMD4 |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | N/A |  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
| DC_7A_n77A DC_7A-7A_n77(2A) DC_7A-7A_n77(3A) DC_7A_n77(2A) DC_7A_n77(3A) DC_7C_n77A DC_7C_n77(2A) | 7 | 2540 | 5 | 25 | 2660 | 7.1 | IMD4 | CA_n7-n77 | n7 | 2540 | 5 | 25 | 2660 | 7.1 | FDD | IMD4 |
|  | n77 | 3870 | 10 | 50 | 3870 | N/A | N/A |  | n77 | 3870 | 10 | 50 | 3870 | N/A | TDD | N/A |
| DC_8A_n1A | 8 | 887.5 | 5 | 25 | 932.5 | N/A | N/A | CA_n1-n8 | n1 | 1965 | 5 | 25 | 2155 | 6.0 | FDD | IMD4 |
|  | n1 | 1965 | 5 | 25 | 2155 | 6 | IMD4 |  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | FDD | N/A |
| DC_8A_n3A | 8 | 900 | 5 | 25 | 945 | 8 | IMD43 | CA_n3-n8 | n3 | 1755 | 10 | 50 | 1850 | N/A | FDD | N/A |
|  | n3 | 1755 | 10 | 50 | 1850 | N/A | N/A |  | n8 | 900 | 5 | 25 | 945 | 8 | FDD | IMD44 |
|  | 8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A | CA_n3-n8 | n3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | FDD | IMD5 |
|  | n3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | IMD5 |  | n8 | 897.5 | 5 | 25 | 942.5 | N/A | FDD | N/A |
| DC_8A_n41A DC_8A_SUL_n41A-n81A | 8 | 882.5 | 5 | 25 | 927.5 | 12.1 | IMD33 | CA_n8-n41 | n8 | 882.5 | 5 | 25 | 927.5 | 12.1 | FDD | IMD34 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
| DC_8A_n77A, DC_8A_n78A, DC_8B_n78A DC_8A_n78(2A), DC_8A_n77(3A), DC_8A_SUL_n78A-n81A | 8 | 897.5 | 5 | 25 | 942.5 | 8.3 | IMD4 | CA_n8-n78 | n8 | 897.5 | 5 | 25 | 942.5 | 8.3 | FDD | IMD4 |
|  | n77, n78 | 3635 | 10 | 50 | 3635 | N/A | N/A |  | n78 | 3635 | 10 | 50 | 3635 | N/A | TDD | N/A |
| DC_8A_n79A, DC_8A_n79C, DC_8A_SUL_n79A-n81A | 8 | 897.5 | 5 | 25 | 942.5 | 4.8 | IMD5 | CA_n8-n79 | n8 | 897.5 | 5 | 25 | 942.5 | 4.8 | FDD | IMD5 |
|  | n79 | 4532.5 | 40 | 216 | 4532.5 | N/A | N/A |  | n79 | 4532.5 | 40 | 216 | 4532.5 | N/A | TDD | N/A |
| DC_12A_n77A DC_12A_n77(2A) | 12 | 702 | 5 | 20 | 732 | 5.5 | IMD5 | CA_n12-n77 | n12 | 702 | 5 | 20 | 732 | 5.5 | FDD | IMD5 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |  | n77 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
| DC_13A_n77A | 13 | 784.5 | 5 | 20 | 753.5 | 5.5 | IMD5 | CA_n13-n77 | n13 | 782 | 5 | 20 | 751 | 5.5 | FDD | IMD5 |
|  | n77 | 3891.5 | 10 | 50 | 3891.5 | N/A | N/A |  | n77 | 3880 | 10 | 50 | 3880 | N/A | TDD | N/A |
| DC_14A_n5A | 14 | 791 | 5 | 25 | 761 | N/A | N/A | CA_n5-n14 | n5 | 836 | 5 | 25 | 881 | 25 | FDD | IMD34 |
|  | n5 | 836 | 5 | 25 | 881 | 25 | IMD3 |  | n14 | 791 | 5 | 25 | 761 | N/A | FDD | N/A |
|  | 14 | 795.5 | 5 | 25 | 765.5 | 25 | IMD3 | CA_n5-n14 | n5 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |  | n14 | 795.5 | 5 | 25 | 765.5 | 25 | FDD | IMD3 |
| DC_14A_n77A DC_14A_n77(2A) | 14 | 795.5 | 5 | 15 | 765.5 | 5.5 | IMD5 | CA_n14-n77 | n14 | 793 | 5 | 20 | 763 | 5.5 | FDD | IMD5 |
|  | n77 | 3947.5 | 10 | 50 | 3947.5 | N/A | N/A |  | n77 | 3935 | 10 | 50 | 3935 | N/A | TDD | N/A |
| DC_18A_n3A | 18 | 823 | 5 | 25 | 868 | N/A | N/A | CA_n3-n18 | n18 | 818 | 5 | 25 | 863 | N/A | FDD | N/A |
|  | n3 | 1721 | 5 | 25 | 1816 | 4 | IMD4 |  | n3 | 1731 | 5 | 25 | 1826 | 4 | FDD | IMD4 |
| DC_18A_n77A DC_18A_n78A | 18 | N/A | N/A | N/A | N/A | N/A | IMD4 | CA_n18-n779 CA_n18-n789 | n18 | N/A | N/A | N/A | N/A | N/A | FDD | IMD4 |
|  | n77, n78 | N/A | N/A | N/A | N/A | N/A | N/A |  | n77, n78 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| DC_20A_n3A | 20 | 840 | 5 | 25 | 799 | N/A | N/A | CA_n3-n20 | 3 | 1775 | 5 | 25 | 1870 | 4 | FDD | IMD4 |
|  | n3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |  | 20 | 840 | 5 | 25 | 799 | N/A | FDD | N/A |
|  | 20 | 847 | 5 | 25 | 806 | 9 | IMD4 |  | 3 | 1735 | 5 | 25 | 1830 | N/A | FDD | N/A |
|  | n3 | 1735 | 5 | 25 | 1830 | N/A | N/A |  | 20 | 847 | 5 | 25 | 806 | 9 | FDD | IMD4 |
| DC_20A_n77A, DC_20A_n78A DC_20A_n78C7, DC_20A_n78(2A), DC_20A_SUL_n78A-n82A | 20 | 850 | 5 | 25 | 809 | 11 | IMD4 | CA_n20-n78 | n20 | 850 | 5 | 25 | 809 | 11 | FDD | IMD4 |
|  | n77, n78 | 3359 | 10 | 50 | 3359 | N/A | N/A |  | n78 | 3359 | 10 | 50 | 3359 | N/A | TDD | N/A |
| DC_25A_n77A DC_25A-25A_n77A | 25 | 1855 | 5 | 25 | 1935 | 26 | IMD2 | CA_n25-n77 | n25 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | N/A |  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | 25 | 1900 | 5 | 25 | 1980 | 8 | IMD4 |  | n25 | 1900 | 5 | 25 | 1980 | 8.0 | FDD | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |  | n77 | 3690 | 10 | 50 | 3690 | N/A | TDD | N/A |
|  | 25 | 1885 | 5 | 25 | 1965 | 5 | IMD5 |  | n25 | 1885 | 5 | 25 | 1965 | 5 | FDD | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | N/A |  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| DC_25A_n78A DC_25A-25A_n78A | 25 | 1855 | 5 | 25 | 1935 | 26 | IMD2 | CA_n25-n78 | n25 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD24 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| DC_28_n50 | 28 | 730 | 10 | 50 | 775 | 15.3 | IMD 2 | CA_n28-n50 | n28 | 730 | 10 | 50 | 775 | 15.3 | FDD | IMD2 |
|  | n50 | 1500 | 10 | 50 | 1500 | N/A | N/A |  | n50 | 1500 | 10 | 50 | 1500 | N/A | TDD | N/A |
|  | 28 | 740 | 10 | 50 | 785 | 6 | IMD 4 | CA_n28-n50 | n28 | 740 | 10 | 50 | 785 | 6.0 | FDD | IMD44 |
|  | n50 | 1500 | 10 | 50 | 1500 | N/A | N/A |  | n50 | 1500 | 10 | 50 | 1500 | N/A | TDD | N/A |
| DC_26A_n77A, DC_26A_n78A | 26 | 836.5 | 5 | 25 | 881.5 | 11.1 | IMD4 | CA_n26-n78 | n26 | 836.5 | 5 | 25 | 881.5 | 11.1 | FDD | IMD4 |
|  | n77, n78 | 3391 | 10 | 50 | 3391 | N/A | N/A |  | n78 | 3391 | 10 | 50 | 3391 | N/A | TDD | N/A |
| DC_28A_n77A, DC_28A_n78A, DC_28A_n78(2A), DC_28A_SUL_n78A-n83A | 28 | 705.5 | 5 | 25 | 760.5 | 5.5 | IMD5 | CA_n28-n77 | n28 | 705.5 | 5 | 25 | 760.5 | 5.5 | FDD | IMD5 |
|  | n77, n78 | 3582.5 | 10 | 50 | 3582.5 | N/A | N/A |  | n77/n78 | 3582.5 | 10 | 50 | 3582.5 | N/A | TDD | N/A |
| DC_30A_n77A DC_30A_n77(2A) | 30 | 2310 | 5 | 25 | 2355 | 8.0 | IMD4 | CA_n30-n77 | n30 | 2310 | 5 | 25 | 2355 | 8.0 | FDD | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | N/A |  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | TDD | N/A |
| DC_38A_n3A | n3 | 1713 | 5 | 25 | 1808 | 8.2 | IMD4 | CA_n3-n38 | n3 | 1713 | 5 | 25 | 1808 | 8.2 | FDD | IMD4 |
|  | 38 | 2617 | 5 | 25 | 2617 | N/A | N/A |  | n38 | 2617 | 5 | 25 | 2617 | N/A | TDD | N/A |
| DC_41A_n3A DC_41C_n3A | n3 | 1740 | 5 | 25 | 1835 | 8.2 | IMD4 | CA_n3-n41 | n3 | 1740 | 5 | 25 | 1835 | 8.2 | FDD | IMD4 |
|  | 41 | 2657.5 | 5 | 25 | 2657.5 | N/A | N/A |  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | TDD | N/A |
| DC_48A_n2A DC_48C_n2A DC_48D_n2A DC_48E_n2A | 48 | 3625 | 20 | 100 | 3625 | N/A | N/A | CA_n2-n48 | n2 | 1852.5 | 5 | 25 | 1932.5 | 12 | FDD | IMD4 |
|  | n2 | 1852.5 | 5 | 25 | 1932.5 | 12 | IMD4 |  | n48 | 3625 | 20 | 100 | 3625 | N/A | TDD | N/A |
| DC_48A_n25A DC_48C_n25A DC_48D_n25A | 48 | 3625 | 20 | 100 | 3625 | N/A | N/A | CA_n25-n48 | n25 | 1852.5 | 5 | 25 | 1932.5 | 12 | FDD | IMD4 |
|  | n25 | 1852.5 | 5 | 25 | 1932.5 | 12 | IMD4 |  | n48 | 3625 | 20 | 100 | 3625 | N/A | TDD | N/A |
| DC_48A_n66A DC_48C_n66A DC_48D_n66A | 48 | 3630 | 20 | 100 | 3630 | N/A | N/A | CA_n48-n66 | n48 | 3660 | 5 | 25 | 3660 | N/A | TDD | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | 4 | IMD5 |  | n66 | 1730 | 5 | 25 | 2130 | 5.0 | FDD | IMD5 |
| DC_66A_n2A, DC_66A-66A_n2A | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A | CA_n2-n66 | n2 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD3 |
|  | n2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |  | n66 | 1775 | 5 | 25 | 2175 | N/A | FDD | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 | CA_n2-n66 | n2 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |
|  | n2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |  | n66 | 1750 | 5 | 25 | 2150 | 4 | FDD | IMD5 |
| DC_66A_n5A | n5 | 838 | 5 | 25 | 883 | 30 | IMD23 | CA_n5-n66 | n5 | 838 | 5 | 25 | 883 | 30 | FDD | IMD24 |
|  | 66 | 1721 | 5 | 25 | 2121 | N/A | N/A |  | n66 | 1721 | 5 | 25 | 2121 | N/A | FDD | N/A |
| DC_66A_n7A DC_66A-66A_n7A DC_66A_n7(2A) DC_66A-66A_n7(2A) | 66 | 1730 | 5 | 25 | 2130 | N/A | N/A | CA_n7-n66 | n7 | 2535 | 10 | 50 | 2655 | 15 | FDD | IMD4 |
|  | n7 | 2535 | 10 | 50 | 2655 | 15 | IMD4 |  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
| DC_66A_n25A | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A | CA_n25-n66 | n66 | 1775 | 5 | 25 | 2175 | N/A | FDD | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |  | n25 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD3 |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | 23 | IMD3 | CA_n25-n66 | n66 | 1712.5 | 5 | 25 | 2112.5 | 23 | FDD | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | FDD | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 | CA_n25-n66 | n66 | 1750 | 5 | 25 | 2150 | 4 | FDD | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |
| DC_66A_n48A | 66 | 1715 | 5 | 25 | 2115 | 4 | IMD5 | CA_n48-n66 | n48 | 3660 | 5 | 25 | 3660 | N/A | TDD | N/A |
|  | n48 | 3630 | 20 | 100 | 3630 | N/A | N/A |  | n66 | 1730 | 5 | 25 | 2130 | 5.0 | FDD | IMD5 |
| DC_66A_n71A | 66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 | CA_n66-n71 | n66 | 1750 | 5 | 25 | 2150 | 5 | FDD | IMD4 |
|  | n71 | 675 | 5 | 25 | 629 | N/A | N/A |  | n71 | 675 | 5 | 25 | 629 | N/A | FDD | N/A |
| DC_66A_n77A DC_66A_n77(2A) DC_66A-66A_n77A DC_66A-66A_n77(2A) DC_66A-66A-66A_n77A DC_66A-66A-66A_n77(2A) | 66 | 1775 | 5 | 25 | 2175 | 31.0 | IMD2 | CA_n66-n77 | n66 | 1775 | 5 | 25 | 2175 | 31 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | N/A |  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 5.0 | IMD5 | CA_n66-n77 | n66 | 1760 | 5 | 25 | 2160 | 5.0 | FDD | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
| DC_66A_n78A | 66 | 1730 | 5 | 25 | 2150 | 5.0 | IMD5 | CA_n66-n78 | n66 | 1730 | 5 | 25 | 2130 | 5.0 | FDD | IMD5 |
|  | n78 | 3660 | 10 | 50 | 3660 | N/A | N/A |  | n78 | 3660 | 10 | 50 | 3660 | N/A | TDD | N/A |
| DC_71A_n41A | 71 | 666 | 5 | 25 | 620 | 11 | IMD4 | CA_n41-n71 | n41 | 2614 | 5 | 25 | 2614 | N/A | TDD | N/A |
|  | n41 | 2618 | 5 | 25 | 2618 | N/A | N/A |  | n71 | 665 | 5 | 25 | 619 | 11 | FDD | IMD4 |
| DC_71A_n66A | 71 | 675 | 5 | 25 | 629 | N/A | N/A | CA_n66-n71 | n66 | 1750 | 5 | 25 | 2150 | 5 | FDD | IMD4 |
|  | n66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 |  | n71 | 675 | 5 | 25 | 629 | N/A | FDD | N/A |
| DC_71A_n77A8 | 71 | 671 | 5 | 25 | 625 | 5.5 | IMD5 | CA_n71-n7713 | n71 | 671 | 5 | 25 | 625 | 5.5 | FDD | IMD5 |
|  | n77 | 3309 | 10 | 50 | 3309 | N/A | N/A |  | n77 | 3309 | 10 | 50 | 3309 | N/A | TDD | N/A |
| DC_71A_n78A | 71 | 681.5 | 5 | 25 | 635.5 | 5.5 | IMD5 | CA_n71-n78 | n71 | 681.5 | 5 | 25 | 635.5 | 5.5 | FDD | IMD5 |
| DC_71A_n78(2A) | n78 | 3361.5 | 10 | 50 | 3361.5 | N/A | N/A |  | n78 | 3361.5 | 10 | 50 | 3361.5 | N/A | TDD | N/A |
| NOTE 1: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBstart = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: Void NOTE 5: Void NOTE 6: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 7: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This band is subject to IMD2, IMD4 and IMD5 fall in n28 also which MSD is not specified. In addition, this band is subject to IMD4 fall in B21 also which MSD is not specified. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  | NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4 NOTE 2: RBSTART = 0, 15 kHz SCS is assumed. NOTE 3: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the FDD band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3 apply). NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Void. NOTE 6: Considering the spectrum holdings of the operator for CA_n77(2A) (when one uplink sub block is assigned within 3300-3400MHz, the other uplink sub block is not assigned within 4000-4200MHz or vice versa), no IMD5 result will fall in Rx frequency range of band n3. Therefore, no MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). NOTE 7: In current release the maximum separation bandwidth class is 600MHz, therefore, no IMD2 MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). NOTE8: There is no IMD4/5 products in band n18 downlink for n77 operating in 3520 – 3560 MHz, 3700 – 3800MHz and 4000 - 4100MHz frequency range. NOTE 9: There is no IMD4 product in band n18 downlink for n78 operating in 3520 – 3560MHz and 3700-3800MHz frequency range. NOTE 10: There is no IMD4 product in band n24 downlink for n77 operating in 3450 – 3980 MHz and n24 uplink restricted to between 1627.5 – 1637.5 MHz and between 1646.5 – 1656.5 MHz. NOTE 11: This band is subject to IMD5 also which MSD is not specified.. NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 14: Applicable when n41 spectrum is restricted to 2515-2675MHz |  |  |  |  |  |  |  |  |

Summary

Proposal 1: The following text proposals are provided to be captured into TR 38.846 based on the analysis.

MSD due to IMD for two band combinations for UL NR CA, UL NR DC, EN-DC and NE-DC band combinations are shown in table 1 based on the TS 38.101-1-i00 and TS 38.101-3-i00. One band combination can be chosen to verify the requirements for some band combinations in same row in table 1, as a result of reducing test burden. It’s suggested to randomly choose one band combination that UE support for MSD testing. The proposed test reduction could be considered for an informative annex in the TS since the final decision should be taken by RAN5 based on the industry certification testing needs.

- References

[1] R4-2213163, General discussion on Simplification of band combination specification, Huawei, HiSilicon